TVP5158 Capture Driver for DM365 using DVSDK 3.x

Version 00.02.00

Release Notes

June 2011

Build ID: 00.02.00.00

The release note is divided into the following sections:

- New In This Release

- Validation Information

- Supported Features

- Test Application Requirements

- Building The Driver and Application From Source Code

- Launching Sample Application

- Fixed In This Release

- Limitations / Known Issues

# New In This Release

- Added support for simultaneous video capture from two TVP5158s in DVSDK 3.x.

- Added support for all video modes with Dual-TVP5158 setup.

- 2Ch D1 - Port A4Ch CIF

# Validation Information

- This release is validated on the following:

- DM365 DVR .

- DVSDK v3.10.00.19

- Git kernel PSP v3.02 with Linux kernel v2.6.32

- Arago root file system for DM6467T

# Supported Features

The following TVP5158 capture modes of operation are supported in this release:

| - Mode - Number | - Mode | - Supported in this - Version |
| --- | --- | --- |
| - 00 | - 2Ch D1 (BT656) | - Yes |
| - 01 | - 4Ch D1 (BT656 ) | - Yes |
| - 02 | - 4Ch Half-D1 (BT656 ) | - Yes |
| - 03 | - 4Ch CIF (BT656 ) | - Yes |
| - 04 | - 4Ch D1 (BT1120) | - No |
| - 05 | - 4Ch Half-D1 (BT1120) | - No |
| - 06 | - 8Ch Half-D1 (BT656 ) | - Yes* |
| - 07 | - 8Ch CIF (BT656 ) | - Yes |
| - 08 | - 4Ch Half-D1 + D1 (BT656 ) | - Yes |
| - 09 | - 4Ch CIF + D1 (BT656 ) | - Yes |
| - 0a | - 8Ch CIF + D1 (BT656 ) | - Yes |
| - 0b | - 4Ch Half-D1 CIF progressive (BT656 ) | - Yes |
| - 0c | - 4Ch Half-D1 CIF progressive (BT1120) | - No |
| - 0d | - 8Ch Half-D1 CIF progressive (BT656 ) | - Yes* |
| - 0e | - 2Ch Half-D1 (BT656) | - Yes |
| - 10 | - 2Ch D1 (BT656 ) CROP | - Yes |
| - 11 | - 4Ch D1 (BT656 ) CROP | - Yes |
| - 12 | - 4Ch Half-D1 (BT656 ) CROP | - Yes |
| - 13 | - 4Ch CIF (BT656 ) CROP | - Yes |
| - 14 | - 4Ch D1 (BT1120) CROP | - No |
| - 15 | - 4Ch Half-D1 (BT1120) CROP | - No |
| - 16 | - 8Ch Half-D1 (BT656 ) CROP | - Yes* |
| - 17 | - 8Ch CIF (BT656 ) CROP | - Yes |
| - 18 | - 4Ch Half-D1 + D1 (BT656 ) CROP | - Yes |
| - 19 | - 4Ch CIF + D1 (BT656 ) CROP | - Yes |
| - 1a | - 8Ch CIF + D1 (BT656 ) CROP | - Yes |
| - 1b | - 4Ch Half-D1 CIF progressive (BT656 ) CROP | - Yes |
| - 1c | - 4Ch Half-D1 CIF progressive (BT1120) CROP | - No |
| - 1d | - 8Ch Half-D1 CIF progressive (BT656 ) CROP | - Yes |

- * This mode has color artifacts in the TVP5158 captured output.

# Building the driver and Application From Source Code

Building TVP5158 Driver:

Go to the directory mcvip_tvp5158/build and run the following to apply the TVP5158 driver patch to the kernel:

$ sh kernel_patch.sh <git_kernel_path>

Where,  <git_kernel_path> is the installed git kernel path in the host PC.

To generate the uImage, build the updated kernel by running the following command in the DVSDK 3.x directory:

$ make linux

Building Test Application:

Go to the directory mcvip_tvp5158/build and run the following:

$ make clean; make

This will generate the application executable 'mcvip_test.out' in the directory mcvip_tvp5158/bin

# Launching Sample Application

Bootup the board. Please ensure that you are using kernel-image provided with this release (available

under Bin/kernel-Image directory).

All the prebuilt executables and scripts are available under Bin/bin folder.

Start the test application as follows:

$ ./modules.sh

$ ./mcvip_test.out

This will give the user interface to select the required capture mode. On selecting the particular mode,  user has the option to select the channel to preview.

Launch Audio Test (Apply the audio driver patch and build the uImage )

To test the audio driver use the ALSA arecord , aplay and amixer

16KHz

$ arecord  -twav -f S16_LE -c4 -r16000 | aplay

8KHz

$ arecord  -twav -f S16_LE -c4 -r8000 | aplay

# Fixed In This Release

- Fixed the problem during simultaneous capture from two TVP5158 EVM's.

# Limitations / Known Issues

- The TVP5158 driver only supports the modes specified under the section 'Supported Features'.

- The  driver is validated with limited test suit.

- Color artifact is observed with the capture modes involving 8-CH half-D1.

- Does not support audio.

- With the sample application provided, jitteriness is observed in the display.

- It is observed that switching between different channels for display makes the display freeze for some channels.

- While displaying the CIF and Half-D1 resolutions, the background on D1 display is not cleared, showing jitteriness in the other part of display.