Hi,

I am designing a board with a TFP410 connected to an FPGA.

The output of the TFP410 is connected to a Samsung 961BF monitor.

The image is 1024*768 pixels.

The clock freq.  (IDCK+) is 65MHz, and the Samsung 961BF monitor requirements for a 1024x768 pixel image are as follows:

- Hsync, Vsync and DE are synchronized to the rising edge of the 65 MHz clock

- Hsync is active low for 320 clocks , during which time DE is low (i.e., line rate = 48.363KHz as per Samsung 961BF monitor requirements)

- Vsync is active low for 50738 clocks, during which time DE is low (i.e., frame rate = 60.004 Hz as per Samsung 961BF monitor requirements)

- Hsync and DE change together

- Vsync follows Hsync while DE remains low

The TFADJ resistor is 510 Ohm and connected to 3.3v.

PD pin is connected to 3.3v via a pullup resistor

ISEL is low. ->I2C disabled

EDGE is high ->Rising Edge

BSEL is high -> 24 Bit

DKEN is low ->DSKEW disabled

CTL1 CTL2 CTL3 are low.

All the inputs to the TFP410 were sampled with a scope and appear to be OK.

The TFP410 outputs appear to be active and the clock is clean.

The TFP410 data sheet does not have much information concerning timing and does not describe any special requirements needed to initialize the monitor

Our monitor remains black, so it seems that we are still missing some definition.

So I have some questions:

1. Should I need to reverse the DE signal?

2. What is the importance of 65 MHz clock frequency as long as the line and frame rates are satisfied.

3. Does the monitor require a wake up command or are the Hsync and Vsync are enough?

From Samsung 961BF monitor datasheet: