## How to use ACOAC

Acoac uses a Launchpad to mimic the sound of castanets. (It is a cow of another color.)

Please send your comments or questions to old_cow_yellow@yahoo.com

# Hardware Required

a) TI Launchpad Kit (MSP-EXP430G2).

b) A Piezo Transducer that can produce sound when driven with ~3V square-wave at ~2.5kHz. (e.g., RadioShack, part number 273-073A)

c) A PC to download acoac into the target uP. It is also used as a terminal to “program” acoac.

## Software Required

a) A PC program to download acoac into the target uP. (e.g., KickStart from TI/IAR)

b) A PC program to emulate a terminal for acoac. (e.g., Hyperterminal under Windows)

## Step One - Set up the Launchpad Board

a) Make sure all five jumpers of J3 and two jumpers of J5 are in place (as delivered from factory).

b) The Launchpad kit includes two MSP430G22x1 chips. The G2231 already installed on the board has a TI demo program in it. The G2211 is blank. The next step will erase and re-write the Flash memory of whatever chip that is in the IC1 socket. Thus you may want to replace the chip in that socket. Take notice of the orientation and offset of the factory installed G2311 before you remove it. Install the G2211 the same way.

c) Connect the Piezo Transducer to P1.6 and P1.7 of the Launchpad board. E.g., RadioShack, P# 273-073A buzzer can be used. Connect the red-wire to P1.6 and the black-wire to P1.7 of the Launchpad.

d) The 32kHz crystal included in the kit is not needed. But if you have already soldered that to the board, it is okay too.

## Step Two - Download ACOAC into the target uP

There are many different ways to download acoac into the target uP, depends on the PC and the software used.

If you use KickStart, the source code and project files under KickStart are included in a sub-folder called acoac_project.

If you use FET-Pro430 (or the free Lite version), the object code of acoac is also included here in two different formats. Using TI msp430-txt format, the object file is called acoac.txt. Using Intel-extended format, the object file is called acoac.hex.

If you use CCS or gcc etc., you may be able to download one of the object files. Otherwise, you may need to convert the source code, as they tend to use different pseudo-ops and directives. I do not know how to do that. Those who know, please share the information.

## Step Three - Test Drive

## After acoac is downloaded into the target uP, exit the PC program, and recycle the power (e.g., by unplug and re-plug the USB cable). This is needed because the PC program may put the target uP in a test mode and cannot function normally.

## Now press the S2 button and you should hear a sequence of clicking sound. The green LED should also flash at the same time. The S1 button is for resetting the uP. It is normally not used.

## Beyond 1, 2, 3 - Connect a “Terminal” to ACOAC

The sequence of clicking sound you heard is just an example of what acoac can generate. You can “program” acoac to generate what you want with a “terminal”.

The Launchpad USB interface includes support of a “MSP Application UART” in the PC under Windows. Thus Windows Applications such as Hyperterminal can be used to emulate a “terminal” for acoac.

If you are using CCS or IAR (including Kickstart), the USB driver is “automatically” installed. You do not need to do anything. (Occasionally, it “automatically” does not work, and you cannot do anything.)

Anyway, Hyperterminal or similar terminal emulators can be used. The basic settings are:

9600 b/s, 8 data-bits, no parity

No flow control

No local echo

No line-feeds after line-ends

In addition, the following options are desirable:

Back-space key sends Ctrl-H, space, Ctrl-H

Line delay: 100 msec

## Command Line

Here are all the commands that acoac understands:

do …	do one of the machine language coded words or one of the “let” defined words.

let …	define a new word or re-define an existing word.

list	list all the “let” defined words.

erase	erase all the “let” defined words.

[blank]  	(empty/blank line) show number of Flash bytes left to store “let” words.

;…	(line start with “;”) comments, to be ignored by caoca.

The machine language coded words are:

m 	for macho, or lower pitched castanets

h 	for hembra, or higher pitched castanets

2m 	for two macho clicks

4h 	for four hembra clicks

- 	for a quiet period with no sound

p 	for piano, or soft clicks, from now on until changed by f

f 	for forty, or hard clicks, from now on until changed by p

Thus when there is no “let” defined words, you can only “do” one of the above words. But an example of a few “let” defined words is already “programmed”. You can use “list” to see them, and you can “do” any one of them too. You can use “let” command to add more words of your own. And you can use “erase” to start from scratch.

“let” command

The “let” command is a little tedious to type. You can use a text editor in your PC to prepare a text file and let the “terminal” emulator do the typing, instead of typing it yourself directly. Either way, the syntax of the “let” command is as follows:

let [word] = [sequence]<Enter>

When accepted by acoac, the [word] on the left-hand side of “=” is defined to the [sequence] on the right-hand side of “=”.

[sequence] is a sequence of one or more (a) machine language coded word, (b) “let” defined word, or (c) [repeat sequence].

[repeat sequence] is “n( ” followed by a [sequence] and “) ”. Where n is a digit between 2 and 9.

Use “list” command to see some examples of “let” commands. Note that all the elements in the “let” command must be separated with one and only one blank (ASCII 0x20).

You can and perhaps should use “let” to define the word “S2”. Pushing the S2 button on the Launchpad board automatically generates a “do S2” command without using the “terminal”.

Have fun!