|  | Checklist | Note |
| --- | --- | --- |
|  | General Visual Inspection |  |
|  | Check for any damaged components |  |
|  | Verify actual component value vs. schematic design value |  |
|  | Check for any bad solder joints |  |
|  | Check for missing or desoldered components |  |
|  | Wrong Cell Voltage Measurement |  |
|  | Check actual cell voltage at battery cell side vs. VCx pins | May have bad components (the RC filters) if voltage value doesn't match. If initial reported voltage is correct, but then it slowly drifted off. This could be caused by a bad filter cap. Inspect or replace the component. |
|  | If VCELL+/- pins are accessible. Measure the VCELL+/- waveform. | See Fig.1 |
|  | If external cell balancing is used, check for any damaged FETs | If one cell is reporting a small voltage, while the cell next to it is report a very high voltage. It is very likely one of the cell balancing FETs is damaged |
|  | If calibration data looks abnormal, than save a copy of .gg or .senc file and re-calibrate voltage. |  |
|  | Wrong Current Measurement |  |
|  | Measure voltage across sense resistor. I = Vsense/Rsense. Compared the measured current vs. device reported current | Could be a bad IC if measured vs. reported current doesn't match |
|  | If calibration data looks abnormal, than save a copy of .gg or .senc file and re-calibrate CC offset and CC gain. |  |
|  | Wrong Temperature Measurement |  |
|  | Check thermistor contact/soldering | Open: reporting a very high temperature >120degC (at room) Short: reporting a very small temperature <-30degC (at room) |
|  | Check thermistor waveform on TOUT pin | See Fig.1 |
|  | Verify data flash Temp coef vs. device default (if 103AT-2 NTC is used) |  |
|  | If non-103AT NTC is used, verify data flash temp coef vs. thermistor coef Calculator | Thermistor Coefficient Calculator for TI Advanced Fuel Gauges: slua398 |
|  | If calibration data looks abnormal, than save a copy of .gg or .senc file and re-calibrate temperature |  |
|  | No Communication |  |
|  | Measure regulator voltages | REG33 = 3.3V REG25 = 2.5V |
|  | If regulators are not on, measure VBAT pin and make sure VBAT is >6V Perform a jump start at PACK+ | If device is still not powering up, apply voltage on PACK+ (at least 6V). Send failure PCB to FA if device still fails to power up (could be a bad IC) |
|  | If regulator voltages are correct, but there is no communication. Provide SMBus scope capture to TI for analysis |  |
|  | Cannot Write to Flash |  |
|  | Check if device is sealed. Can only write to flash is device is unseal |  |
|  | Check the data flash "Flash Update Ok Voltage" value. Make sure battery voltage is higher than this parameter setting for flash update | For 2-cell application, the default "Flash Update Ok Voltage" is too high. Apply a charger (i.e. voltage at PACK+), and reduce this parameter value to 6V |
|  | IT related Issue |  |
|  | For all IT related issues, please provide: - gg file/senc file - Log data - Test procedure | Refer to SLUA474 for detail |

AFE cell channels offset :

cell0

cell1

cell2

cell3

Vcell0~3

AFE ref

2. Tout

1. Cell+

AFE gain

Pack+ measurement

ADC Offset

a

Fig.1: bq20zxx VCELL+ and Tout waveforms. The pulse period is ~250msec.

The VCELL+ signal is inverted – the smaller the Cell Voltage, the higher the VCELL+ amplitude. The delta between Vcell and AFE channel offset (i.e. value a) = the scaled cell voltage = Actual cell voltage * 0.15