//******************************************************************************

//	 TLC5927 Application Source Code

//

//     Description: The MSP430G2553 acts as the SPI master to three

//	 TLC5927/TLC5926 LED drivers sinking current from 16 channels each.

//	 The drivers sink current from 16 RGB LEDs with each driver sinking

//	 current from a particular color node (Red, Green, or Blue).  The SPI

//	 SCLK is sourced by the SMCLK = DCO ~16MHZ.  The SPI module used by the

//	 SPI master is USCI_B0 in SPI mode.

//

//	 SPI communication with the drivers is through a SPI daisy chain configuration.

//

//	 Timer1_A3 is used to provide PWM outputs in order to dim LEDS.  This is

//	 done through the OE' pin on the drivers.  The timer is also sourced by

//	 the SMCLK.

//

//

//                   MSP430G2553

//                 -----------------

//          LED <-|P1.0             |

//                |                 |

//          TXD <-|P1.1          RST|-> Reset Button

//                |                 |

//          RXD <-|P1.2         P1.7|-> SIMO (UCB0SIMO)

//                |                 |

//    Button S2 <-|P1.3		      |

//		      |		      |

//  		 LE <-|P1.4         P1.6|<- SOMI (UCB0SOMI)

//                |                 |

//  		OE' <-|P2.1         P1.5|-> SCLK (UCB0CLK)

//

//

//   Roberto Mancera Jr

//   Texas Instruments Inc.

//   June 2011

//   Built with CCS Version 4.2.3

//******************************************************************************

#include "msp430g2553.h"

#define SCLK		BIT5

#define LE 			BIT4

#define OE 			BIT1

void initialize_clk();

void initialize_SPI();

void latch_out(unsigned int seq_R, unsigned int seq_G, unsigned int seq_B);

void pwm_config( unsigned int half_duty, unsigned int half_period);

void special_conf(unsigned int config_code_R, unsigned int config_code_G,

unsigned int config_code_B);

void switch_mode(unsigned char mode);

unsigned int rDriverConfig;			    // store configuration code and error code

unsigned int gDriverConfig;			    // for each LED driver

unsigned int bDriverConfig;

unsigned int rErr, gErr, bErr;

void main()

{

initialize_clk();							// Set up basic clk module

WDTCTL = WDTPW + WDTHOLD;                 // Stop watchdog timer

P1OUT = 0x00;

P2SEL = 0x00;

P2OUT = BIT1;								// Initialize OE' (LEDs off)

P1DIR = BIT0 + BIT4;

P2DIR = BIT1;

P1IE |= BIT3;								// Initialize P1.3(button) interrupt

__bis_SR_register(GIE);

rDriverConfig = gDriverConfig = bDriverConfig = 0xFFFF; //Store default config codes

__delay_cycles(75);                 		// Wait for LED driver to initialize

latch_out(0xFFFF, 0xFFFF, 0xFFFF);		// Latch LED output (white all LEDs on)

/*comment line below to write gain code*/

pwm_config(0x7FFF, 0xFFFF);				// dim to half duty cycle brightness

/*uncomment 4 lines below to write gain code*/

//switch_mode(0);

//latch_out(0x00FF, 0x00FF , 0x00FF );		// dim (gain code 0x00 for all 3 drivers)

//switch_mode(1);

//P2OUT &= ~OE;

while(1);

}

void initialize_clk()

{

if (CALBC1_16MHZ == 0xFF || CALDCO_16MHZ == 0XFF) // Calibrate DCO at 16 MHz

while(1);										// 		DCO=MCLK=SMCLK

BCSCTL1 = CALBC1_16MHZ;

BCSCTL2 = DIVS_1;

DCOCTL = CALDCO_16MHZ;

}

/*

*  Function: initialize_SPI

*  Description: Initializes module USCI_B0 in SPI 3 wire mode.  This

* 		module is to be used to communicate with the TLC5927 LED drivers.

* 		the module is set up to output to the SDI by send the LSB of a 16

* 		bit word first.

*  Parameters: none

*  Return: none

*/

void initialize_SPI()

{

P1SEL = BIT5 + BIT6 + BIT7;				// setup SCLK = p1.5, SOMI = p1.6,

P1SEL2 = BIT5 + BIT6 + BIT7;				//		SIMO = P1.7

UCB0CTL0 |= UCMST + UCSYNC + UCCKPH;  				// 3-pin, 8-bit SPI master

UCB0CTL1 |= UCSSEL_2;                     // SMCLK

UCB0BR0 |= 0x01;                          // SMCLK/1

UCB0BR1 = 0;

UCB0CTL1 &= ~UCSWRST;                     // **Initialize USCI state machine**

}

/*

*  Function: latch_out

*  Description: This function latches on/off sequence to the output register, but

* 		does not modify the OE' pin on the LED drivers, it also latches configuration

* 		code to the configuration code register.  Function should be executed

* 		in normal mode or special mode to write 16 bit configuration code.

* 	Note: When latching a config code bits 15-8 are the low config code bits (0-8)

*  Paramters: Each driver recieves a 16 bit sequence outlining the channels

* 		that will be on/off.

*  Return:

*/

void latch_out(unsigned int seq_R, unsigned int seq_G, unsigned int seq_B)

{

initialize_SPI();

_delay_cycles(10);						   // turn LEDs off while writing

while (!(IFG2 & UCB0TXIFG));              // USCI_B0 TX buffer ready

UCB0TXBUF = (unsigned char)seq_B;				   // TX OUT15 - OUT8 intructions for

while (!(IFG2 & UCB0TXIFG)); 			   // Green LED driver.  Lsb first.

UCB0TXBUF = (unsigned char)(seq_B >> 8);		   // Transmit OUT7-OUT0

while (!(IFG2 & UCB0TXIFG));

UCB0TXBUF = (unsigned char)seq_G;

while (!(IFG2 & UCB0TXIFG));

UCB0TXBUF = (unsigned char)(seq_G >> 8);

while (!(IFG2 & UCB0TXIFG));

UCB0TXBUF = (unsigned char)seq_R;

while (!(IFG2 & UCB0TXIFG));

UCB0TXBUF = (unsigned char)(seq_R >> 8);

while (!(IFG2 & UCB0TXIFG));

P1OUT |= BIT4;								// Pulse LE to store to output

_delay_cycles(1);							//	register.

P1OUT &= ~BIT4;

}

/*

*  Function: pwm_config

*  Description: Utilizes timerA to produce a pwm output on the OE' pin

* 		of the LED driver.  To be utilized in normal mode to choose dimm on

* 		LEDs and to turn them on.

*  Paramters: User provided 1/2 duty cycle and 1/2 period that the

* 		LEDs are to be turned off. 1/2 period must be less than or equal to 0xFFFF clock

* 		cycles.

*  Return: none

*/

void pwm_config( unsigned int half_duty, unsigned int half_period)

{

P2DIR |= BIT1;							//configure p1.2 for pwm out to OE'

P2SEL |= BIT1;

TA1CCR0 = half_period;

TA1CCTL1 = OUTMOD_6;                      // CCR1 toggle/set

TA1CCR1 = half_period - half_duty;        // CCR1 PWM duty cycle

TA1CTL = TASSEL_2 + MC_3;                 // SMCLK, up-down mode

}

/*

*  Function: switch_mode

*  Description: MCU communication via LE and OE' pins of TLC5927 to command the devices

* 		to engage in mode switching between special mode and normal mode.

*  Parameters: mode = 0 to switch from normal to special mode, and mode = 1 to switch

* 		from special mode to normal mode.

*  Return: none

*/

void switch_mode(unsigned char mode)

{

/* period 1 */

P1OUT &= ~LE;

P2OUT |= OE;							 // Set OE' as high and set

P1SEL &= ~SCLK;						  	 // SCLK as regular output

P1SEL2 &= ~SCLK;						 	 // low slave clock

P1OUT &= ~SCLK;

P1DIR |= SCLK;

_delay_cycles(1);

P1OUT |= SCLK;							 // high slave clock

_delay_cycles(1);

/* period 2 */

P2OUT &= ~OE;

P1OUT &= ~SCLK;  							 // low slave clock

_delay_cycles(1);

P1OUT |= SCLK;							 // high slave clock

_delay_cycles(1);

/* period 3 */

P2OUT |= OE;

P1OUT &= ~SCLK;

_delay_cycles(1);

P1OUT |= SCLK;

_delay_cycles(1);

/* period 4 */

if (mode == 0)

P1OUT |= LE;

P1OUT &= ~SCLK;

_delay_cycles(1);

P1OUT |= SCLK;

_delay_cycles(1);

/* period 5 */

if (mode == 0)							  // switch to special/normal

P1OUT &= ~LE;

P1OUT &= ~SCLK;

_delay_cycles(1);

P1OUT |= SCLK;

_delay_cycles(1);

P1OUT &= ~SCLK;							  // reset slave clk

_delay_cycles(4);

}

/**********************************************************************************/