- Inter DSP communication using Hyperlink

1. Overview

This document shows how C6670 users can make a generic packet data transaction between DSPs by using the current capability built in C6670 or C6678.

This model uses multi core Navigator and QMSS packet DMA infrastructure mode as a basic DMA method and Hyperlink is used to translate certain local address map area to the remote DSP.  Hyperlink Example project has an example configuration for Hyperlink, QM, QMSS PktDMA, Accumulator and CPU interrupt to check the data receiving time.

2. High level view of the example

Hyperlink Example project shows the special use case that using the infrastructure PktDMA in QMSS of the local device to move data from local DSP to remote DSP.

- This project shows symmetric data flow between local and remote DSP

- Using infrastructure PktDMA inside QMSS for automatic memory copy between local and remote DSP

- Using monolithic packet descriptor to improve the efficiency. It works with the host descriptors too, but with longer latency.

- No software intervention needed during run time, address translation is done by Hyperlink mode during run time.

3. Memory View in Local Device

Hyperlink memory address range is 0x4000_0000 to 0x4FFF_FFFF.

Example maps the remote QM physical memory in certain HyperLink address range from 0x4000_0000 to 0x401F_FFFF

It also maps the remote MSMC SRAM region area in certain HyperLink address range from 0x4020_0000 to 0x403F_FFFF

QM in local DSP only manages the memory locally. In other words, the descriptor pointers in the local QM do not point to the memory at the remote side and QM in the remote side only manages the memory on the remote side.

PktDMA normally can handle two physically different QMs (logically 4 QMs)

The first logical QM handles the first 4096 local queues and the second QM handles the second 4096 local queues. The third logical QM handles the first 4096 remote queues and the fourth QM handles the second 4096 remote queues.

The following figure shows the memory map in local DSP. Some memory segments in local HyperLink are mapped to remote QM and MSMC memory for this test.

- Memory map in local DSP

4. Example code summary

The main purpose of this program is to make the local DSP move data to the remote MSMC SRAM mono memory region and let remote QM handle received data by accumulator (generating interrupt when the list is full)

On the remote device, allocate some memory region in MSMC SRAM just for receiving packet data from the remote device.

The local DSP configures the QM and assigns the Tx free queue and Tx queue for infrastructure mode, and configures the HyperLink module to turn on the SERDES and map remote QM and MSMC area to local HyperLink memory space that starts from 0x4000_0000.

When the local DSP pushes 8 packets into the Tx queue, the PktDMA in QMSS transfers those packets to Rx side (loopback mode is default setup) and PktDMA Rx side checks the Rx flow and finds the Rx free queue and Rx queue is handled by QM3 which is in the remote device. QM3 memory region is in 0x4020_0000 and it is mapped to remote MSMC memory mono region, so the packet is transferred to the remote MSMC SRAM through HyperLink output data line and the mono descriptor pointer is pushed into the Rx queue in the remote DSP through the QM memory mapping (0x4000_0000  remote QM)

Accumulator in remote DSP gathers the packets and issues interrupt when it is fully filled. Interrupt service routine in remote DSP will activate data compare and module closing routine. The same procedures must be done in opposite direction if user starts two DSPs on the EVM at the same time.

5. Data flow step detail

- Local device pushes packets to the TX queue (800), and the QM asserts the queue pending signal to the infrastructure PktDMA

- Infrastructure PktDMA fetches the Tx free descriptor pointers from local QM to transmit the packets in the TX queue

- Local PktDMA fetches the monolithic descriptor from the local memory and sends the packet to the remote DSP

- PktDMA fetches the Rx free descriptor pointer from the remote QM for receiving packets (the transaction is sent to local HyperLink slave port) the address 0x4000_0000 is translated by the HyperLink to 0x34020000 (QM address in remote QM)

- The transaction is sent to the remote side through HyperLink to the remote QM, and Tx descriptor pointer is recycled by the local PktDMA

- PktDMA sends packets to HyperLink address 0x4020_0000

- This write transaction to address 0x4020_0000 is sent to HyperLink slave port and then transported to the remote device. At the remote device, the address 0x4020_0000 is translated by the HyperLink to 0x0C01_0000, and data is sent to the MSMC SRAM in the remote device.

- PktDMA pushes descriptor pointer 0x4000_0000 to the Rx queue in the remote QM. The receiving queue is the high priority accumulation queue (704) in the remote device.

- The application DSP in the remote QM pops the descriptor pointer to the CPU local memory. When the receiving queue meets the watermark, an interrupt is sent to the CPU in the remote DSP. And CPU takes the descriptor pointer, and start to process the receiving packet

- After processing the packet, the remote DSP recycles the Rx descriptor back to the Rx free queue

The following figure shows the above procedures and data flow.

- Data flow step detail