CONSOLE COMMANDS FOR ADS1247EVM

'r' = datarate (Use integers 0-9 that corresponds to bits 3:0 of SYS0 register)

‘p’= pga (Integers 0-7 and corresponds to bits 6:4 of SYS0)

'cp' = + input channel (Integers 0-3 and corresponds to bits 5:3 of MUX0)

'cm' = - input channel (Integers 0-3 and corresponds to bits 2:0 of MUX0)

'v' = version

's' = stream data

'd' = get one data byte (word)

'ee1' = write line contents to eeprom buffer 1.  Maximum characters is 31.

'ee2' = write line contents to eeprom buffer 2.  Maximum characters is 15.

'ee3' = write line contents to eeprom buffer 3.  Maximum characters is 15.

'ee4' = write line contents to eeprom buffer 4.  Maximum characters is 15.

'ee5' = write line contents to eeprom buffer 5.  Maximum characters is 15.

‘er’ = Read configuration data from EVM I2C EEPROM.  Data returned ‘|’ 			delimited ending in a CRLF.  Format is similar to:

ADS1247EVM|6496961-A|6496963-A|6496960-A|20090707(CRLF)

'ew' = write eeprom buffers to EVM EEPROM.

'q' = query parameters and display (see below for format)

'k' = calibrate (Self Offset)

'a' = credits

'n' = burn out current source (Integers 0-3 and corresponds to bits 7:6 of MUX0)

'o' = bias output voltage connection and selects which inputs receive a bias voltage.  Returns register setting as two character hex.  To change setting, enter as hex representation of byte. (Setting follows VBIAS 00 to 0F)

'w' = reference select, i.e. internal or external (Integers 0,2 and 3 and corresponds 			to bits 4:3 of MUX1)

'e' = internal voltage reference on or off (Integers 0-2 and corresponds to bits 6:5

of MUX1)

'i' = current delivered by current source (Integers 0-7 and corresponds to bits 2:0

of IDAC0)

'x' = IDAC0 output (Integers 0-3 and ‘:’ for 10/off, and corresponds to bits 7:4 of

IDAC1)

'y' = IDAC1 output (Integers 0-3 and ‘:’ for 10/off, and corresponds to bits 3:0 of

IDAC1)

'f' = GPIO input configuration and selects inputs as analog or GPIO. Returns register setting as two character hex.  To change setting, enter as hex representation of byte. (Setting follows GPIOCFG 00 to 0F)

'u' = GPIO i/o direction. Returns register setting as two character hex.  To change setting, enter as hex representation of byte. (Setting follows GPIODIR 00 to 0F)

'g' = GPIO write outputs.  Returns register setting as two character hex.  To change setting, enter as hex representation of byte. (Again, similar to above and follows register write for GPIODAT 00 to 0F)

'j' = GPIO read inputs. Returns GPIO register setting as two character hex (00 to 			0F.)

'm' = MUX change for internal measurements (Integers 0-7 and corresponds to

bits 2:0 of MUX1)

‘h’ = Full scale calibration coefficient and sets or returns register settings in 6 	character hex format with MSB first.  (Follows register settings for FSC2, 	FSC1 and FSC0)

‘l’ = Offset calibration coefficient and sets or returns register settings in 6 	character hex format with MSB first.  (Follows register settings for OFC2, 	OFC1 and OFC0)

‘z’ = DAC output on MMB3.  Send hex bytes to set output representative of 20 	bits (5 characters MSB first.)  Returns ‘ok’ to acknowledge.

‘id’ = ID bits from the DUT.  From IDAC0 bits 7:4 and returns single character hex.

‘su’ = analog power supply configuration. A ‘0’ sets the supply to +/- 2.5V bipolar operation.  A ‘1’ sets the supply to +5V unipolar.

‘pn’ = DUT power down. Places START pin low with ‘0’ and in power down. A ‘1’ allows conversions and DUT is powered up.

‘rs’ = hardware DUT reset.

‘vw’ = displays characters on LCD.  Maximum of 7 characters are displayed.

‘q’ returns the following format using ‘|’ for delimiter and ends with CRLF:

P0|R0|CP0|CM1|N0|O00|W1|E0|I0|X:|Y:|M0|F00|UFF|J00|LFFFFF5|H404040|ID4|SU1|PN1