--------------------------------------------------------------------------------

-- DVI_top.vhd

--

-- Digital video INTERFACE color bar generator

--

--

--

--

--------------------------------------------------------------------------------

--

library ieee;

use ieee.std_logic_1164.all;

use ieee.std_logic_unsigned.all;

--use ieee.std_logic_arith.all;

use ieee.numeric_std.all;

Library UNISIM;

use UNISIM.vcomponents.all;

entity DVI_top is

generic (

image_width_i   : integer   := 1920;         --- 640 FOR 1280X1024

sig_hsync : integer :=88;                  --- 56 FOR 1280X1024

hbporch : integer :=  148;               --- 70 FOR 1280X1024

sig_hfproch : integer := 88;                --- 12 FOR 1280X1024

image_height_i   : integer   := 1080;        --- 1024 FOR 1280X1024

sig_Vsync : integer := 5;                   --- 3 FOR 1280X1024

sig_VBproch : integer := 36;                --- 38FOR 1280X1024

sig_VFproch : integer := 4  ;                --- 1 FOR 1280X1024

DATA_width        : integer   := 8

);

port (

sys_clk_i			:in  std_logic;                                 -- clock input

sys_rst_i			:in  std_logic;                                 -- async reset input

dvi_data_o 		:out std_logic_vector(23 downto 0);           -- video output

dvi_h_sync_o		:out std_logic;                                 -- horizontal sync out

dvi_v_sync_o		:out std_logic;                                 -- vertical sync out

dvi_DE_o		   :out std_ulogic ;

dvi_clk_p_o 	   :out std_logic ;

dvi_clk_n_o 	   :out std_logic ;

dvi_CTL_o       :out std_logic_vector(2 downto 0)

);

end DVI_top;

architecture Behavioral of DVI_top is

COMPONENT Bram_pattern

PORT (

clka : IN STD_LOGIC;

wea : IN STD_LOGIC_VECTOR(0 DOWNTO 0);

addra : IN STD_LOGIC_VECTOR(4 DOWNTO 0);

dina : IN STD_LOGIC_VECTOR(23 DOWNTO 0);

clkb : IN STD_LOGIC;

addrb : IN STD_LOGIC_VECTOR(4 DOWNTO 0);

doutb : OUT STD_LOGIC_VECTOR(23 DOWNTO 0)

);

END COMPONENT;

COMPONENT dcm_dvi

PORT(

CLKIN1_IN : IN std_logic;

RST_IN : IN std_logic;

CLKOUT0_OUT : OUT std_logic;

--		CLKOUT1_OUT : OUT std_logic;

LOCKED_OUT : OUT std_logic

);

END COMPONENT;

signal LOCKED  :std_logic ;

signal RSTn : std_logic ;

signal pixelcount_sig        : std_logic_vector(15 downto 0);

signal linecount_sig       : std_logic_vector(15 downto 0);

signal testpattern_sig : std_logic_vector(23 downto 0);

signal testpat_wrreq_sig  : std_logic;

signal white_sig  :integer;

signal yellow_sig : integer;

signal cyan_sig : integer;

signal green_sig: integer;

signal magenta_sig: integer;

signal red_sig: integer;

signal blue_sig: integer;

signal black_sig : integer;

signal nonactive : integer  ;

signal sig_hbporch : integer  ;

signal DATA_byte_reg_i : integer  := 4;

signal testenable : std_logic;

signal hsync_sig: std_logic;

signal vsync_sig: std_logic;

signal hfull_sig: std_logic;

signal vfull_sig : std_logic;

signal start,sig1 : std_logic;

signal dvi_clk : std_logic;

signal rd_en : std_logic;

signal sig_ena :  STD_LOGIC;

signal sig_wea :  STD_LOGIC_VECTOR(0 DOWNTO 0);

signal sig_addra :  STD_LOGIC_VECTOR(15 DOWNTO 0);

signal sig_dina :  STD_LOGIC_VECTOR(23 DOWNTO 0);

signal data_in : std_logic_vector(23 downto 0)  := (others=>'0') ;

signal wr_addra :  std_logic_vector(4 DOWNTO 0):= (others=>'0') ;

signal rd_addrb :  std_logic_vector(4 DOWNTO 0):= (others=>'0');

--	signal image_width_i   : integer   ;

--	signal sig_hsync : integer ;

--	signal hbporch : integer;

--	signal sig_hfproch : integer;

--	signal image_height_i   : integer ;

--	signal sig_Vsync : integer ;

--	signal sig_VFproch : integer;

--	signal sig_VBproch : integer;

begin

dcm_dvi_clk: dcm_dvi PORT MAP(

CLKIN1_IN => sys_clk_i	 ,

RST_IN => not sys_rst_i	,

CLKOUT0_OUT => dvi_clk , -- 147.22 mhz

--		CLKOUT1_OUT =>clk_54mhz , --- 53.33 mhz

LOCKED_OUT => LOCKED

);

dvi_clk_p_o <= dvi_clk;

dvi_clk_n_o <= '0';

RSTn   <= LOCKED;

dvi_CTL_o   <=  "101";

----- Resoultion parameters ----------

--image_width_i  <=conv_integer(H_DE);

--sig_hsync      <=conv_integer(H_sync);

--hbporch        <=conv_integer(H_B_porch) ;

--sig_hfproch    <=conv_integer(H_F_porch);

--image_height_i <=conv_integer(V_DE);

--sig_Vsync      <=conv_integer(V_Sync);

--sig_VBproch    <=conv_integer(V_B_porch);

--sig_VFproch    <=conv_integer(V_F_porch);

RGB_pattern : Bram_pattern

PORT MAP (

clka => dvi_clk,

wea => sig_wea,

addra => wr_addra,

dina => data_in,

clkb => dvi_clk,

addrb => rd_addrb,

doutb => testpattern_sig

);

process(RSTn,dvi_clk)

begin

if(RSTn = '0') then

sig1 <= '0';

rd_addrb <= (others => '0');

elsif (rising_edge(dvi_clk)) then

if  ( rd_en  ='1' and testpat_wrreq_sig = '1' )then

rd_addrb <= rd_addrb + '1';

elsif (  testpat_wrreq_sig = '0') then

rd_addrb <= (others => '0');

end if;

end if;

end process;  ----------------------

--  ---------------- vedio Hsync,Vsync,DE gen ---------

process(RSTn,dvi_clk)

begin

if(RSTn = '0') then

pixelcount_sig <= (others => '0');

elsif (rising_edge(dvi_clk)) then

if (pixelcount_sig=(image_width_i+sig_hbporch+sig_hfproch)-1)then

pixelcount_sig <= (others => '0');

else

pixelcount_sig <= pixelcount_sig + '1';

end if;

end if;

end process;

testpat_wrreq_sig <= '1'	 when ((pixelcount_sig > sig_hbporch)and (pixelcount_sig <=(image_width_i+sig_hbporch))) else '0';

hsync_sig <= '1' when ((pixelcount_sig > (sig_hsync-1)) and (pixelcount_sig <= (image_width_i+sig_hbporch+sig_hfproch)-1))else '0';

dvi_h_sync_o<=  hsync_sig;

dvi_DE_o <= testpat_wrreq_sig;

sig_hbporch <= sig_hsync + hbporch ;

nonactive <= sig_hbporch-1;

--------------------------------------------------------------------------------

--Line counter and Vsync

--------------------------------------------------------------------------------

process(RSTn,dvi_clk)

begin

if(RSTn = '0') then

linecount_sig <= (others => '0');

elsif rising_edge(dvi_clk) then

if (linecount_sig=(image_height_i+sig_Vsync+sig_VBproch+sig_VFproch)-1)then

linecount_sig <= (others => '0');         --linecount_sig;

elsif(pixelcount_sig=(image_width_i+sig_hbporch+sig_hfproch)-1)then

linecount_sig <= linecount_sig + '1';

end if;

end if;

end process;

vsync_sig <= '1' when ((linecount_sig> (sig_Vsync-1)) and (linecount_sig<= (image_height_i+sig_Vsync+sig_VBproch+sig_VFproch)-1))else '0';

dvi_v_sync_o <=   vsync_sig;  ---

--------------------------------------------------------------------------------

--test pattern out

--------------------------------------------------------------------------------

white_sig <= ((image_width_i)/8);

yellow_sig <= 2*white_sig;

cyan_sig <= 3*white_sig;

green_sig <= 4*white_sig;

magenta_sig <= 5*white_sig;

red_sig <= 6*white_sig;

blue_sig <= 7*white_sig;

black_sig <= 8*blue_sig;

rd_en <= '1' when  ((pixelcount_sig = white_sig+nonactive or pixelcount_sig = yellow_sig+nonactive or pixelcount_sig = cyan_sig+nonactive or pixelcount_sig = green_sig+nonactive or pixelcount_sig = magenta_sig+nonactive

or pixelcount_sig = red_sig+nonactive or pixelcount_sig = blue_sig+nonactive or pixelcount_sig = black_sig+nonactive))	else

'0';

dvi_data_o <= testpattern_sig(23 downto 0)	;

end Behavioral;