### Date: Dec 6, 2012

A) Transformer for 5V / 0.8A, 12V / 0.65 & 5V / 0.5A SMPS:

- Core: EE2507, N87 or eqlt, air gap in each limb = 0.2mm (8 mil), Vertical Bobbin, 10 Pins, No margin tape .

- W1A, ½ Primary Winding:  37T, 1 strand x 39 SWG, ONE layer, uniformly wind to cover the bobbin width. Start: Pin 4, Finish: Pin 3.

- 1 layer of 2 mil tape.

- W2, Bias Winding : W2, 86T, 1 strands x 43 SWG, uniformly wind to cover the bobbin width. Start pin 5 & finish pin 2.

- 3 Layers of 2-mil tape.

- W3, 5V / 800mA Secondary Winding: W3 is 24T, 1 strand x 29 SWG, wind from one end of the bobbin. Start: Pin 10 &  Finish: Pin 9 .

- 2 layers of 2 mil tape.

- 12V / 1A Secondary winding : 58T, 1 strand x 29 SWG, TWO layers, wound 29T per layer,  uniformly to cover the bobbin width. Start pin 6 & Finish pin 7.

- 3 layers of 2 mil.

- W1, remaining ½ primary winding : 37T, 1 strand x 39 SWG, ONE layer, uniformly wind to cover the bobbin width. Start: Pin 3, Finish: Pin 1

- Fasten core halves ensuring uniform air-gap in all limbs.

- Primary Inductance, Pin 1-4: 315uH +10% (measure at 40kHz)

- Leakage Inductance, Pin 1-4, pins 6 to 10 shorted: approx 1.2uH (measure at 40kHz)

- Dip impregnate with varnish

- Use equal air gap on each of the limbs.

- Put 4 mm wide copper foil as FLUX Band .

Regards,

Atul Bhakay