How to flash the DSP User Bootloader into SPI-Flash using the DSP Flash Utility

- Open CCS3.3 and connect to the DSP.

- Open project “spiflash_writer_dsp.pjt” in folder spi_flash_writer\ccsv3.3.

- Build the flash utility project with either debug or release configuration.

- Execute the flash utility. It should print out the following message. A stdin window will also be popped out to ask to input the type of binary to be flashed. Please type “dspais” in the window.

SF: Got idcode ef 30 16 SF: Detected W25X32 with page size 256, total 4194304 bytes Flash page size: 256 bytes Flash sector size: 4096 bytes Starting SPIWriter. Enter the image type (one of "dspais" "armubl" "uboot" "other")

- The second stdin window will be popped out to ask to input the path of the user bootloader binary file. Please supply the path of the file to be flashed. Note that the DSP user bootloader to be flashed needs to be in AIS format. For details on how to prepare for an AIS bootloader, please refer to TI application report SPRAB04.

- The flash utility will continue printing out some messages, indicating that it is flashing the bootloader into SPI-flash. Once it runs successfully, the following message will be printed out.

Reading verifying the file.. Files matched

- Disconnect CCS from DSP and switch off the board. Set the SW2 switch on board as follows.

| Pin# | 7 | 2 | 1 | 0 | 3 |
| --- | --- | --- | --- | --- | --- |
| Position | 0 | 1 | 0 | 1 | x |

- Power on the board again and the DSP should boot from SPI flash.