Launch target configuration and connect to core 0

Load eeprom_writer to core 0

Load i2crom….

Into address 0x0c000000

Also have changed eepromwriter_input.txt to

file_name  = i2crom_0x51_c6670_le.bin

bus_addr   = 0x51

start_addr = 0

swap_data  = 0

The result of which is shown

Press F8 in Console to run eeprom writer

Now with core 0 still connected, load i2cparam to core 0

Press F8 to start this program

Open GEL file

Run Scripts->c6678 (not shown)

Press return in Console to complete

Now terminate and disconnect.  Change switches.