HSDC Pro & TW14J56 Triggered Cap/Gen configuration:

- To use the trigger on 14J56 board we need to do couple of modification on the board as listed below,

- Remove R241

- Remove R242

- Replace C61 with ‘0’ ohm resistor

- ‘REF_OSC_IN’ is the SMA connector to be used to provide the Trigger input.

- In the ADC/DAC Device INI, present under ‘14J56 Details’ folder of HSDC Pro installation directory, we need to ‘add/update’ the below shown Parameter so that it configures the SMA as trigger source. (Figure 2)

“Is Capture Trigger SMA = 1”

- Remove the ‘Trigger Options’ string from the ‘Menu Disable’ parameter of the INI file present in the below given HSDC Pro installation path (Figure 2),

C:\Program Files (x86)\Texas Instruments\High Speed Data Converter Pro\14J56 Details\Device and File Info.ini

- Then in HSDC Pro, enable the Trigger Mode from the menu option (Data Capture Options -> Trigger Option) before capturing. (Refer the HSDC Pro User manual for different Trigger mode description.)

Figure 1: Snapshot of the Schematics

Figure 2:

Figure3: