#include<p24FJ256GB106.h>

/****************************** CONFIGURATION **************************************/

_CONFIG1( JTAGEN_OFF & GCP_OFF & GWRP_OFF & FWDTEN_OFF & ICS_PGx3 )

_CONFIG2( PLL_96MHZ_ON & IESO_OFF & FCKSM_CSDCMD & DISUVREG_OFF & OSCIOFNC_OFF & POSCMOD_HS & FNOSC_PRIPLL & PLLDIV_DIV5 & IOL1WAY_ON)

/***********************************************************************************/

typedef unsigned char 	UINT8;

typedef signed char		INT8;

typedef unsigned int 	UINT16;

typedef signed int		INT16;

typedef unsigned long 	UINT32;

typedef signed long int INT32;

#define ADC_START PORTDbits.RD11

#define ADC_DRDY PORTDbits.RD0

#define ADC_RESET PORTCbits.RC14

void Pin_Remap(void);

void init_spi(void);

void delay(UINT16);

INT16 ADC_VALUE=0;

INT16 main (void)

{

TRISDbits.TRISD11 = 0;

TRISDbits.TRISD0 = 1;

TRISCbits.TRISC14 = 0;

Pin_Remap();

init_spi();

delay(30);

ADC_RESET = 0;

delay(20);

ADC_RESET = 1;

ADC_START = 1;				// Begin Conversion

delay(210);

SPI1BUF = 0x17;		// command to stop reading data continuously

delay(210);

SPI1BUF = 0b01000001;    	  // command for writing to VBIAS reg

delay(50);

SPI1BUF = 0b00000010;	// Command  for selectng number of bytes to be written

delay(50);

SPI1BUF = 0b00000000;			// Data to VBIAS Reg

delay(50);

SPI1BUF = 0b00100000;		// Data to MUX1 Reg

delay(50);

SPI1BUF = 0b01010000	;	// Data to SYS0 Reg

delay(50);

SPI1BUF = 0b01001010;     	  // command for writing to IDAC0 reg

delay(50);

SPI1BUF = 0b00000001;		// Command  for selectng number of bytes to be written

delay(50);

SPI1BUF = 0b00000100	;	// Data to IDAC0

delay(50);

SPI1BUF = 0b00000001;			// Data to IDAC1

delay(50);

SPI1BUF = 0x15;		//Read data continuously

delay(210);

while(1)

{

SPI1BUF= 0xFF;				// NOP

SPI1BUF= 0xFF;				// NOP

SPI1BUF= 0xFF;				// NOP

delay(210);

if(SPI1STATbits.SPIRBF==1)

{

ADC_VALUE=SPI1BUF;		// read register value

}

delay(210);

}

}

void __attribute__((__interrupt__, auto_psv)) _SPI1Interrupt(void)

{

IFS0bits.SPI1IF = 0;

flag = 1;

delay(100);

}

void Pin_Remap(void)

{

/* Unlock Registers */

OSCCON = 0X46;

OSCCON = 0X57;

OSCCONbits.IOLOCK = 0;

/*SPI1*/

RPOR1bits.RP3R = 8;		// Configure Output Functions for SPI1

RPOR2bits.RP4R = 7;

RPINR20bits.SDI1R = 2;	// Configure Input Functions for SPI1

/*Lock Registers*/

OSCCON = 0X46;

OSCCON = 0X57;

OSCCONbits.IOLOCK = 1;

}

void init_spi (void)

{

//  IFS0bits.SPI1IF = 0;

// IEC0bits.SPI1IE = 1;                        //SPI1 Interrupt enable

//IPC2bits.SPI1IP = 5;

SPI1STATbits.SPIRBF = 0;

SPI1CON1bits.PPRE = 2;				// Primary Prescaler

SPI1CON1bits.SPRE = 6;				// Secondary Prescaler

SPI1CON1bits.DISSCK = 0;			// Internal Serial Clock is Enabled

SPI1CON1bits.DISSDO = 0;			// SDO1 pin is controlled by the module

SPI1CON1bits.MODE16 = 1;			// Communication is word-wide (16 bits)

SPI1CON1bits.SMP = 0;				// Input data sampled at end of data output time

SPI1CON1bits.CKP = 0;				// Idle state for clock is a low level; active state is a high level

SPI1CON1bits.CKE = 0;				// Output data changes on transition from active to Idle clock state

SPI1CON1bits.MSTEN = 1;				// Enable Master Mode

SPI1STATbits.SPIEN = 1;				// Enable SPI1 module

//ADC_START = 1;

//delay(500);

//while(SPI1STATbits.SPITBF);			// Cleared by hardware when transfers data from SPI2TXB to SPI2SR

//SPI1BUF = 0b01001010;               // command for writing to mux1 reg

}

void delay(UINT16 delay)

{

UINT16 count1,count2;

for(count1 = 0; count1 < delay; count1++)

{

for(count2=0; count2 < 2133; count2++);  //one milli second delay

}

}