Matlab interface for TSW1200 Rev B (firmware 0.3)

May 20, 2008

- Installation Instructions

  - Before running the matlab scripts, unzip the “Matlab_fix_serial.zip” to the default directory.  It should unzip to “C:\MATLAB\R2007b\java\patch\com\mathworks\toolbox\instrument\ where “C:\MATLAB\R2007b” is replaced by the Matlab install directory.

This fixes know issues with Matlab and serial port communications.  It has been tested in Matlab 2007

- Usage

  - Run “makeCapture.m”, which will ask the user for several questions:

    - Q1)Enter Sampling Frequency [MHz] <>   \\Enter the ADC Sampling frequency – this should coincide with the signal generator

    - Q2)Enter Input Frequency [MHz] <>  \\Enter approximately the desired input tone

    - Q3) Number of Samples? [32768]  <>  \\Enter the number of samples, in a power of 2 format  (ex. 4096 up to 65536)

    - A1) Coherent Input Frequency is XYZ <> At this point the software will display the coherent input frequency, please use this to program the signal generator for the analog input.

    - Q4) ADC Name? {ads548x,ads6149,ads6129,ads62c1x,ads62p4x,ads62p2x} [ads6149]  <>    \\Type in the adc name from the list.  Only supported ADCs are displayed here.

    - A2) FPGA version is 0.3  \\This is meant for debugging, the firmware version should be 0.3 or larger.

    - A3) Further details are 1 \\This is also for debugging, albeit it is more cryptic.  This integer value tells the factory what type of interface firmware is loaded into the TSW1200.  It should match the connected ADC.  (1=Parallel DDR, 2 = 2 wire 14bit, 3 = 2 wire 12bit, 4 = 1 wire 14 bit, 5 = 1 wire 12 bit)

  - Capture is completed and results are displayed. Basic AC Performance attributes are labeled on the plot.  Typical result plots are shown in the following pages.

The raw time domain data is now accessible in Matlab under the vector name “ADCout” in which further processing can be done on the data.

ADS5483 results

ADS62C15 results