/*

* app_flash.c

*

*  Created on: 05-Dec-2013

*/

/*

* 		WRITE MODE

* 			  BLKWRT   WRT

* 				0		1			BYTE/WORD WRITE

* 				1		1			BLOCK WRITE

*

*		ERASE FLASH MEMORY

*			MERAS	ERASE

*				0		1			SEGMENT ERASE

*				1		0			MASS ERASE ALL MAIN MEMORY SEGMENT

*				1		1			ERASE ALL FLASH MEMORY

*

*		IN ERASE MODE ANY WRITE CAN ERASE WHOLE SEGEMNT OR AS PER SELECTION.

*		WITHOUT ANY CONFIGURATION ANY WRITE OPERATION IGNORED

*

*		As per data sheet flash controller Clock Frequency should be 257-476 KHz.

*

*/

#define BLOCK_SIZE					64

#define SEGMENT_SIZE				0x80

#define SEGMENT_A_START_ADDRESS		0x1000

#define SEGMENT_B_START_ADDRESS		0x1080

#define SEGMENT_A_SIZE				0X80

#define SEGMENT_B_SIZE				0X80

#define FLW_KEY						0xA500				// key that needed at every write cycle

#define FL_MCLK						0x0040				// MCLK for flash controller

#define FL_DIV20					0x0013				// clock is devided by value + 1

#define SET_LOCK					FCTL3 = FLW_KEY + LOCK				// Lock the any write or erasing operation(ignored) to flash.

#define CLEAR_LOCK					FCTL3 = FLW_KEY						// Clear the lock to enable write and erasing.

#define WRITE_BYTE_MODE				FCTL1 = FLW_KEY + WRT				// set write byte or word mode

#define WRITE_BLOCK_MODE			FCTL1 = FLW_KEY + BLKWRT + WRT		// set block write mode

#define CLEAR_WRITEBIT				FCTL1 = FLW_KEY

#define SEGMENT_ERASE				FCTL1 = FLW_KEY +ERASE				// Set segment erase mode

#define MAINMEMORY_ERASE			FCTL1 = FLW_KEY +MERAS				// Set main memory erase mode

#define FLASH_ERASE					FCTL1 = ERASE + MERAS				// Set flash erase mode

#define WAIT_FORLOCK				while((FCTL3 & LOCK))

#define WAIT_FORNEXTBYTE			while(!(FCTL3 & WAIT))				// During block write cycle

// After every byte write we have to wait for this byte set

// This says that i am ready for nex byte

/************************************************************************************************

* 		FUNCTION		:	InitFlashController

* 		DISCRIPTION		:	INIT THE FLASH CONTROLLER TO CONTROLL THE OPERATION (WRITE/ERASE).

************************************************************************************************

*/

void InitFlashController() {

FCTL2 = FLW_KEY + FL_MCLK + FL_DIV20;						// Clock will be around 400 KHz

// MCLK is selected

}

/************************************************************************************************

* 		FUNCTION		:	Flash_WriteByte

* 		DISCRIPTION		:	This can modify the byte in flash memory

*

* 		NOTE			:	CAN ONLY DO THE BIT 1 VALUE TO 0 VALUE IF YOU WNT TO DO THE REVERSE

* 							ONLY WAY TO ERASE THE SEGMENT AND REWRITE THE SEGMENT

* 							EXAMPLE : 0XFF CAN BE TRANSFERED TO 0XAA, OR ANY OTHER VALUE BUT

* 									  0X00 CAN NOT BE TRANSFERED TO ANY VALUE

************************************************************************************************

*/

void Flash_WriteByte(unsigned int address, unsigned char Write_Data) {

__disable_interrupt();

WRITE_BYTE_MODE;

CLEAR_LOCK;

*((unsigned char*)address) = Write_Data;

SET_LOCK;

__enable_interrupt();

}

/************************************************************************************************

* 		FUNCTION		:	Flash_WriteBlock

* 		DISCRIPTION		:	TO MODIFY VALUE OF THE ENTIRE BLOCK

************************************************************************************************

*/

void Flash_WriteSegment(unsigned int address, unsigned char* Write_Data) {

unsigned char i;

for(i=SEGMENT_SIZE; i; i--) {

Flash_WriteByte(address++, *(Write_Data++));

}

}

/************************************************************************************************

* 		FUNCTION		:	Erase_Segment

* 		DISCRIPTION		:	TO ERASE SEGMENT YOU HAVE TO PASS THE ANY ADDRESS FALL WITHIN THAT

* 							segment RANGE

************************************************************************************************

*/

void Erase_Segment(unsigned int segment_address) {

__disable_interrupt();

SEGMENT_ERASE;

CLEAR_LOCK;

*((unsigned char*)segment_address) = 0;

SET_LOCK;

__enable_interrupt();

}

/************************************************************************************************

* 		FUNCTION		:	ModifyInfoASegment

* 		DISCRIPTION		:	TO MODIFY THE SEGMENT A

************************************************************************************************

*/

void ModifyInfoASegment(unsigned char* Info_MemoryBuffer) {

Erase_Segment(SEGMENT_A_START_ADDRESS);

Flash_WriteBlock(SEGMENT_A_START_ADDRESS, Info_MemoryBuffer);

}

/************************************************************************************************

* 		FUNCTION		:	ModifyInfoBSegment

* 		DISCRIPTION		:	TO MODIFY THE SEGMENT B

************************************************************************************************

*/

void ModifyInfoBSegment(unsigned char *Info_MemoryBuffer) {

Erase_Segment(SEGMENT_B_START_ADDRESS);

Flash_WriteBlock(SEGMENT_B_START_ADDRESS, Info_MemoryBuffer);

}

/************************************************************************************************

* 		FUNCTION		:	ModifyInfoBSegment

* 		DISCRIPTION		:	TO MODIFY INFO SEGMENT

************************************************************************************************

*/

void WriteInfoMemory(unsigned char* Info_MemoryBuffer) {

ModifyInfoASegment(Info_MemoryBuffer);

ModifyInfoBSegment(Info_MemoryBuffer + SEGMENT_A_SIZE);

}