# ISO1228 SPI Flowchart & Algorithm

# Control Pins set by Jumpers on board: 
Burst_EN = GND (Jumper set on Board)
SYNC = VCC1 (Jumper set on Board)

F1 = VCC1 (Jumper set on Board)

F0 = Float

COMM_SEL = controlling by MCU

OUT_EN = controlling by MCU

nRST = controlling by MCU

File: main.c (ISO1228 serial-only integrated)

What this doc contains

- High-level flowchart (mermaid) of the program

- Detailed step-by-step algorithm (initialization, SPI read/write, pin transactions)

- Pin/timing transaction sequences for serial start/stop and register R/W

- Pseudocode / state-machine for the main loop and iso_quick_test

- SPI byte-level examples and explanation of returned bytes

- Quick suggestions for instrumentation and improvements

## Legend / Definitions

- ISO_REG_* — register addresses defined in code (0x00..0x06)

- CS — chip select (active LOW) = ISO_CS_PIN (SP3I_CS)

- COMM_SEL — COMM_SEL pin (selects serial vs parallel) = ISO_COMM_SEL_PIN (PA8)

- nRST — active-low reset pin = ISO_nRST_PIN (PB10)

- OUT_EN — outputs enable (1 = outputs enabled) = ISO_OUT_EN_PIN (PB4)

- HAL_SPI_TransmitReceive(&hspi3, tx, rx, 2, ...) — 2‑byte full-duplex SPI transfer

- Timing notes: delay_us(x) uses DWT; code often uses 1–5 µs and HAL_Delay(ms)

## High-level flowchart (Mermaid)

flowchart TD

Start[Start / Reset MCU]

InitPeripherals[MX_GPIO_Init, MX_USART2_UART_Init, MX_SPI3_Init]

DWTInit[DWT_Delay_Init]

SetCSHigh[iso_cs_set(HIGH)]

BootPinState[Set initial MCU pin states: nRST=LOW, OUT_EN=LOW, COMM_SEL=LOW]

PreSerial[Prepare for serial: COMM_SEL=HIGH, OUT_EN=LOW, nRST=LOW]

SerialStart[iso_serial_start_sequence() -> COMM_SEL=H, release reset, wait, OUT_EN=H]

QuickTest[iso_quick_test()]

ClearFilters[Write 0x00 to FILTER03..06]

Readback[Readback regs 03,00,01,02]

MainLoop[Main loop: periodic reads; handle fault changes]

End

Start --> InitPeripherals --> DWTInit --> SetCSHigh --> BootPinState --> PreSerial --> SerialStart --> QuickTest --> ClearFilters --> Readback --> MainLoop --> End

## Algorithm (high-level steps)

- MCU & peripheral init

  - HAL_Init()

  - SystemClock_Config()

  - MX_GPIO_Init()

  - MX_USART2_UART_Init()

  - MX_SPI3_Init()

  - HAL_Delay(10) to let UART stabilise

  - DWT_Delay_Init() for microsecond delays

- Initial pin state setup

  - iso_cs_set(HIGH) to make CS inactive

  - Drive MCU outputs to mimic board boot wiring:

    - iso_set_nrst(LOW) (hold ISO1228 in reset)

    - iso_set_out_en(LOW) (tri-state outputs)

    - iso_set_comm_sel(LOW) (default)

  - HAL_Delay(5) to settle

- Switch to serial for SPI comms

  - iso_set_comm_sel(HIGH) (enable serial mode on ISO1228)

  - Ensure iso_set_out_en(LOW) and iso_set_nrst(LOW) before starting

  - delay_us(5) to settle

  - Call iso_serial_start_sequence() — sequence below

- iso_serial_start_sequence()

  - iso_set_comm_sel(HIGH) (ensures serial)

  - iso_set_nrst(LOW) then hold (1 µs) (Recommended min 35 nSec)

  - iso_set_nrst(HIGH) (release reset)

  - delay_us(2) (allow SPI logic init) (Recommended min 200 nSec)

  - iso_set_out_en(HIGH) (enable outputs)

  - small delay_us(2) then ready for SPI transfers

- SPI register access primitives

  - iso_spi_write_reg(reg, data, &r0, &r1)

    - Build tx[0] = 0x80 | (reg & 0x7F) (MSB=1 indicates WRITE)

    - tx[1] = data

    - iso_cs_set(LOW); delay_us(1); HAL_SPI_TransmitReceive(...,2); delay_us(1); iso_cs_set(HIGH)

    - Device returns rx[0], rx[1] — function returns them via rcv0/rcv1

  - iso_spi_read_reg(reg, &pdata, &r0, &r1)

    - tx[0] = reg & 0x7F (MSB=0 indicates READ)

    - tx[1] = 0x00

    - Same CS toggling and transfer as write

    - pdata is set to rx[1] (per debug log: second response byte is register value)

- iso_quick_test()

  - Print START

  - Perform three cycles of raw reads of registers INPUT_DATA (0x00), WIRE_BREAK (0x01), FAULT (0x02); log rx0, rx1, rx1 as value

  - Re-init sequence: set COMM_SEL=H, toggle nRST (low then high), set OUT_EN=H and read INPUT_DATA

  - Toggle OUT_EN low briefly and re-read

  - Change SPI prescaler (fast -> prescaler=16) one read; restore prescaler=2

  - Test changing CPHA from 1EDGE to 2EDGE for one read; restore

  - Print inverted-bit sample for debugging

- Filter init and readback

  - Write 0xBB to FILTER03 and print burst-style debug

  - Write zero to FILTER03..FILTER06

  - Read-back registers FILTER03 (0x03), INPUT_DATA (0x00), WIRE_BREAK (0x01), FAULT (0x02) and print

- Main loop

  - Repeatedly read registers: INPUT_DATA, WIRE_BREAK, FAULT

  - Print read values (or "SPI read error" if any HAL call fails)

  - If FAULT changed from prev_fault:

    - update prev_fault

    - If fault != 0: set COMM_SEL=H, delay_us(2), call iso_serial_start_sequence() to ensure SPI ready, re-read WIRE_BREAK and FAULT and print

  - HAL_Delay(250) (4 Hz sampling)

- Error handling

  - Error_Handler() disables IRQs and spins in while(1)

## Pin & SPI transaction timing sequences (detailed)

### Serial START sequence (timing used in code)

- COMM_SEL = HIGH (ensures serial mode)

- nRST = LOW (hold reset)

  - code uses delay_us(1) (>=35 ns required by spec)

- nRST = HIGH (release reset)

- delay_us(2) (>=200 ns for SPI logic to power up)

- OUT_EN = HIGH (enable outputs)

- delay_us(2) before starting SPI

CS behavior for each SPI transfer

- CS asserted by iso_cs_set(LOW) before transfer

- delay_us(1) then HAL_SPI_TransmitReceive(..., 2)

- delay_us(1) then iso_cs_set(HIGH) to release

### SPI READ (reg 0x00 example)

- Master transmits: [reg_byte, 0x00] where reg_byte = reg & 0x7F (MSB=0)

- Slave returns: [rx0, rx1] where rx1 is documented as register value

- Example: master sends [0x00, 0x00] → receive [0x00, 0xAB] → register value = 0xAB

### SPI WRITE (reg 0x03 example)

- Master transmits: [0x80 | reg, data] (MSB=1 indicates WRITE)

- Slave returns two bytes (status or previous data) captured as rx[0], rx[1]

- Example: to write 0xBB to reg 0x03: master sends [0x83, 0xBB]

## Byte-level map & example logs from code

- Read INPUT_DATA (0x00): send [0x00 0x00] rcv [rx0 rx1] → value rx1

- Write FILTER03 (0x03) with 0xBB: send [0x83 0xBB] rcv [rx0 rx1]

- Code prints lines like:

  - RAW IN: rx0=0x00 rx1=0x00 val=0x00

  - BURST W: send[83 BB] rcv[00 00]

These example logs match how the iso_spi_* helpers capture and print rx[0] and rx[1] returned by the device.

## State machine / Pseudocode (condensed)

// Initialization

HAL_Init(); SystemClock_Config(); MX_GPIO_Init(); MX_USART2_UART_Init(); MX_SPI3_Init();

DWT_Delay_Init(); iso_cs_set(HIGH);

iso_set_nrst(LOW); iso_set_out_en(LOW); iso_set_comm_sel(LOW); HAL_Delay(5);

// Prepare serial

iso_set_comm_sel(HIGH); iso_set_out_en(LOW); iso_set_nrst(LOW); delay_us(5);

iso_serial_start_sequence(); // releases reset, sets OUT_EN HIGH

iso_quick_test();

// Clear filters

iso_spi_write_reg(0x03, 0xBB, &r0,&r1); // prints

iso_spi_write_reg(0x03,0x00,NULL,NULL); // and others

// Readback registers

iso_spi_read_reg(0x03,&val03,&r0,&r1);

...

// Main loop

prev_fault = 0xFF;

while(1) {

if (iso_spi_read_reg(0x00,&in) == OK && ... ) {

print IN/WB/FAULT

} else print error

if (fault != prev_fault) {

prev_fault = fault;

if (fault != 0) {

iso_set_comm_sel(HIGH); delay_us(2);

iso_serial_start_sequence();

iso_spi_read_reg(0x01,&wb2...);

iso_spi_read_reg(0x02,&fault2...);

}

}

HAL_Delay(250);

}

## iso_quick_test() flow (more detailed)

- Print header

- For i=0..2: read INPUT_DATA, WIRE_BREAK, FAULT (using iso_spi_read_reg) and print rx0, rx1, value for each

- Re-init: COMM_SEL=H; nRST low->high pulse; OUT_EN=H → delay

- Read INPUT_DATA and print

- Toggle OUT_EN low for 5 ms then high and read INPUT_DATA

- Temporarily change SPI prescaler to SPI_BAUDRATEPRESCALER_16 and do one read; restore prescaler

- Temporarily change CPHA to SPI_PHASE_2EDGE and do one read; restore

- Print inverted view sample and footer

## Notes & suggestions

- CS timing: the code asserts CS -> small delay -> SPI transfer -> small delay -> deassert CS. This is safe; keep delay_us(1) as the device seems tolerant.

- When changing HAL SPI init parameters (prescaler/phase), the code disables SPI, changes fields, re-inits, and re-enables — this is correct.

- If you need guaranteed atomic sequences between multiple registers, consider using a burst mode (if device supports) or keep CS asserted across multiple HAL_SPI_TransmitReceive calls.

- Add timeout or error counters when repeated HAL_SPI_TransmitReceive calls fail to allow recovery (e.g. re-init SPI or toggle reset).

- Consider reporting rx0 semantics: it often appears zero in logs — check datasheet for meaning (status flags?).

## What I included in this document (checklist)

- Flowchart (mermaid)

- Full algorithm / step-by-step walkthrough

- Detailed pin & SPI transaction timings

- Pseudocode and state machine for main loop and quick test

- Byte-level examples and how the code interprets rx[0]/rx[1]

- Suggestions for robustness

If you want, I can:

- Convert the mermaid flowchart into an image and attach it

- Produce a printable PDF of this document

- Generate an annotated sequence diagram (per-pin timing) as an image

- Add comments inline in your source to match each step