AWR1642 EVM issues description:

My development environment is the following：

- I bought the TI AWR1642 Evaluation Module mmWave sensing solutions, part number: AWR1642BOOST from Internet.

- Per the document ”Getting Started Guide: TI mmWave Lab Driver Vital Signs ”（Guide) burn the “xwr16xx_vitalSigns_lab.bin” successfully。The jumper settings are 101.

- Question1： per the Guide, the bin file name is “xwr16xx_mmw_demo.bin”. But I’d like to check that, I should load the file “xwr16xx_vitalSigns_lab.bin” if I want to demostrate the heartRate and the breathing rate, right?

- The two serial ports can be found in the devices management，as the following.

- Question2：these two serial ports’ Baud rate are both default 9600. Do I need to set them to any special value?

- Checked the power supply and the USB are both connected correctly.

- Question3：My power supply is 5V/3A. Is it sufficient to support the radar antenna to work correctly?

- Per the Guide to start the PC host tool “VitalSignsRadar_Demo.exe”. But when starting, the console will output a warning “QMetaObject::connectSlotsByName: No matching signal for on_buttonBox_accepted().”

- Per the information from Internet, I have changed a function’s name as the following in “Qt Creator 4.13.1” development IDE for the source code. Rebuilt and run it, the warning has gone.

//void DialogSettings::on_buttonBox_accepted()

void DialogSettings::on_buttonBoxAccepted()

- The output is the following：

10:37:34: Starting D:\vitalSigns_host\gui\build-VitalSignsRadar_Demo-Desktop_Qt_5_12_10_MinGW_32_bit-Debug\debug\VitalSignsRadar_Demo.exe ...

QT version =  5.12.10

Serial Port Found

Data Port Number is "COM7"

User Port Number is "COM8"

- Click the “start” in the “VitalSignsRadar_Demo.exe”，the output is the following：

"COM8"

Configuration File Path is %s/n "D:/vitalSigns_host/gui/build-VitalSignsRadar_Demo-Desktop_Qt_5_12_10_MinGW_32_bit-Debug/profiles/profile_2d_VitalSigns_20fps_Back.cfg"

"sensorStop"

"flushCfg"

"dfeDataOutputMode 1"

"channelCfg 15 3 0"

"adcCfg 2 1"

"adcbufCfg -1 0 0 1 0"

"profileCfg 0 77 7 6 57 0 0 70 1 100 2000 0 0 40"

"chirpCfg 0 0 0 0 0 0 0 1"

"frameCfg 0 0 2 0 50 1 0"

"lowPower 0 0"

"guiMonitor 0 0 0 0 1"

"vitalSignsCfg 0.1 0.7 256 320 1 0.1 0.05 300000 300000"

"motionDetection 1 20 0.04 0"

"sensorStart"

""

""

"sensorStop"

"flushCfg"

"dfeDataOutputMode 1"

"channelCfg 15 3 0"

"adcCfg 2 1"

"adcbufCfg -1 0 0 1 0"

"profileCfg 0 77 7 6 57 0 0 70 1 100 2000 0 0 40"

"chirpCfg 0 0 0 0 0 0 0 1"

"frameCfg 0 0 2 0 50 1 0"

"lowPower 0 0"

"guiMonitor 0 0 0 0 1"

"vitalSignsCfg 0.1 0.7 256 320 1 0.1 0.05 300000 300000"

0.1

0.7

1

"motionDetection 1 20 0.04 0"

"sensorStart"

""

""

Chirp Duration in us is : 50

Chirp Bandwidth in kHz is :  3500

Maximum Range in Meters is : 4.28571

Range-FFT size is : 128

Range-Bin size is : 0.0334821

Range-Bin Start Index is : 2

Range-Bin End Index is : 20

Total Payload size from the UART is : 220

Padding Factor is : 7

Total Payload size from the UART is : 224

And the screen is the following:

- Unfortunately，There was nothing in the screen as the snapshot above. The board status was LED DS3 off(never on)，jumper status was 001, as the following:

- Per the information from internet, LED DS3 should be on after click the “start” as the following if everything is OK. But I have not seen it in my development environment.

- Quesiton4: what’s the probable reason/issue on my board/enviromnet and the reasonable solutions?