Test procedure:

- Configure the board (JP2, JP6 & S1) as shown in FPD23DAEVM User’s guide for default mode operation.

- Connect 12V DC power supply to J6 or to J1 as shown on silk screen. LED D1 (green) should light up.

- For FPD23DAEVM-001 connect parallel RGB888, HS, VS, DE & PCLK signal to J2. For FPD23DAEVM-002 connect LVDS input to J3.

- Hook up the board to an HDMI/DVI compatible display using an HDMI cable or an HDMI to DVI adapter cable. Check the signal on TP4 (MSEN/PO1), it should remain high after detecting a display device connected to J4.

- Expected output can be observed on the display.