Table 6-4 Control Message Types

| Bits 4..0 | Message Type | Sent by | Description | Valid Start of Packet |
| --- | --- | --- | --- | --- |
| 0 0000 | Reserved | N/A | All values not explicitly defined are Reserved and shall not be used. |  |
| 0 0001 | GoodCRC | Source, Sink or Cable Plug | See Section 6.3.1. | SOP* |
| 0 0010 | GotoMin | Source only | See Section 6.3.2. | SOP only |
| 0 0011 | Accept | Source, Sink or Cable Plug | See Section 6.3.3. | SOP* |
| 0 0100 | Reject | Source or Sink | See Section 6.3.4. | SOP only |
| 0 0101 | Ping | Source only | See Section 6.3.5. | SOP only |
| 0 0110 | PS_RDY | Source or Sink | See Section 6.3.6. | SOP only |
| 0 0111 | Get_Source_Cap | Sink or DRP | See Section 6.3.7. | SOP only |
| 0 1000 | Get_Sink_Cap | Source or DRP | See Section 6.3.8. | SOP only |
| 0 1001 | DR_Swap | Source or Sink | See Section 6.3.9 | SOP only |
| 0 1010 | PR_Swap | Source or Sink | See Section 6.3.10 | SOP only |
| 0 1011 | VCONN_Swap | Source or Sink | See Section 6.3.11 | SOP only |
| 0 1100 | Wait | Source or Sink | See Section 6.3.12 | SOP only |
| 0 1101 | Soft_Reset | Source or Sink | See Section 6.3.13 | SOP* |
| 0 1110- 0 1111 | Reserved | N/A | All values not explicitly defined are Reserved and shall not be used. |  |
| 1 0000 | Not_Supported | Source, Sink or Cable Plug | See Section 6.3.14 | SOP* |
| 1 0001 | Get_Source_Cap_Extended | Sink or DRP | See Section 6.3.15 | SOP only |
| 1 0010 | Get_Source_Status | Source or Sink | See Section 6.3.16 | SOP only |
| 1 0011 | FR_Swap | Sink1 | See Section 6.3.17 | SOP only |
| 1 0100- 1 1111 | Reserved | N/A | All values not explicitly defined are Reserved and shall not be used. |  |
| Note 1: In this case the Port is providing vSafe5V however it will have Rd asserted rather than Rp and sets the Port Power Role field to Sink, until the Fast Role Swap AMS has completed. |  |  |  |  |

Figure 6-23 Source_Capabilities_Extended Message

Table 6-37 Source Capabilities Extended Data Block (SCEDB)

| Offset | Field | Size | Value | Description |
| --- | --- | --- | --- | --- |
| 0 | VID | 2 | Numeric | Vendor ID (assigned by the USB-IF) |
| 2 | PID | 2 | Numeric | Product ID (assigned by the manufacturer) |
| 4 | XID | 4 | Numeric | Value provided by the USB-IF to assign to product |
| 8 | FW Version | 1 | Numeric | Firmware version number |
| 9 | HW Version | 1 | Numeric | Hardware version number |
| 10 | Voltage Regulation | 1 | Bit Field | Bit; Description. 1..0; 00b: 150mA/us Load Step (default) 11b..01b: Reserved and shall not be used. 2; 0b: 25% IoC (default) 1b: 90% IoC. 3..7; Reserved and shall be set to zero. |
| 11 | Holdup Time | 1 | Numeric | Output will stay with regulated limits for this number of milliseconds after removal of the AC from the input. 0x00 = feature not supported Note: a value of 3ms should be used |
| 12 | Compliance | 1 | Bit Field | Bit; Description. 0; LPS compliant when set. 1; PS1 compliant when set. 2; PS2 compliant when set. 3..7; Reserved and shall be set to zero. |
| 13 | Touch Current | 1 | Bit Field | Bit; Description. 0; Low touch Current EPS when set. 1; Ground pin supported when set. 2; Ground pin intended for protective earth when set. 3..7; Reserved and shall be set to zero. |
| 14 | Peak Current1 | 2 | Bit field | Bit; Description. 0..4; Percent overload in 10% increments Values higher than 25 (11001b) are clipped to 250%.. 5..10; Overload period in 20ms. 11.14; Duty cycle in 5% increments. 15; VBUS Voltage droop. |
| 16 | Peak Current2 | 2 | Bit field | Bit; Description. 0..4; Percent overload in 10% increments Values higher than 25 (11001b) are clipped to 250%.. 5..10; Overload period in 20ms. 11.14; Duty cycle in 5% increments. 15; VBUS Voltage droop. |
| 18 | Peak Current3 | 2 | Bit field | Bit; Description. 0..4; Percent overload in 10% increments Values higher than 25 (11001b) are clipped to 250%.. 5..10; Overload period in 20ms. 11.14; Duty cycle in 5% increments. 15; VBUS Voltage droop. |
| 20 | Touch Temp | 1 | Value | Temperature conforms to: 0 = [IEC 60950-1] (default) 1 = [IEC 62368-1] TS1 2 = [IEC 62368-1] TS2 Note: All other values Reserved |
| 21 | Source Inputs | 1 | Bit field | Bit; Description. 0; 0b: No external supply 1b: External supply present. 1; If bit 0 is set: 0b: External supply is constrained 1b: External supply is unconstrained If bit 0 is not set Reserved and shall be set to zero. 2; 0b: No internal Battery 1b: Internal Battery present. 3..7; Reserved and shall be set to zero. |
| 22 | Batteries | 1 | Byte | Upper Nibble = Number of Hot Swappable Batteries (0..4) Lower Nibble = Number of Fixed Batteries (0..4) |