PCIE16X-402EVK User Guide

PCIe 16X Lane Card Evaluation Kit

- General Description:

The PCIE16X-402EVK is a PCIe add-in riser card for PCIe 16x applications. It provides a complete platform to evaluate 4 - DS80PCI402SQ, 4 Lane PCIe repeaters for PCIe system protocol and lane negotiation validation. The card has a 16X PCIe edge fingers at J1 which plugs into a motherboard that has a PCIe 16X connector. The card also has a PCIe 16X connector at J2 for endpoint connection (PCIe graphic card or SATA/SAS raid controller card).

- Features:

- ■ 8 channel PCIe repeater up to 8 Gbps (GEN 3)

- ■ Low power consumption, with option to power down unused channels

- ■ Adjustable receive equalization

- ■ Adjustable transmit VOD and De-emphasis

- ■ IDLE detection — squelch function auto mutes the output

- ■ Programmable via pin selection or SMBus interface

- ■ Single supply operation: VIN = 3.3V±10% or VDD = 2.5V ±5%

- ■ -40°C to +85°C Operation

- ■ >6 kV HBM ESD Rating

- ■ High speed signal flow–thru pin-out package - SQA54A: 54-pin LLP (10 mm x 5.5 mm, 0.5 mm pitch)

- Applications:

■ Extends FR-4 Backplane Trace for PCIe Applications

- PCIE16X-402EVK Demo Kit Contents:

■ End User License Agreement

■ PCIE16X-402EVK User Guide Rev 1.2

■ PCIE16X-402EVK Board

- Ordering Information:

- DEVICE:  DS80PCI402SQE: QTY = 250, DS80PCI402SQ: QTY = 2,000

- SMA Evaluation Kit: PCIE16X-402EVK

Figure 1. PCIE16X-402EVK Evaluation Board

Table 1. Switches to set the 4-level input control pins

| 4 – level Input Settings | Setting for 3 pin switches (3-2-1) |
| --- | --- |
| 0 – Tie 249 ohm to GND | ON – OFF – OFF |
| R – Tie 5k ohm to GND | OFF – ON – OFF |
| F – FLOAT (open) | OFF – OFF – OFF |
| 1 – Tie 249 ohm to VIH | OFF – OFF – ON |

The following switches are used to set the input condition for the 4-level inputs:

SW1, SW2, SW3, SW5, SW6, SW10, SW11.

There are 3 switches connected to an input signal pin. Each switch when set to the ON position sets the pin to one of the 4-level setting. The 6 pin switches are assigned similar to the 3 pin switches. The only difference is 2 signal pins are connected and thus 6-5-4 is for the one signal pin and 3-2-1 is for another signal pin. Please note only 1 switch at the ON position is allowed.

Table 2. Connection and Control Description

| Component | Name | Function |
| --- | --- | --- |
| J1 | PCIE TX/RX | High speed differential TX/RX from/to Root Complex |
| J2 | PCIE TX/RX | High speed differential TX/RX to/from End Point |
| J3, J5 | 3.3V to VIN | 3.3V DC Power – VIN to DS80PCI800SQ Jumper ON = 3.3V mode operation Jumper OFF = 2.5V mode operation |
| J4, J6 | 2.5V to VDD | 2.5V DC Power – VDD to DS80PCI800SQ Jumper ON (1-2, 3-4) = 2.5V mode operation Jumper OFF (1-2, 3-4) = 3.3V mode operation |
| J7 | VIN or VDD | Jumper VIH: set 1-2 = VIN (3.3V) or set 2-3 = VDD (2.5V) |
| J8 | SDA, SCL | Optional SMBUS access pins. See the datasheet for additional information on SMBUS. |
| J9 | EEPROM | Optional socket for EEPROM |
| SW1 | EQB[1:0] or AD[3:2] | PIN MODE – EQ control for channel B inputs SMBUS MODE – AD[3:2] device address bits |
| SW2 | ENSMB | ENSMB = LOW – PIN MODE ENSMB = HIGH – SMBUS (slave mode) ENSMB = FLOAT – SMBUS (master mode – load configuration from EEPROM) |
| SW3 | DEMA[1:0] | PIN MODE – DE control for channel A outputs |
| SW4 | SDA/SCL | “ON” position connects SDA and SCL lines to the device pin. |
| SW5 | DEMB[1:0] or AD[1:0] | PIN MODE – DE control for channel B outputs SMBUS MODE – AD[1:0] device address bits |
| SW6 | SD_TH and LPBK - RES | SD_TH – Signal detect threshold level (FLOAT = Default level) LPBK function for PCI402 and RESERVED for PCI800 (FLOAT = Normal operation) |
| SW7 | VDD_SEL1 VDD_SEL2 | VDD_SEL – Enable or disable the internal 3.3V to 2.5V regulator for U1 and U2. ON connects to GND to enable the internal LDO regulator for 3.3V mode operation. |
| SW8 | READ_EN, RD_EN2, RD_EN3 and RD_EN4 | For manual control of loading the external EEPROM and daisy chain the READ_EN to the ALL_DONE pins. Pin1 = ON connects the SW13 push button to the READ_EN of U1. Pin2,3,4 = OFF |
| SW9 | A_D1 to RD_EN2 … A_D3 to RD_EN4 | Pin1 = ON connects the ALL_DONE of U1 to READ_EN of U2. Pin2 = ON connects the ALL_DONE of U2 to READ_EN of U3. Pin3 = ON connects the ALL_DONE of U3 to READ_EN of U4. Pin4 = OFF |
| SW10 | RXDET and RATE | RXDET – Input internal 50 ohm to VDD terminations RXDET = F (AUTO RX Detect), RXDET = 1 (50 ohm input termination). RATE = 0 (GEN1,2) = 2.5G / 5.0G. RATE = R (GEN3) = 8.0G. RATE = F (AUTO Detect). The RATE auto detect circuit requires the idle and active signal which occurs during the link training negotiation. |
| SW11 | EQA[1:0] | PIN MODE – EQ control for channel A inputs |
| SW12 | SDA/SCL to SMCLK/SMDAT | “ON” connect the SDA/SCL bus to the PCIe SMCLK and SMDAT bus. Default is “OFF”. |
| SW13 | READ_EN | ENSMB = FLOAT – SMBUS (master mode – load configuration from EEPROM) SW6: SD_TH becomes the READ_EN pin. To start the loading at power up, set SW6 pin 3 to “ON” position (pull to GND). To manually control the start, set SW6 to “OFF” position and set SW8 pin1 to “ON” and pin2 to “OFF” position and push the SW13 button for the high to low transition to start the loading. When the loading is complete the LEDs – D1 and D2 light should turn OFF. |
| SW14 | PRSNT | “ON” connects the PCIe PRSNT signal to the device PRSNT pin. For 16X, set all the switches to the “ON” position. |

Quick Start User Guide:

1. Connect J1 – PCIe 16x edge finger to the motherboard (root complex)

2. Connect J2 - PCIe 16x connector to an add-in card (end point).

3. For 3.3V mode operation, set J3 jumper to ON and do not use J4 (leave jumper OFF).

For 2.5V mode operation, set J3 jumper to OFF and set J4 jumper to ON (1-2 and 3-4).

4. Set jumper – J7 for VIH connection to VIN (3.3V) or VDD (2.5V). Default is 1-2 set to ON for VIH = 3.3V.

5. Set SW7 – VDD_SEL1_2 and VDD_SEL3_4 to “ON” position for 3.3V mode operation.

6. Set the control pins for normal operation

SW10 – RXDET = F (continuous receiver detection): set switches (3-2-1) = (OFF-OFF-OFF).

RXDET = 1 (50 ohm input termination): set switches (3-2-1) = (OFF-OFF-ON).

SW10 – RATE = F (enable rate detection): set switches (6-5-4) to (OFF-OFF-OFF).

RATE = R (GEN3 mode): set switches (6-5-4) = (OFF-ON-OFF).

RATE = 0 (GEN1,2 mode): set switches (6-5-4) = (ON-OFF-OFF).

SW6 – SD_TH = F (default signal detect threshold level): set switches (3-2-1) = (OFF-OFF-OFF).

SW6 – LPBK - RES = F (normal operation): set switches (6-5-4) = (OFF-OFF-OFF).

SW8: Set switches to “OFF” position.

SW9: Set switches to “OFF” position.

SW14 – PRSNT = GND (enables the device): set switches to “ON” position.

5. Set the input equalization level.

For external pin mode control of the equalization level:

Set ENSMB = 0 (1kohm to GND) by using the SW2 (3-2-1) = (ON-OFF-OFF).

SW4 pin1,2 must be set to the OFF positions, so the SMBUS signals are disconnected.

Refer to Table 1 for information on the 3 switch settings for the 4 level input.

Example:

Set EQB[1:0] with SW1 for the B bank of inputs (top 2 left inputs of DS80PCI402).

SW1 (6-5-4), (3-2-1) = (OFF-ON-OFF), (OFF-ON-OFF) = EQB[1:0] = R,R = 14.6 dB at 4 GHz (level 6).

Set EQA[1:0]  with SW11 for the A bank of inputs (bottom 2 left inputs of DS80PCI402).

SW8 (6-5-4), (3-2-1) = (OFF-ON-OFF), (OFF-ON-OFF) = EQA[1:0] = R,R = 14.6 dB at 4 GHz (level 6).

The table below is the 16 possible EQ settings.

| Level | EQA/B[1:0] | SW1 - EQB[1:0] SW11 - EQA[1:0] |  |  |  |  |  | EQ (dB) at 4 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 6 | 5 | 4 | 3 | 2 | 1 |  |
| 1 | 0, 0 | ON | OFF | OFF | ON | OFF | OFF | 4.9 |
| 2 | 0, R | ON | OFF | OFF | OFF | ON | OFF | 7.9 |
| 3 | 0, F | ON | OFF | OFF | OFF | OFF | OFF | 9.9 |
| 4 | 0, 1 | ON | OFF | OFF | OFF | OFF | ON | 11.0 |
| 5 | R, 0 | OFF | ON | OFF | ON | OFF | OFF | 14.3 |
| 6 | R, R | OFF | ON | OFF | OFF | ON | OFF | 14.6 |
| 7 | R, F | OFF | ON | OFF | OFF | OFF | OFF | 17.0 |
| 8 | R ,1 | OFF | ON | OFF | OFF | OFF | ON | 18.5 |
| 9 | F ,0 | OFF | OFF | OFF | ON | OFF | OFF | 18.0 |
| 10 | F, R | OFF | OFF | OFF | OFF | ON | OFF | 22.0 |
| 11 | F, F | OFF | OFF | OFF | OFF | OFF | OFF | 24.4 |
| 12 | F, 1 | OFF | OFF | OFF | OFF | OFF | ON | 25.8 |
| 13 | 1, 0 | OFF | OFF | ON | ON | OFF | OFF | 27.4 |
| 14 | 1, R | OFF | OFF | ON | OFF | ON | OFF | 29.0 |
| 15 | 1, F | OFF | OFF | ON | OFF | OFF | OFF | 31.4 |
| 16 | 1, 1 | OFF | OFF | ON | OFF | OFF | ON | 32.7 |

6. Set the output VOD and De-emphasis level.

For external pin mode control for the VOD and De-emphasis level:

Set ENSMB = 0 (1kohm to GND) by using the SW2 (3-2-1) = (ON-OFF-OFF).

SW4 pin1,2 must be set to the OFF positions, so the SMBUS signals are disconnected.

Refer to Table 1 for information on the 3 switch settings for the 4 level input.

Example:

Set DEMB[1:0] with SW5 for the B bank of outputs (top 2 right outputs of DS80PCI402).

SW5 (6-5-4), (3-2-1) = (ON-OFF-OFF), (OFF-OFF-ON) = DEMB[1:0] = 0,1 (VOD=1.0V, DE=0 dB).

Set DEMA[1:0] with SW3 for the A bank of outputs (bottom 2 right outputs of DS80PCI402).

SW3 (6-5-4), (3-2-1) = (ON-OFF-OFF), (OFF-OFF-ON) = DEMA1:0] = 0,1 (VOD=1.0V, DE=0 dB).

The table below is the 16 possible settings of VOD and DE when in pin mode.

In Gen 1/2, the de-emphasis level can be set with the DEMx[1:0] pins, but is not available in Gen 3.

| Level | DEMA/B[1:0] | SW5 - DEMB[1:0] SW3 - DEMA[1:0] |  |  |  |  |  | GEN1,2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 6 | 5 | 4 | 3 | 2 | 1 | VOD (Vp-p) | DE (dB) |
| 1 | 0, 0 | ON | OFF | OFF | ON | OFF | OFF | 0.8 | 0 |
| 2 | 0, R | ON | OFF | OFF | OFF | ON | OFF | 0.9 | 0 |
| 3 | 0, F | ON | OFF | OFF | OFF | OFF | OFF | 0.9 | -3.5 |
| 4 | 0, 1 | ON | OFF | OFF | OFF | OFF | ON | 1.0 | 0 |
| 5 | R, 0 | OFF | ON | OFF | ON | OFF | OFF | 1.0 | -3.5 |
| 6 | R, R | OFF | ON | OFF | OFF | ON | OFF | 1.0 | -6 |
| 7 | R, F | OFF | ON | OFF | OFF | OFF | OFF | 1.1 | 0 |
| 8 | R ,1 | OFF | ON | OFF | OFF | OFF | ON | 1.1 | -3.5 |
| 9 | F ,0 | OFF | OFF | OFF | ON | OFF | OFF | 1.1 | -6 |
| 10 | F, R | OFF | OFF | OFF | OFF | ON | OFF | 1.2 | 0 |
| 11 | F, F | OFF | OFF | OFF | OFF | OFF | OFF | 1.2 | -3.5 |
| 12 | F, 1 | OFF | OFF | OFF | OFF | OFF | ON | 1.2 | -6 |
| 13 | 1, 0 | OFF | OFF | ON | ON | OFF | OFF | 1.3 | 0 |
| 14 | 1, R | OFF | OFF | ON | OFF | ON | OFF | 1.3 | -3.5 |
| 15 | 1, F | OFF | OFF | ON | OFF | OFF | OFF | 1.3 | -6 |
| 16 | 1, 1 | OFF | OFF | ON | OFF | OFF | ON | 1.3 | -9 |

For SMBUS mode control of the EQ, VOD and De-emphasis level:

Set ENSMB = 1 (1kohm to VIH) by using the SW2 (3-2-1) = (OFF-OFF-ON).

Set SW4 pin1,2 to the ON position so the SMBUS signals are connected.

Set SW3 pin1 thru pin6 switches to the OFF position so they do not connect to the SDA and SCL line.

Set the SW1 and SW5 for the AD[3:0] pins. AD[3:0]=0000 sets device slave address = B0’hex.

Connect SDA, SCL and GND to J17. Please refer to datasheet for register map for EQ, VOD and DEM.

Bill of Materials for PCIE16X_402EVK:

| Item | Qty | Reference | Digikey PN | Manufacture PN | Descriptions |
| --- | --- | --- | --- | --- | --- |
| 1 | 20 | C1,C2,C3,C4,C5,C6,C7,C8, | 445-4711-1-ND | C0603X5R0J104M | CAP CER .10UF 6.3V X5R 0201 |
|  |  | C9,C10,C11,C12,C13,C14, |  |  |  |
|  |  | C15,C16,C17,C18,C19,C20 |  |  |  |
| 2 | 4 | C21,C22,C27,C28 | 445-3448-1-ND | C1608Y5V0J106Z | CAP CER 10UF 6.3V Y5V 0603 |
| 3 | 2 | C23,C24 | 587-1966-1-ND | AMK105BJ475MV-F | CAP CER 4.7UF 4V X5R 0402 |
| 4 | 2 | C25,C26 | 445-5515-1-ND | C0402X5R0J103K | CAP CER 0.01UF 6.3V X5R 01005 |
| 5 | 2 | C29,C30 | 490-1261-1-ND | GRM033R71C102KD01D | CAP CER 1000PF 16V 10% X7R 0201 |
| 6 | 2 | C160,C161 | 445-4998-1-ND | C1005X5R0J105K | CAP CER 1.0UF 6.3V X5R 0402 |
| 7 | 128 | C31,C32,C33,C34,C35,C36, | 587-2483-1-ND | LMK063BJ224MP-F | CAP CER .22UF 10V X5R 20% 0201 |
|  |  | C37,C38,C39,C40,C41,C42, |  |  |  |
|  |  | C43,C44,C45,C46,C47,C48, |  |  |  |
|  |  | C49,C50,C51,C52,C53,C54, |  |  |  |
|  |  | C55,C56,C57,C58,C59,C60, |  |  |  |
|  |  | C61,C62,C63,C64,C65,C66, |  |  |  |
|  |  | C67,C68,C69,C70,C71,C72, |  |  |  |
|  |  | C73,C74,C75,C76,C77,C78, |  |  |  |
|  |  | C79,C80,C81,C82,C83,C84, |  |  |  |
|  |  | C85,C86,C87,C88,C89,C90, |  |  |  |
|  |  | C91,C92,C93,C94,C95,C96, |  |  |  |
|  |  | C97,C98,C99,C100,C101, |  |  |  |
|  |  | C102,C103,C104,C105,C106, |  |  |  |
|  |  | C107,C108,C109,C110,C111, |  |  |  |
|  |  | C112,C113,C114,C115,C116, |  |  |  |
|  |  | C117,C118,C119,C120,C121, |  |  |  |
|  |  | C122,C123,C124,C125,C126, |  |  |  |
|  |  | C127,C128,C129,C130,C131, |  |  |  |
|  |  | C132,C133,C134,C135,C136, |  |  |  |
|  |  | C137,C138,C139,C140,C141, |  |  |  |
|  |  | C142,C143,C144,C145,C146, |  |  |  |
|  |  | C147,C148,C149,C150,C151, |  |  |  |
|  |  | C152,C153,C154,C155,C156, |  |  |  |
|  |  | C157,C158 |  |  |  |
| 8 | 4 | D1,D2,D3,D4 | 511-1592-1-ND | SML-P12PTT86 | LED GREEN 0.2MM 13MCD 0402 SMD |
| 9 |  | J1 | NA | NA | PCIE EDGE FINGERS |
| 10 | 1 | J2 | S2806-ND | NWE82DHRN-T9410 | CONN PCI EXPRESS 164POS VERT PCB |
| 11 | 2 | J3, J5 | WM6502-ND | 22-28-4023 | CONN HEADER 2POS .100 VERT GOLD |
| 12 | 1 | J7 | WM6503-ND | 22-28-4033 | CONN HEADER 3POS .100 VERT GOLD |
| 13 | 3 | J4,J6,J8 | WM6504-ND | 22-28-4043 | CONN HEADER 4POS .100 VERT GOLD |
| 14 | 1 | J9 | 3M5473-ND | 4808-3004-CP | SOCKET IC OPEN FRAME 8POS .3" |
| 15 | 2 | R1,R4 | P1.50KLCT-ND | ERJ-2RKF1501X | RES 1.50K OHM 1/10W 1% 0402 SMD |
| 16 | 7 | R2,R5,R50,R51,R52,R53, | P1.00KLCT-ND | ERJ-2RKF1001X | RES 1.00K OHM 1/10W 1% 0402 SMD |
|  |  | R54 |  |  |  |
| 17 | 2 | R3, R6 | P51.0KLCT-ND | ERJ-2RKF5102X | RES 51.0K OHM 1/10W 1% 0402 SMD |
| 18 | 4 | R7,R8,R9,R10 | P220LCT-ND | ERJ-2RKF2200X | RES 220 OHM 1/10W 1% 0402 SMD |
| 19 | 26 | R11,R13,R14,R16,R17,R19, | P249LCT-ND | ERJ-2RKF2490X | RES 249 OHM 1/10W 1% 0402 SMD |
|  |  | R20,R22,R23,R25,R26,R28, |  |  |  |
|  |  | R29,R31,R32,R34,R35,R37, |  |  |  |
|  |  | R38,R40,R41,R43,R44,R46, |  |  |  |
|  |  | R47,R49 |  |  |  |
| 20 | 13 | R12,R15,R18,R21,R24,R27, | P4.99KLCT-ND | ERJ-2RKF4991X | RES 4.99K OHM 1/10W 1% 0402 SMD |
|  |  | R30,R33,R36,R39,R42,R45, |  |  |  |
|  |  | R48 |  |  |  |
| 21 | 2 | R55,R56 | P2.00KLCT-ND | ERJ-2RKF2001X | RES 2.00K OHM 1/10W 1% 0402 SMD |
| 22 | 6 | SW1,SW3,SW5,SW6,SW10, | CT2196MST-ND | 219-6MST | SWITCH TAPE SEAL 6 POS SMD |
|  |  | SW11 |  |  |  |
| 23 | 1 | SW2 | CT2193MST-ND | 219-3MST | SWITCH TAPE SEAL 3 POS SMD |
| 24 | 3 | SW4,SW7,SW12 | CT2192MST-ND | 219-2MST | SWITCH TAPE SEAL 2 POS SMD |
| 25 | 3 | SW8,SW9,SW14 | CT2194MST-ND | 219-4MST | SWITCH TAPE SEAL 4 POS SMD |
| 26 | 1 | SW13 | P12225SCT-ND | EVQ-21505R | SWITCH LT 6MM 160GF 5MM HEIGHT |
| 27 | 4 | U1,U2,U3,U4 | NA | DS80PCI402SQ | PCIE REPEATER |
| 28 | 2 | U5,U6 | LP3878MR-ADJCT-ND | LP3878MR-ADJ/NOPB | IC VREG 800MA ADJ 8-PSOP |

Document ID: PCIE16X-402EVK User Guide

Date: November, 2011

Rev: 1.2