Problem Statement:

After multiple plug-in and plug-out of RJ-45 cable with PoE, PHY (DP83867ISRGZR) goes faulty due to which auto-negotiation is not able to complete successfully and link doesn’t establish.

It is observed that impedance of Differential Transmit and Receive Signal Pins (Pin No. 7 and 8, 10 and 11) of PHY (DP83867ISRGZR) goes low.

Figure 1 TI PHY (DP83867ISRGZR) Used

Circuit Design Implemented:

Figure 2 TI PHY (DP83867ISRGZR) IC Design

PHY Bootstrap Settings:

Figure 3 Bootstrap 1,2 and 3 Design

Figure 4 Bootstrap 4 and 5 Design

Figure 5 Bootstrap 6,7 and 8 Design

Protection Circuit:

Below is the protection devices detail that we have used in our design:

Table 1 Bootstrap Settings

| RefDes | Manufacturer | MPN |
| --- | --- | --- |
| D6, D7, D36, D37 | littelfuse | LC03-3.3BTG |
| U32 | Pulse | HX6098NL |
| G1-G4 | BOURNS | 2036-09-SM-RPLF |
| RV1- RV4, RV6, RV7 | littelfuse | TMOV14RP115E |
| D17-D19, D27-D31 | VISHAY | V12P10-M3/86A |

Figure 6 Protection Diodes D6,D7 and D36,D37

Figure 7 Transformer U32

Figure 8 Protection MOVs and Schottky Diodes & Rectifiers

Power (48V) Input Design:

Figure 9 Ethernet Connector

Figure 10 2-pin power connector for dc supply

Problem Description:

In our board we have used TI PHY (DP83867ISRGZR) and other protection circuits to power-up using PoE of 48V through RJ-45 connector as in above images.

It is designed to have an option for power-up using both DC supply and PoE with PHY DP83867ISRGZR as in above images.

When we were using dc supply to power the board and using normal ethernet, the board was working properly with 1Gbps link speed.

After we started powering-up with PoE, observed failures over time.

Detail testing exercise done is as below:

- 48V dc power is given at pair C and pair D using PoE adaptor.

- Initially it was working properly with successful auto-negotiation completion and link establishment.

- When multiple (10 times) plug-in and plug-out of RJ-45 was performed, auto-negotiation has not been able to complete successfully and link didn’t establish.

- Then it is observed that there is variation in impedance (measured w.r.t. GND) at PHY Differential Transmit and Receive Signal Pins (Pin No. 7 and 8, 10 and 11). They are getting low as in below table.

- We have observed this issue on 4-5 chipsets.

Table 2 Impedance Variation on different chipsets

| Chip- Sets | PHY Pin | Affect on Impedance (measured w.r.t. GND (TPAD)) |
| --- | --- | --- |
| 1 | Pair C : Pin 7 and 8 Pair D : Pin 10 and 11 | Pair C: 1.6K instead of 2K Pair D: 500E instead of 2K |
| 2 |  | Pair C: 2K Pair D: 5E instead of 2K |
| 3 |  | Pair C: 161E instead of 2K Pair D: 1.3K instead of 2K |
| 4 |  | Pair C: 1.9K instead of 2K Pair D: 210E instead of 2K |