Display Bring-Up Report

Quectel SC668S  +  Riverdi RVT101HVDNWC00-B

| Item | Detail |
| --- | --- |
| Quectel ticket | 235345 |
| Host module | Quectel SC668S (Qualcomm QCM6125), Android |
| Display module | Riverdi RVT101HVDNWC00-B, 10.1" 1280 x 800 IPS, optically bonded, PCAP touch |
| DSI-side device | Texas Instruments SN65DSI83ZXHR — MIPI DSI to LVDS bridge |
| Internal TFT | Riverdi RVT101HVLNWC00-B (LVDS) |
| Touch controller | ILI2132A (I2C) |
| Current status | Backlight functional. No image. Bridge never initialised. |

# 1.  Purpose of this document

Quectel has suggested that we obtain "the init code of the panel" from the display supplier. This document explains why that request does not apply to this particular display, what the display actually requires instead, and exactly what needs to be implemented in the SC668S bootloader.

All register values, timings and sequences needed to write the bootloader patch are contained in this document. No further information from the display supplier is required.

# 2.  Summary

The RVT101HVDNWC00-B is not a raw MIPI-DSI panel, and no panel init code exists for it. The display module carries a Texas Instruments SN65DSI83 MIPI-DSI-to-LVDS bridge IC on its own PCB. The MIPI DSI signals from the SC668S terminate at this bridge, which converts them to LVDS and drives the internal TFT. The TFT itself is never addressed directly by the host.

The practical consequences are:

- There is no DCS initialisation sequence. No 0x11 (Sleep Out), no 0x29 (Display On), no vendor register writes over DSI.

- qcom,mdss-dsi-on-command and qcom,mdss-dsi-off-command must be empty.

- The bridge is configured entirely over I2C, not over the DSI link.

- The bridge PLL uses the incoming DSI clock as its reference, so the DSI clock must already be running before the bridge PLL is enabled. This is the opposite of the standard Qualcomm bootloader flow.

- Because this requires I2C register writes plus a reordered clock sequence, it cannot be expressed as a DTSI command array and requires C code in the bootloader display driver.

Why only the backlight is currently lit:  the backlight is driven by an on-board TPS61500 boost converter directly from the PWM pin, entirely independent of the DSI and LVDS paths. A lit backlight with no image is exactly the expected symptom of a bridge that has never been enabled or configured.

# 3.  Action required in the bootloader

The following six items must be implemented in XBL/ABL. Each is listed with the reason it is necessary, so that the intent is clear even where the SC668S bootloader structure differs from the reference design.

| # | Action | Reason it is required |
| --- | --- | --- |
| 1 | Power rails and GPIOs: enable the 5.0 V module supply, drive VREF, hold PWR_DN low initially, enable backlight PWM. | Nothing on the module can be probed until it is powered. VREF sets the logic level for I2C, PWM and PWR_DN — if it floats, I2C will not respond. This stage is already working on our board. |
| 2 | Enable an I2C (QUP) master in the bootloader on the bus wired to the display connector. | The bridge is configured only over I2C. Bootloaders commonly ship with most QUP instances disabled because nothing needs them that early. If no I2C master is available in XBL/ABL, this is the first blocker and nothing else can proceed. |
| 3 | Bring up the DSI host and start the clock lane in continuous HS mode BEFORE enabling the bridge PLL. | The SN65DSI83 has no crystal or external reference. It derives its PLL reference from the incoming DSI clock. If the clock is absent or non-continuous when the PLL is enabled, PLL_LOCK never asserts and the LVDS output stays dark. The stock Qualcomm flow sends panel init before starting the clock, which cannot work here. This ordering change is the single most important item. |
| 4 | Write the SN65DSI83 CSR registers over I2C, enable the PLL, poll for lock, then issue a soft reset. | This is the actual "init code" for this display. It tells the bridge the DSI clock rate, lane count, pixel format, and the LVDS timing it must generate. Until these are written, the bridge produces no LVDS output at all. |
| 5 | Start the DSI video stream only after PLL lock and soft reset are complete. | The bridge must be locked and configured before pixel data arrives, otherwise it reports synchronisation errors and does not display a stable image. |
| 6 | Read and clear error register 0xE5, then re-read to confirm a clean link. | Provides positive confirmation that the link is healthy, and identifies precisely which stage failed if it is not — far faster than diagnosing a black screen. |

Note on scope: this work is required in the bootloader (not only the kernel) because the boot logo, fastboot UI, recovery screen and charging animation are all rendered by XBL/ABL before the kernel starts. It is also where display state is handed off via continuous splash, so an incorrect bootloader configuration can break the kernel display path as well.

# 4.  Display connector interface

Riverdi J1 connector, 34-pin ZIF, 0.5 mm pitch, down-side contact. Signals relevant to bring-up:

| Pin | Signal | Direction | Notes |
| --- | --- | --- | --- |
| 2 / 3 | DSI_D0P / D0N | I/O | MIPI DSI data lane 0 |
| 5 / 6 | DSI_D1P / D1N | In | MIPI DSI data lane 1 |
| 8 / 9 | DSI_CLKP / CLKN | In | MIPI DSI clock pair — must be continuous HS |
| 11 / 12 | DSI_D2P / D2N | In | MIPI DSI data lane 2 |
| 14 / 15 | DSI_D3P / D3N | In | MIPI DSI data lane 3 |
| 17 | PWR_DN | In | Bridge enable. Active low = powered down. Drive HIGH to enable. 15 k pull-up to VREF |
| 18 | PWM | In | Backlight brightness. 200 Hz to 1 kHz. Logic levels independent of VREF |
| 19 | INT | Out | Touch panel interrupt. Open drain, active low |
| 20 / 21 | I2C_SCL / I2C_SDA | I/O | Shared bus: SN65DSI83 bridge AND ILI2132A touch controller |
| 22 | RESET | In | Touch panel reset only, active low. Does not reset the bridge |
| 24 | VREF | Power | Reference voltage, 1.8 V or 3.3 V. Must be driven — sets I/O logic level |
| 26 - 29 | 5.0 V | Power | Module supply |
| 1, 4, 7, 10, 13, 16, 30 | GND | Power | Ground |

Bridge I2C address: 7-bit 0x2C or 0x2D, selected by the ADDR strap on the Riverdi PCB. The bus must be scanned to confirm which. The ILI2132A touch controller is a separate device on the same bus and must not be confused with the bridge.

# 5.  Panel timing derivation

Taken from the Riverdi RVT101HVDNWC00-B datasheet Rev 1.0 (2023-11-15), section "TFT Timing Characteristics".

| Parameter | Symbol | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| LVDS pixel clock | FDCLK | 66.3 | 72.4 | 78.9 | MHz |
| Horizontal total | TH | 1380 | 1440 | 1500 | DCLK |
| Horizontal active | THD | — | 1280 | — | DCLK |
| Horizontal back porch (incl. pulse) | THBP | 88 | 88 | 88 | DCLK |
| Horizontal front porch | THFP | 12 | 72 | 132 | DCLK |
| Horizontal sync pulse width | THPW | 1 | — | 40 | DCLK |
| Vertical total | TV | 824 | 838 | 872 | H |
| Vertical active | TVD | — | 800 | — | H |
| Vertical back porch (incl. pulse) | TVBP | 23 | 23 | 23 | H |
| Vertical front porch | TVFP | 1 | 15 | 49 | H |
| Vertical sync pulse width | TVW | 1 | — | 20 | H |

Consistency check: 1440 x 838 x 60 Hz = 72.4 MHz, which matches the typical pixel clock. The timing set is self-consistent.

## 5.1  Sync and porch split

The datasheet specifies the back porch including the sync pulse width, so the pulse width must be subtracted to obtain the values the bridge expects:

- HSA = 20, HBP = 88 − 20 = 68, HFP = 72

- VSA = 3, VBP = 23 − 3 = 20, VFP = 15

The sync pulse widths are free choices within the datasheet limits; the values above sit comfortably inside the permitted ranges and keep the totals exact.

## 5.2  Derived DSI parameters

For 4 data lanes and RGB888 (24 bits per pixel):

DSI lane rate = 72.4 MHz x 24 bpp / 4 lanes  = 434.4 Mbps per lane
DSI clock     = 434.4 Mbps / 2 (DDR)         = 217.2 MHz

# 6.  DSI host configuration (Qualcomm side)

| Setting | Value | Reason |
| --- | --- | --- |
| Data lanes | 4 | All four lanes are wired to the bridge |
| Pixel format | RGB888 (24 bpp) | Bridge input format |
| Mode | Video mode, burst | The bridge regenerates its own LVDS timing from its CSRs and does not need DSI blanking packets |
| Clock lane | Continuous HS — mandatory | Bridge PLL reference. Non-continuous clock will prevent PLL lock |
| EoT packet | Disabled | Bridge does not expect an End-of-Transmission packet; appending one produces low-level protocol errors |
| HSA / HBP / HFP blanking packets | Not required | Timing is generated by the bridge, not carried over DSI |
| Panel timings | 1280 x 800 as per section 5 | Must still be programmed on the host so the pixel clock is correct |

Relevant DTSI properties:

qcom,mdss-dsi-panel-width          = <1280>;
qcom,mdss-dsi-panel-height         = <800>;
qcom,mdss-dsi-h-front-porch        = <72>;
qcom,mdss-dsi-h-back-porch         = <68>;
qcom,mdss-dsi-h-pulse-width        = <20>;
qcom,mdss-dsi-v-front-porch        = <15>;
qcom,mdss-dsi-v-back-porch         = <20>;
qcom,mdss-dsi-v-pulse-width        = <3>;
qcom,mdss-dsi-bpp                  = <24>;
qcom,mdss-dsi-lane-map             = "lane_map_0123";
qcom,mdss-dsi-traffic-mode         = "burst_mode";
qcom,mdss-dsi-panel-type           = "dsi_video_mode";

      
/* continuous clock - mandatory for the bridge PLL reference */
qcom,mdss-dsi-force-clock-lane-hs;

      
/* qcom,mdss-dsi-tx-eot-append deliberately omitted - EoT must NOT be sent */

      
/* no DCS init - the bridge is configured over I2C */
qcom,mdss-dsi-on-command  = [];
qcom,mdss-dsi-off-command = [];

# 7.  Bootloader power-up and initialisation sequence

Critical:  this ordering is mandatory and the default Qualcomm bootloader flow violates it. XBL/ABL normally sends panel initialisation before starting the DSI clock. The SN65DSI83 requires the clock to be present first, because it is the PLL reference. Steps 2 and 3 must not be reversed.

- Assert PWR_DN low. Hold for at least 10 ms. This places the bridge in a known reset state.

- Bring up the DSI host and start the clock lane in continuous HS mode. The clock must be stable before the bridge is enabled.

- Release PWR_DN high. Wait at least 10 ms for the bridge internal reset to complete and the I2C interface to become available.

- Write 0x09 = 0x00 (clear soft reset) and 0x0D = 0x00 (PLL disabled) to guarantee a clean starting state.

- Write all CSR registers as listed in section 8.

- Write 0x0D = 0x01 to enable the PLL. Wait at least 3 ms.

- Poll 0x0A bit 7 (PLL_EN_STAT) until it is set, with a timeout of approximately 100 ms. If this bit never sets, the DSI clock is absent or the clock-range registers are wrong — stop and investigate before proceeding.

- Write 0x09 = 0x01 (soft reset) to latch the CSR configuration. Wait at least 10 ms.

- Start the DSI video stream.

- Read 0xE5, write the value back to clear it, wait 1 ms, then read 0xE5 again. It should read 0x00.

- Enable the backlight PWM last, so that no partially initialised frame is visible.

## 7.1  Shutdown sequence

Disable interrupts (0xE1 = 0x00, then 0xE0 = 0x00), stop the video stream, drive PWR_DN low, and hold it low for at least 10 ms before removing power.

# 8.  SN65DSI83 register values

All values below are computed for this specific panel: 1280 x 800 at 60 Hz, 72.4 MHz LVDS pixel clock, 4 DSI lanes, RGB888, single-link LVDS. Register semantics follow the TI SN65DSI83 datasheet and the mainline Linux driver drivers/gpu/drm/bridge/ti-sn65dsi83.c, from which the derivation formulas are taken.

| Reg | Name | Value | Derivation / purpose |
| --- | --- | --- | --- |
| 0x09 | RC_RESET | 0x00 | Cleared during configuration. Written as 0x01 at step 8 to latch the CSRs |
| 0x0A | RC_LVDS_PLL | 0x05 | LVDS_CLK_RANGE = (72400 − 12500) / 25000 = 2, shifted left 1 = 0x04; plus HS_CLK_SRC_DPHY = bit 0 |
| 0x0B | RC_DSI_CLK | 0x10 | DSI_CLK_DIVIDER = (24 bpp / 4 lanes / 2) − 1 = 2, shifted left 3 |
| 0x0D | RC_PLL_EN | 0x00 then 0x01 | Disabled during configuration, enabled at step 6 |
| 0x10 | DSI_LANE | 0x26 | CHANNEL_MODE_SINGLE = bit 5; CHA_DSI_LANES = ~(4−1) & 0x3 = 0, shifted left 3; CHB_DSI_LANES = 3, shifted left 1 (unused on DSI83, set to default) |
| 0x11 | DSI_EQ | 0x00 | No equalisation required at this lane rate |
| 0x12 | DSI_CLK | 0x2B | CHA_DSI_CLK_RANGE = 217200 kHz / 5000 = 43 |
| 0x18 | LVDS_FMT | 0x18 | LVDS_LINK_CFG = bit 4 (single link); CHA_24BPP_MODE = bit 3; SPWG/VESA mapping (FORMAT1 clear). See section 8.1 |
| 0x19 | LVDS_VCOM | 0x05 | LVDS output voltage swing setting 1 for both channels |
| 0x1A | LVDS_LANE | 0x03 | 200 ohm near-end termination, both channels |
| 0x1B | LVDS_CM | 0x00 | Default common-mode adjustment |
| 0x20 / 0x21 | CHA_ACTIVE_LINE_LENGTH | 0x00 / 0x05 | 1280, little-endian |
| 0x24 / 0x25 | CHA_VERTICAL_DISPLAY_SIZE | 0x20 / 0x03 | 800, little-endian |
| 0x28 / 0x29 | CHA_SYNC_DELAY | 0x21 / 0x00 | 33 (= 32 + 1) pixel clocks, required by the datasheet for correct operation |
| 0x2C / 0x2D | CHA_HSYNC_PULSE_WIDTH | 0x14 / 0x00 | HSA = 20 |
| 0x30 / 0x31 | CHA_VSYNC_PULSE_WIDTH | 0x03 / 0x00 | VSA = 3 |
| 0x34 | CHA_HORIZONTAL_BACK_PORCH | 0x44 | HBP = 68 |
| 0x36 | CHA_VERTICAL_BACK_PORCH | 0x14 | VBP = 20 |
| 0x38 | CHA_HORIZONTAL_FRONT_PORCH | 0x48 | HFP = 72 |
| 0x3A | CHA_VERTICAL_FRONT_PORCH | 0x0F | VFP = 15 |
| 0x3C | CHA_TEST_PATTERN | 0x00 | Set to 0x10 to enable the internal colour-bar test pattern (see section 9) |
| 0xE0 | IRQ_GLOBAL | 0x01 | Optional — global interrupt enable |
| 0xE1 | IRQ_EN | 0xFE | Optional — all error interrupts except PLL_UNLOCK |
| 0xE5 | IRQ_STAT | read / write back | Cleared after init, then verified as 0x00 |

## 8.1  Two values to be confirmed

Register 0x18 depends on the LVDS mapping of the internal TFT, which the RVT101HVDNWC00-B datasheet does not state. These affect image quality, not whether an image appears at all, so they should not block bring-up:

- LVDS format: SPWG/VESA (0x18) versus JEIDA/Format 1 (0x1A). An incorrect choice produces wrong colours, not a blank screen.

- Bit depth: 24 bpp is assumed. If the internal TFT is 18 bpp, clear bit 3.

- Sync polarity: bit 7 (DE), bit 6 (HSYNC) and bit 5 (VSYNC) select negative polarity. Most LVDS panels are DE-driven and insensitive to this.

Recommendation: start with 0x18 and adjust only if the image appears with incorrect colours.

# 9.  Recommended debug sequence

These three steps isolate the fault quickly and can largely be run without a working display pipeline.

## Step 1 — Confirm I2C communication

With PWR_DN held high, scan the I2C bus. The bridge must acknowledge at 0x2C or 0x2D. Read ID registers 0x00 to 0x08; they return the ASCII device identifier. If there is no acknowledgement, stop here — check VREF and PWR_DN before writing any display code.

## Step 2 — Internal test pattern

Run the full sequence from section 7 with 0x3C = 0x10 and no video stream at all. The bridge generates its own colour bars internally.

- Bars appear: the DSI clock, PLL lock, bridge configuration and LVDS output are all correct. Any remaining problem is confined to the video stream from the SoC.

- Nothing appears: the fault is in the clock, PLL or I2C path. Check PLL_EN_STAT (0x0A bit 7) and confirm the DSI clock is actually present at the connector.

This single test splits the problem in half and is the fastest way to establish whether the bridge configuration is correct.

## Step 3 — Interpret the error register

After enabling the video stream, a non-zero value in 0xE5 identifies the failure:

| Bit | Error flag | Typical cause |
| --- | --- | --- |
| 7 | CHA_SYNCH_ERR | Timing mismatch between the DSI stream and the CSR timing values |
| 6 | CHA_CRC_ERR | Signal integrity on the DSI data lanes |
| 5 | CHA_UNC_ECC_ERR | Uncorrectable ECC — signal integrity or incorrect lane mapping |
| 4 | CHA_COR_ECC_ERR | Correctable ECC — marginal signal integrity |
| 3 | CHA_LLP_ERR | Low-level protocol error — frequently caused by an EoT packet being appended, or an LP-mode entry issue |
| 2 | CHA_SOT_BIT_ERR | Start-of-transmission error — lane skew or insufficient drive strength |
| 0 | CHA_PLL_UNLOCK | DSI clock absent, non-continuous, or clock-range registers (0x0A, 0x0B, 0x12) incorrect |

# 10.  Information requested from Quectel

- Confirmation that an I2C (QUP) master is available in XBL/ABL on the bus routed to the display connector. If none is enabled in the bootloader, this is the first blocker and must be addressed before anything else.

- Confirmation of the GPIO number and control method for PWR_DN on the SC668S side, so it can be driven by the bootloader display driver.

- A bootloader build incorporating the sequence in section 7 and the register values in section 8, so that the test pattern check in section 9 can be performed on our hardware.

- Confirmation that the equivalent bridge initialisation is present in the kernel display path. The mainline Linux driver drivers/gpu/drm/bridge/ti-sn65dsi83.c can be used as a reference or ported directly.

Our schematic is provided separately with this ticket.

# 11.  References

- Riverdi RVT101HVDNWC00-B datasheet, Rev 1.0, 2023-11-15 — download.riverdi.com/RVT101HVDNWC00-B/DS_RVT101HVDNWC00-B_Rev.1.0.pdf

- Texas Instruments SN65DSI83 datasheet — ti.com/lit/ds/symlink/sn65dsi83.pdf

- Linux mainline driver — github.com/torvalds/linux/blob/master/drivers/gpu/drm/bridge/ti-sn65dsi83.c

- Quectel SC668S Series Smart Module Specification V1.3