Refer to snla356-DS90UB941AS-Q1 DSI Bringup Guide-->4.3 Incorrect DSI Packet Timing, use mipi analyzer to analyze and adjust imx8 dsi stream, but the backend dlpc230 still reports an error and cannot display: "Open LDI total pixels per line are unstable "

- Attached are the dsi source events we captured through the mipi analyzer. The hfp, hbp, hsync timings are very close to the theoretical values. Please help analyze whether the front-end dsi source still has problems, why the back-end dlpc230 still reports an error of Open LDI total pixels per line are unstable.

- Please help to provide a dsi source events with a front-end resolution of 1152*576, 60Hz, which can be displayed normally on the dlpc230 for our reference.

Mipi analyzer screenshot 1--video_analysis

Mipi analyzer screen shot 2--events

Block diagram：

Fig 3. Block diagram

I.MX8 SoC timing configuration ：

clock = 45000,

hdisplay = 1152,

hfront_porch = 40,

hsync_len = 8,

hback_porch = 50, （but on last line HBP has been tested having 35 more clocks）

vdisplay = 576,

vfront_porch = 1,

vsync_len =  8,

vback_porch = 15,

vrefresh = 60,

dsi source timing test result：

Fig 4--dsi source test data, the 4th column means actual test result and the 5th column means value test after adjusting SoC configuration

Using ds90ub941 patgen test result:

- ds90ub941 internal timing(0x65: 0x0C)，dsi clock，display normally;

- ds90ub941 external timing(0x65: 0x08)，dsi clock，display abnormally;

Display resolution requirement:

Could you please tell us how to configure the correct timing with I.mx 8 to 941AS then 941AS could output a qualified timing to DLPC230?