MC33078P Burn-in result:

A total of 840 pcs of IC in 210 boards (each board contains 4 ICs) are burned in and the result is as follows:

| Day | No of ICs failed | Cumulative No. of ICs failed |
| --- | --- | --- |
| 1 | 4 | 4 |
| 2 | 2 | 6 |
| 3 | 2 | 8 |
| 4 | 0 | 8 |
| 5 | 2 | 10 |

Failure symptom: Supplies to +15V and -15V are fused.

Failure analysis: Pin 8 (VCC+) and Pin 4 (VCC-) of the IC are shorted as compared to other good IC.

Illustrative burn-in circuit: