# Texas Instruments

# HPA665 Test Procedure

# Rev. A

1.0 GENERAL

1.1 PURPOSE

1.1.1 To provide detailed instructions for testing HPA665 Modules USB2ANY.

1.2 SCOPE

1.2.1 Covers complete instructions for testing HPA665-001 and -002.

1.3 REFERENCE DOCUMENTATION

1.3.1 Schematic: HPA665_SCH.PDF

1.3.2 Assembly DWGs: HPA665_PCB.PDF

1.3.3 Users Guide N/A

1.4 MATERIALS\Software

1.4.1 USB cable A-B (mini).

1.4.2 PR2046  ( USB2ANY test jig/breakout board with 3 shorting blocks installed (TI

Supplied)

1.4.3 USB2ANY_PC_Software.ZIP Programming and test software. (TI supplied)

1.5 DEFINITIONS

1.5.1 Use if definitions of terms are required

2.0 SAFETY

2.1 Safety Glasses are to be worn.

2.2 This test must be performed by qualified personnel trained in electronics theory and

understand the risks and hazards of the assembly to be tested.

2.3 ESD precautions must be followed while handling electronic assemblies while

performing this test.

2.4 Precautions should be observed to avoid touching areas of the assembly that may get

hot or present a shock hazard during testing.

3.0 QUALITY

3.1 Test data shall be made available upon request by Texas Instruments.

4.0 APPAREL

4.1 Electrostatic smock

4.2 Electrostatic Gloves or finger cots

4.3 Safety Glasses

4.4 Ground ESD wrist strap.

5.0 EQUIPMENT

5.1 Computer with Microsoft XP SP2 (minimum) operating system with .NET 2.0 framework and available USB port.

5.2

6.0 EQUIPMENT SETUP

6.1  Specify how equipment is to be setup and power on sequence if needed.

6.2             Software Installation   (If installed skip this step)

- The programming and test software will be provided in zip file USB2ANY_PC_Software.zip.  These must be run on a PC with Windows XP, SP2 at a minimum, which should already have the .NET 2.0 framework installed. With this, no program installer is needed.

- Simply copy the folders to the hard drive of the test computer.

- The USB2ANY Firmware folder contains the FirmwareUpgradeUtility.exe and the necessary firmware file, which ends with a .txt extension.

The PC_Software folder contains the USB2ANY_GUI.exe and support files.

7.0 PROCEDURE

7.1 Programming Procedure

- 7.1.1 Install programming and test software supplied. (See Section 6.2)

7.1.2 This step programs the firmware into the USB2ANY. Programming may be done in a

batch mode, so all boards may be programmed first, then tested.

      - Connect the USB cable to the testing PC. Do NOT have the USB2ANY connected to the USB cable at this time.

      - Press and hold S1, located as shown in Figure 1.

S1

Figure 1. Location of S1

      - While pressing S1, connect the USB cable from the computer to the USB2ANY. The LED on the USB2ANY should slowly light up until it is a solid green color. Release S1.

      - On the computer, start up the FirmwareUpgradeUtility.exe. The initial screen will appear as shown in Figure 2. Press Next>

Figure 2. Firmware Upgrade Utility opening screen.

      - The screen in Figure 3 will appear. Select  “I accept this license agreement” and press Next>.

Figure 3. License agreement screen of Firmware Upgrade Utility.

      - The screen of Figure 4 will then be presented. Verify that it shows “Found 1 device”. If it does not, unplug the USB cable, press and hold S1, and replug.

Verify that a

device is found

Figure 4. Screen showing device found.

      - Press the Browse button. A standard file dialog box will open. Locate and select the USB2ANY_version_x_x_x_x.txt file, where the x’s will indicate the firmware version. The location of the file will be shown in the box next to the Found 1 device label, as shown in Figure 5.

Figure 5. Firmware located and loaded after pressing Browse button.

      - Press the Upgrade button. The programming steps will be indicated in the box with a scrollbar, and a progressbar will show on the screen. When programming is complete, the LED on the USB2ANY will blink several times and then glow a solid green. The screen on the computer will look like that in Figure 6.

Figure 6. Programming complete.

      - Disconnect the programmed board from the USB cable. To program additional boards, repeat steps 2 and 3; when a new board is connected (while pressing S1), the Found 1 device message should show, and then step 8 can be performed.

      - When all boards are programmed, press the Close button.

7.2 Test Procedure

1. Verify that the test jig has the three shorting bars installed, as shown in Figure 7. Verify that all switches on the test jig’s S1 and S2 switches are in the ON position.

Jumpers installed

at three positions

- Figure 7. Test Jig with Shorting Bars Properly Installed

- 2. Attach the USB2ANY Test Jig/Breakout Board to the USB2ANY, as shown in Figure 8.

Figure 8. Test jig attached to USB2ANY.

- Start up the USB2ANY_GUI.exe. The initial screen presented is shown in Figure 9. (Your screen may look somewhat different, as the GUI is being updated frequently). Note the tabs across the top of the screen, and the status bar at the bottom of the screen. Assure that the status bar shows USB2ANY: Detected. If it shows USB2ANY: NOT Detected, unplug the USB cable from the USB2ANY and replug it until the status bar indicates that the USB2ANY has been detected.

Figure 9. USB2ANY_GUI.exe opening screen

- Select the MFG Test Window tab. The screen will appear as shown in Figure 10. Again, please ensure that the Connection status reads USB2ANY Detected.

Figure 10. Test screen ready to test USB2ANY.

- When ready to begin the test, press the Start Test Procedure button. The program will do some initial tests on the USB2ANY, and show messages in the Test Progress box. The first group of tests will show pass or fail in this message box. The next group of tests will then enable the top row of Yes/No buttons, as shown in Figure 11. This question applies to the state of the LEDs on the test jig, which should have D1, 9, 4, 6, 8, 13, 15, 10 lit up, as seen in Figure 12 (the two near the edge of the board and the row of LEDs closest to the USB2ANY). If they are all lit, press Yes. If any are not lit, press No.

Figure 11. Test screen having tested group 1, preparing for test group 2.

Figure 12. D1, 9, 4, 6, 8, 13, 15, 10 LEDs lit.

- The next group of tests will enable the second row of Yes/No buttons (Figure 13). The test jig should have D1, 9, 3, 5, 7, 14, 12, 11 illuminated, as shown in Figure 14 (the two near the edge of the board and the row farthest from USB2ANY). Press Yes or No, as appropriate.

Figure 13. Preparing to test group 3.

Figure 14. D1, 9, 3, 5, 7, 14, 12, 11 illuminated.

- The last group of tests will turn all the LEDs off, and enable the Are All LEDs OFF? buttons (Figure 15). All LEDs on the test jig should be turned off, as shown in Figure 16. Note that due to some leakage paths, it may be that the +3.3V LED will not be completely OFF (glow very faintly), but this should be considered an OFF condition.

Figure 15. Final test group screen.

Figure 16. All LEDs OFF.

The tests are now complete. If all tests passed, the screen will appear as shown in Figure 17. The EVM board may then be readied for shipment per section 9.0.

Figure 17. Board passed.

- If any step fails, the screen will appear as in Figure 18. If the test fails, set aside the board and make note of which group failed from the messages in the Test Progress box. Refer to the troubleshooting section to determine if the failure can be corrected.

Figure 18. Board failed.

- Disconnect the USB2ANY from the test jig. Repeat steps 1-9 for additional boards to be tested.

8.0 EQUIPMENT SHUTDOWN

8.1 Specify power down sequence as needed.

9.0 MATERIAL DISPOSITION & TRANSFER

## 9.1 CONFORMING MATERIAL

Units that have passed this test procedure shall be packaged into anti-static ESD approved bags, labeled with two labels according to the table below, and shipped per the P.O.

| Label 1 Assembly Number+Dash Number if Applicable | Label 2 IC Number |
| --- | --- |
| HPA665-001 | Not required, no label 2. |
| HPA665-002 | Not required, no label 2. |

## 9.2 NON-CONFORMING MATERIAL

If yield loss is 2% or less, scrap non-conforming units and adjust P.O. to reflect total amount shipped. If yield loss approaches or exceeds 5%, contact EVM coordinator for assistance.