INA226 abnormal measurement value

INA226 setting

- Configuration register  :  Default

- Calibration register  :  2560 (0xA00)

- AlertLimit register  :  0x7FFF

- Mask/Enable register  :  Shunt Voltage Over Limit(SOL)

Measurement

- Bus Voltage

- Current

- 50ms measurement cycle

Issue

- In case of measuring : Bus Voltage  Current

  - At charging

    - About 200mA larger value was seen.

    - INA226 measurement value didn’t change even when actual charging current changed.

  - At discharging

    - INA226 showed “+16000”, while the discharge current direction must be opposite.

    - Sometimes around “-200mA” can be seen.

- Shunt Voltage was measured.

  - When capturing Shunt VoltageBus VoltageCurrent,

    - Bus Voltage was abnormal,

    - Current value was correct.

  - When capturing Bus VoltageShunt VoltageCurrent,

    - Bus Voltage was correct,

    - Current value was abnormal.

Inquiry

- What should the customer do ?

  - Are there any sequence and/or term for correct value reading for INA226 ?

| Functions | Register Pointer | Access | Data |  | Timing |
| --- | --- | --- | --- | --- | --- |
| Bus Voltage | 02h | R Word | ADC result | Voltage Full-scale range = 40.96 V (decimal = 7FFF); LSB = 1.25 mV | Each cycle |
| Current | 04h | R Word | ADC result | Current | Each cycle |
| Calibration | 05h | R/W Word | Coefficient | Current =ShuntVoltage× CalibrationRegister / 2048 | Set at initialization |
| Mask/Enable | 06h | R/W Word | AlertPIN | Shunt Voltage Over-Limit (SOL) | Set at initialization |
| Alert Limit | 07h | R/W Word | Alert threshold |  | Set at initialization |

Battery

INA226

System