//******************************************************

// LMP91000EVM COde for Cyclic Voltametry

// The values of registers are based on the paper

// https://ieeexplore.ieee.org/stamp/stamp.jsp?tp=&arnumber=8095460

// !2-06-2018

//******************************************************

#include <Wire.h>

#include "LMP91000.h"

LMP91000 lmp91000;

void setup(void){

Serial.begin(9600);

Serial.println("LMP91000EVM ");

Wire.begin();

// initialize the slot select pins to "not selected"

pinMode(7, OUTPUT);  digitalWrite(7, LOW);

pinMode(9, OUTPUT);  digitalWrite(9, LOW);

pinMode(10, OUTPUT); digitalWrite(10, LOW);

digitalWrite(9, HIGH); // select CO

// settings for voltametry

uint8_t res = lmp91000.configure(

LMP91000_TIA_GAIN_35K | LMP91000_RLOAD_10OHM,

LMP91000_REF_SOURCE_INT | LMP91000_INT_Z_50PCT

| LMP91000_BIAS_SIGN_POS | LMP91000_BIAS_24PCT,

LMP91000_FET_SHORT_DISABLED | LMP91000_OP_MODE_AMPEROMETRIC

);

Serial.print("Config Result: ");

Serial.println(res);

Serial.print("STATUS: ");

Serial.println(lmp91000.read(LMP91000_STATUS_REG),HEX);

Serial.print("TIACN: ");

Serial.println(lmp91000.read(LMP91000_TIACN_REG),HEX);

Serial.print("REFCN: ");

Serial.println(lmp91000.read(LMP91000_REFCN_REG),HEX);

Serial.print("MODECN: ");

Serial.println(lmp91000.read(LMP91000_MODECN_REG),HEX);

}

void loop(void){

delay(100);

Serial.println("Vout = ");

Serial.println(A0);

Serial.println("Vref = ");

Serial.println(A1);

}