# Texas Instruments

AMPS084

# OPA3S328EVM Test Procedure

# Rev. B

9/21/2020

- GENERAL

  - PURPOSE

    - To provide detailed instructions for testing EVM Modules

  - SCOPE

    - Covers complete instructions for testing ALM2403Q1EVM

  - REFERENCE DOCUMENTATION

    - Schematic OPA3S328EVM

    - Assembly DWG OPA3S328EVM

    - Users Guide OPA3S328EVM (SBOU--- Pending)

Note: Please do not put references to revision letters of documents in the test procedure. This will only cause extra work when docs change.

  - MATERIALS

    - Test Log attached to end of this procedure.

  - DEFINITIONS

Add definitions of terms as required

    - DMM is reference to Digital Multimeters

    - UUT is reference to Unit under Test

    - EVM is reference to Evaluation Module assembly, in this case the UUT

    - “Hi Voltage” is defined as DC voltages > 75V and AC voltages > 50 volts (RMS)

- SAFETY

  - Safety Glasses are to be worn.

  - This test must be performed by qualified personnel trained in electronics theory and understand the risks and hazards of the assembly to be tested.

  - ESD precautions must be followed while handling electronic assemblies while performing this test.

  - Precautions should be observed to avoid touching areas of the assembly that may get hot or present a shock hazard during testing.

- QUALITY

  - Test data or reports shall be made available upon request by Texas Instruments.

- APPAREL

  - Electrostatic smock

  - Electrostatic Gloves or finger cots

  - Safety Glasses

  - Ground ESD wrist strap

- EQUIPMENT

  - Power Supply capable of +5VDC (HP3631 or equivalent)

  - Hand Held Digital voltmeter.

  - 6-1/2 Digit Multimeter (Keysight 34460A or Equivalent)

  - Clipleads/Minigrabbers as indicated.

- EQUIPMENT SETUP

  - Install shunts on jumpers J11pins 2-3, J35 pins 1-2, J16 pins 1-2 in the default positions as shown Figure 1.

Figure 1.  Default Jumper Positions

  - Adjust the power supply to +5VDC. Set current limit to 30mA. Disable supply outputs.

  - Connect +5V supplies to V+ (Banana Plug J1), and GND (Banana Plug J2).

  - Install shunt on Pin 3-2 on J11 Jumper, (V- Supply Select to GND).

- PROCEDURE

  - Enable power supply output. Verify 5V Current supply Consumption is between 13mA to 20mA

  - Set Voltmeter to read DCV and connect to MUX OUT (TP6) and Analog GND (TP17) test points.

  - Voltmeter should read ~2.5VDC(± 0.020V) within the accuracy of the Voltmeter and power supply

  - Set Voltmeter to read DCV and connect to OUTA (TP19) and Analog GND (TP10) test points.

  - Voltmeter should read ~2.5VDC(± 0.020V) within the accuracy of the Voltmeter and power supply

  - Disable power supply.

  - Install shunt on J6 pin 2-3 and Install shunt on J23 2-3.  See Figure 2.

Figure 2.   Jumper J6 shunt 2-3, Jumper J23 shunt 2-3

  - Enable power supply output.

  - Set Voltmeter to read DCV and connect to MUX OUT (TP6) and Analog GND (TP17) test points.

  - Voltmeter should read ~0V (± 0.050V) within the accuracy of the Voltmeter and power supply.

  - Set Voltmeter to read DCV and connect to OUTA (TP19) and Analog GND (TP10) test points.

  - Voltmeter should read ~0V (± 0.050V) within the accuracy of the Voltmeter and power supply.

  - Disable power supply outputs. Remove all power supply and clip lead connections.

  - Remove shunt J6 and shunt J23. Leave Jumpers on J11pins 2-3, J35 pins 1-2, J16 pins 1-2 in the default positions as shown Figure 1.

  - Install shunt on J13 (A0), J15 (B0) and J30 (A0), J32 (B0).  Ensure all jumpers are installed as shown on Figure 3.

Figure 3.   Jumper J6 shunt 2-3, Jumper J23 shunt 2-3

  - Test complete.

  - Repeat testing on next pcb starting at 6.1 until all boards are done.

- EQUIPMENT SHUTDOWN

  - Specify power down sequence as needed if applicable.

- MATERIAL DISPOSITION & TRANSFER

  - CONFORMING MATERIAL

On the ESD bag, in text clearly visible and readable to the naked eye, add

For evaluation only; not FCC approved for resale.

This can be included on the label below or added as a separate label.

Units that have passed this test procedure shall be packaged into anti-static ESD approved bags, labeled per table below and shipped per the P.O.

| Label 1 Assembly Number+Dash Number (if Applicable) | Label 2 IC Number |
| --- | --- |
| AMPS084 | OPA3S328EVM |

  - NON-CONFORMING MATERIAL

If yield loss is 2% or less, scrap non-conforming units and adjust P.O. to reflect total amount shipped. If yield loss approaches, or exceeds 5%, contact EVM coordinator for assistance.

Change History:

| Date | Changes | Reason | By |
| --- | --- | --- | --- |
| 09/08/2020 | Original |  |  |