Hello,

Could you please confirm the intended latch configuration for the TLV3901?

Based on the datasheet, my understanding is that latch mode is disabled when:

- LE is connected to a voltage at least 200 mV below VCCO,

- LEB is left floating,

- and both LE/LEB pins are internally terminated with 50Ω to VCCO.

However, I am seeing different behavior in the TINA  model. When configured according to the datasheet recommendation, the comparator produces no output. The simulation only works when:

- LEB is connected directly to VCCO,

- and LE is connected to approximately VCCO − 200 mV.

The TINA model behavior suggests that the internal pull-up resistors for the LE/LEB pins may not be implemented in the model.

Could you please confirm whether this is a limitation or an issue with the simulation model?

Also, could you confirm that the following hardware configuration is valid for normal (non-latched) operation:

- LEB left floating,

- LE pulled down to GND through a 500Ω resistor.

Thank you very much for your support.