# UniSite Data I/O

## Quick Start

## Main menu - at c:\ prompt type US

#### Programming Concepts

### Select Device    - UniSite selects the appropriate programming algorithm based on manufacturer and device part number selected.

### Insert Device     - Insert Device into socket.

### Load Data          - Load data from device, from UniSite’s internal disk drives or from one of the two serial ports on UniSite.

### Program Device - Transfers device data into device inserted in UniSite.

### Verify Device      - Compares the data in a programmed device against the data in UniSite’s RAM or the data in a disk file.

## Transfering Jedec Code

### Select Device

Insert Device

Load Device

### More commands    - Select More commands menu.

Transfer data         - Menu allows you to transfer data to and from PC.

Download data      - Transfer data from PC (or disk) to Data I/O.

Upload data           - Transfer data from Data I/O to PC (or disk)

UPLOAD DATA TO HOST

Source (Ram, Disk)	                     R

Destination (Remote, Terminal)	T

I/O translation Format                  91

Upload host command

##### Transfer c:\xxxxx.jed

### Getting Around the MENU

### System commands

ENTER      Executes command

CTRL-Z     Abort current operation

SPACE    Toggle a parameter

### Paging commands

CTRL-P    Display previous page

CTRL-N    Display next page

CTRL-R    Repaint screen

##### F1: Main menu

##### F2:  Previous menu

F3:  Online help for current menu & cursor position