VCO R1 and C1

Please let me know what you think...

My fo, center frequency calculation  is:

fmin=520 kHz

fMmax=1750 kHz

1750 kHz- 520- kHz = 1230 kHz for the range

Half above half below fo

Half 1230 = 615 kHz

Midpoint = 520 + 615 = 1135 kHz

Roughly for ease of use, 1000 kHz = fo

1 mHz = 1000 kHz  = 1x10**6..   (looks like you had used 1x10**5?)

For 1000pF (1x10**3), 1 nF....

VISUAL ITTERPOLATION...kind of a pain for log scales....but we'll try here for expediency...

R1 @ 4.5V = about 5K ohms,  R1 @ 6V = about 8K ohms,

5V is 1/3 the distance to 6V, 8k-5k=3k, 1k per .5V

So, 5V = 5k  + 1k = 6k ohms for R1 @ 5v with a 1000 pF capacitor

You had gotten 220k to 330k.... we have a  large difference....

Could you check what you did and let me know what you think?

Also my design calls for a 180 pF capacitor for C1, just curious what you end up with for a corresponding R1 ?

LOW PASS FILTER

Can you find me a similar solution to the lead / lag filter as in scha002?  Another design requirement...