| www.ti.com Low Power Wireless Address: Gaustadalléen 21 N-0349 Oslo, NORWAY Phone: +47 22 95 85 44 Fax: +47 22 95 85 46 Web: www.ti.com | Memo |  |  |
| --- | --- | --- | --- |
|  | Title: CC11xx PA High Level Description |  |  |
|  | Project number: | Project name: CC1101 |  |
|  | Author: Sverre Hellan |  |  |
|  | Document type: Memo |  |  |
|  | Classification: Confidential (NDA) |  |  |
|  | Last update: |  | Pages: 2 |
| Filename: input.doc |  |  |  |

CC11xx Power Amplifier

The CC11xx power amplifier consists of 4 different stages covering different output power levels. Figure 1 shows the overall block diagram of the PA.

Figure 1: Block diagram

The programming of the different stages is slightly complex.

PA_POWER[7:6] = 11: Stage 11 and 10 are turned on. The supply voltage to the pre-drivers is set to 1.79V and the supply voltage for the output stage and hence the output power is controlled by PA_POWER[3:0]. PA_POWER[5:4] are ‘don’t care’.

Example 1: 0xC0 to 0xCF gives decreasing output power.

Example 2: 0xD0 to 0xDF, 0xE0 to 0xEF, 0xF0 to 0xFF gives same output power as 0xC0 to 0xCF.

PA_POWER[7:6] = 10: Stage 10 is turned on. The supply voltage to the pre-drivers is set to 1.64V and the supply voltage of the output stage and hence the output power is controlled by PA_POWER[3:0]. PA_POWER[5:4] are ‘don’t care’.

Example 3: 0x80 to 0x8F gives decreasing output power.

Example 4: 0x90 to 0x9F, 0xA0 to 0xAF, 0xB0 to 0xBF gives same output power as 0x80 to 0x8F.

PA_POWER[7:6] = 01: Stage01 is turned on. This stage has 3 sub-settings:

1) PA_POWER[5:4] = 10:

The power supply for the output stage is set to 1.07V (nominal). The amplitude of the input signal to the output stage is controlled by setting PA_POWER[3:0]. PA_POWER[3:0] = 00 sets the supply voltage to the pre-drivers to a maximum, hence giving maximum input amplitude to the output stage.

Example 5: 0x6F to 0x60 gives increasing output power.

2) PA_POWER[5:4] = 01: The power supply for the output stage is set to 1.36V (nominal). The amplitude of the input signal to the output stage is controlled by setting PA_POWER[2:0]. PA_POWER[2:0] = 00 sets the supply voltage to the pre-drivers to a maximum, hence giving maximum input amplitude to the output stage.

Example 6: 0x57 to 0x50 gives increasing output power.

3) PA_POWER[5:4] = 00: The power supply for the pre-drivers and the output stage is set to 1.64V (nominal).

PA_POWER[7:6] = 00: Stage 00 is turned on. PA_POWER[5:0] controls the bias current to the stage and hence the output power.

Example 7: 0x00 to 0x0F gives increasing output power.