- Copy \Target\FR5739_I2C folder and rename as \Target\FR5969_I2C

- Import project in CCS

- Migrate MSPBoot

  - Select the target configuration (in this case, BSLBased_DualImg)

  - Change device variant to MSP430FR5969

  - Modify the desired .cmd file (in this case, lnk_msp430FR5739_I2C_1_5KB_Dual_Boot.cmd)

    - Use perl script as a guide

    - Modify manually if necessary, check datasheet for memory regions

    - Rename file (in this case, lnk_msp430FR5969_I2C_2KB_Dual_Boot.cmd)

  - CCS will add a new .cmd file (lnk_msp430fr5969.cmd). Delete that file.

  - Modify device initialization

    - Modify Clock_init to configure CS (check datasheet or code examples)

    - Modify MPU_init to configure MPU (check datasheet or code examples and make sure you use the new memory assignment from the .cmd file)

    - Modify HW_Init as needed. We’ll change the button to use P1.1 (instead of P4.1) and FR5969 needs to unlock GPIO

  - Modify CI

    - The PHY_DL layer can be modified as needed but in this case, we’ll use eUSCIB0 with P1[6:7] which is the same as FR5739

  - Modify for addresses above 16bits:

This device has more than 16-bit memory, but the application always resides and executes below 16bits.  Because of this, pointers can use 16-bits, except when writing the download area and validating the application

    - Modify Flash routines

      - Add GetPhysicalAddressFromVirtual to calculate download address from application address

      - Modify TI_MSPBoot_MI_EraseSector and TI_MSPBoot_MI_WriteByte to calculate the physical address correctly

      - Modify TI_MSPBoot_MI_EraseSectorDirect and TI_MSPBoot_MI_WriteByteDirect to use 32-bit addresses. Note that we can use intrinsics to write 32-bit addresses.

      - Modify TI_MSPBoot_MI_EraseSectorDirect and TI_MSPBoot_MI_WriteByteDirect according to new MPU settings

      - Modify TI_MSPBoot_MI_EraseAppDirect to erase both download areas

      - Modify TI_MSPBoot_MI_ReplaceApp to write App area with both download areas

      - Note that new constants can be added to the linker file as needed

  - Modify CRC calculation

    - Add crc16Init and crc16AddByte in order to calculate CRC for non-contiguous locations

    - Modify TI_MSPBoot_AppMgr_AppisValid to calculate CRC of app and download areas

- Migrate Apps

  - Select the target configuration (in this case, BSLBased_DualImg)

  - Change device variant to MSP430FR5969

  - Modify the desired .cmd file (in this case, lnk_msp430FR5739_I2C_1_5KB_Dual_App.cmd.cmd)

    - Use perl script as a guide

    - Modify manually if necessary, check datasheet for memory regions

    - Rename file (in this case, lnk_msp430FR5969_I2C_2KB_Dual_App.cmd)

  - CCS will add a new .cmd file (lnk_msp430fr5969.cmd). Delete that file.

  - Modify the app as needed

    - Change port for LEDs

    - Change port for Switch

    - Change definition of Vectors for dummy_isr

    - Use cast for pointers of Boot vectors to remove warnings

- Modify host

  - G2 demo is used for host

  - Add new target copying TARGET_FR5739 to TARGET_FR5969

  - Replace definition of TARGET_FR5739 -> TARGET_FR5969

  - Add new demo files for FR5969

    - Use perl script 430txt2c.pl to convert files as shown in PrepareCCSOutput_G2_FR5969.bat

  - Modify project to send more sections of application if needed (new App2 has 3 sections instead of 2)

- Test

  - Download G2Host to G2 Launchpad

  - Download MSPBoot to FR5969 Launchpad

  - If desired, debug MSPBoot

    - to check if device is getting data from I2C (TI_MSPBoot_CI_PHYDL_Poll)

    - If packets are decoded correctly (TI_MSPBoot_CI_Process)

    - If data is written correctly to Flash (TI_MSPBoot_MI_FRAMDualImg.c)

    - If CRC sent by host is the same as calculated (TI_MSPBoot_AppMgr_AppisValid)

    - If bootloader is jumping to App (TI_MSPBoot_APPMGR_JUMPTOAPP)

  - Run demo

    - After reset, LED2 and LED1 are ON in FR5969 launchpad

    - Press S2 in G2 launchpad, LED should blink twice and it will send App1

    - FR5969’s LED2 will blink twice fast and then it will blink slowly running App1

    - Pressing S2 in FR5969 launchpad forces bootloader mode

    - Press S2 in G2 Launchpad again, LED should blink twice and it will send App2

    - FR5969’s LED1 will blink twice. Pressing S2 will toggle LED1.

    - Pressing S2 in G2 launchpad again will force bootloader mode in FR5969 (by sending a special command via I2C) and then it will send App1 again.