James,

Thanks.

I done some modifications for use Uart_2 (See attached), after I modify the code, I rebuild all the projects in this order after making changes to any files: emeter-toolkit-67641 -> emeter-metrology-67641 -> emeter-app-67641,but still don’t enter the break point, see the attached.

Use IC: MSP430F67641IPN (80pins)

I done below modify for use Uart_1:

- P2 port setting:

Lucy modify (our application)

P2.0 = B0_SPI_MOSI  out

P2.1 = B0_SPI_MISO  in

P2.2 = UART 2 Rx    in

P2.3 = UART 2 Tx    out

P2.4 = PE_485_TX_EN  out

P2.5 = NC           out

P2.6 = LED_PHASE_2  out

P2.7 = LED_PHASE_3  out

*/

#define P2DIR_INIT                            (BIT1|BIT2| BIT3| BIT4| BIT5| BIT6|BIT7)

#define P2SEL_INIT                                  (BIT0 | BIT1| BIT3 | BIT2)

#define P2OUT_INIT                                  (BIT6 | BIT7)

#define P2REN_INIT                                  (0)

- Uart_2 setting (same as Uart_1):

/*! This defines the speed of eUSCI 2  Lucy add */

#define UART_2_SUPPORT

#define UART_2_BAUD_RATE                            9600

#define UART_2_DLT645_SUPPORT

- Uart_2 init (same as Uart_1): in this function: int  serial_configure(int port, int mode, uint32_t bit_rate)

#endif

#if defined(UART_2_SUPPORT)

case 2:

//Below Lucy modify refer COM1 setup

#if defined(__MSP430_HAS_USCI_AB2__)  ||  defined(__MSP430_HAS_USCI_A2__)  ||  defined(__MSP430_HAS_EUSCI_A2__)

/* Configure the port with the reset bit held high */

UCA2CTL1 |= UCSWRST;                        //Lucy

UCA2CTL0 = ctl0;

UCA2CTL1 = ctl1;

#if defined(__MSP430_HAS_EUSCI_A2__)

UCA2BRW = bitrate_divider;                 //Lucy

UCA2MCTLW = ((uint16_t) mctl << 8);

#else

UCA2BR1 = br1;

UCA2BR0 = br0;

UCA2MCTL = mctl;

UCA2STAT = 0;

#endif

UCA2TXBUF = 0;                               //Lucy

UCA2CTL1 &= ~UCSWRST;

#if defined(UART_2_DMA_SUPPORT)

DMACTL0 = DMA2TSEL_9 | DMA2TSEL_10;

#endif

#if defined(__MSP430_HAS_USCI_AB2__)

UC2IE |= UCA2RXIE;

#else

UCA2IE |= UCRXIE;                            //Lucy

#endif

#elif defined(__MSP430_HAS_UART2__)

UCTL2 = ctl0;

UTCTL2 = ctl1;

UBR12 = br1;

UBR02 = br0;

UMCTL2 = mctl;

UCTL2 &= ~SWRST;

/* Enable USART1 TXD/RXD */

ME2 |= (UTXE2 | URXE2);

IE2 |= URXIE2;

/* If you do not initially kick the Tx port, the TXEPT bit is not set. */

TXBUF2 = 0;

#endif

return 0;

- Uart_2 interrupt router(same as Uart_1):

窗体底端

#elif defined(__MSP430_HAS_EUSCI_A2__)

ISR(USCI_A2, USCI_A2_isr)

{

uint16_t tx;                 //Lucy can enter router

switch (__even_in_range(UCA2IV, USCI_UART_UCTXCPTIFG))

{

case USCI_NONE:

break;

case USCI_UART_UCRXIFG:

uart_rx_core(2, UCA2RXBUF);

break;

case USCI_UART_UCTXIFG:

tx = uart_tx_core(2);

UCA2TXBUF = tx & 0xFF;

if (tx & 0x8000)

UCA2IE &= ~UCTXIE;

break;

case USCI_UART_UCSTTIFG:

break;

case USCI_UART_UCTXCPTIFG:

break;

}

}

Uart_2: can not stop:

UART_1: Normal can stop and enter this break point.