#ifndef I2C_SW_H_

#define I2C_SW_H_

/*-----------------------------------------------------------------------------*/

/* Macro definitions  */

/*-----------------------------------------------------------------------------*/

/*-----------------------------------------------------------------------------*/

/* Local Macro definitions */

/*-----------------------------------------------------------------------------*/

#define I2C_P3SEL		PSEL

#define I2C_P3SEL2		P3SEL2

#define I2C_P3DIR		P3DIR

#define I2C_P3OUT		P3OUT

#define I2C_P3IN		P3IN

#define SCL        		BIT2

#define SDA         	BIT1

#define ACK 			0x00

#define NACK			0x01

#define TIME_DELAY 100

#define I2C_DELAY() __delay_cycles(TIME_DELAY)

/*-----------------------------------------------------------------------------*/

/* Function prototypes  */

/*-----------------------------------------------------------------------------*/

//NOTE: Need custom Read_SCL(), Read_SDA(), Clear_SCL(), Clear_SDA() to compatible Hardware.

unsigned char Read_SCL(void); // Set SCL as input and return current level of line, 0 or 1, nomal is 1 because pullup by res

unsigned char Read_SDA(void); // Set SDA as input and return current level of line, 0 or 1, nomal is 0 because pull by res

void Clear_SCL(void); // Actively drive SCL signal Low

void Clear_SDA(void); // Actively drive SDA signal Low

void I2C_Init(void);

void I2C_Start(void);

void I2C_Stop(void);

void I2C_Writebit(unsigned char bit);

unsigned char I2C_Readbit(void);

void I2C_WriteByte(unsigned char Data);

unsigned char I2C_ReadByte(void);

void I2C_WriteData(unsigned char *Data, unsigned char DevideAddr, unsigned char Register, unsigned char nLength);

void I2C_ReadData(unsigned char *Buff, unsigned char DevideAddr, unsigned char Register, unsigned char nLength);

#endif /* I2C_SW_H_ */

/*

* i2c_sw.c

*

*  Created on: Mar 25, 2019

*      Author: csio

*/

/********************************************************************************

Module		: I2C_SW

Author		: 05/04/2015, by KienLTb - https://kienltb.wordpress.com/

Description	: I2C software using bit-banging.

********************************************************************************/

/*-----------------------------------------------------------------------------*/

/* Header inclusions */

/*-----------------------------------------------------------------------------*/

#include "msp430f2274.h"

#include "i2c_sw.h"

/*-----------------------------------------------------------------------------*/

/* Local Macro definitions */

/*-----------------------------------------------------------------------------*/

/*-----------------------------------------------------------------------------*/

/* Function prototypes */

/*-----------------------------------------------------------------------------*/

unsigned char Read_SCL(void);

unsigned char Read_SDA(void);

void Clear_SCL(void);

void Clear_SDA(void);

void I2C_Delay(void);

void I2C_Init(void);

void I2C_Start(void);

void I2C_Stop(void);

void I2C_Writebit(unsigned char bit);

unsigned char I2C_ReadByte(void);

void I2C_WriteByte(unsigned char Data);

unsigned char I2C_ReadByte(void);

void I2C_WriteData(unsigned char *Data, unsigned char DevideAddr, unsigned char Register, unsigned char nLength);

void I2C_ReadData(unsigned char *Buff, unsigned char DevideAddr, unsigned char Register, unsigned char nLength);

/*--------------------------------------------------------------------------------

Function	: Read_SCL

Purpose		: Set SCL as input and return current Logic level of SCL (0 or 1)

nomal is 1 because pullup by resistor

Parameters	: None

Return		: Logic level of SCL pin

--------------------------------------------------------------------------------*/

unsigned char Read_SCL(void)

{

I2C_P3DIR  &= ~SCL;

return((I2C_P3IN & SCL) != 0);

}

/*--------------------------------------------------------------------------------

Function	: Read_SDA

Purpose		: Set SDA as input and return current Logic level of SDA (0 or 1),

nomal is 1 because pullup by resistor

Parameters	: None

Return		: Logic level of SDA pin

--------------------------------------------------------------------------------*/

unsigned char Read_SDA(void)

{

I2C_P3DIR  &= ~SDA;

return((I2C_P3IN & SDA) != 0);

}

/*--------------------------------------------------------------------------------

Function	: Clear_SCL

Purpose		: Set SCL as Out put, logic Low

Parameters	: None

Return		: None

--------------------------------------------------------------------------------*/

void Clear_SCL(void)

{

I2C_P3DIR  |= SCL;

}

/*--------------------------------------------------------------------------------

Function	: Clear_SDA

Purpose		: Set SDA as Out put, logic LOW

Parameters	: None

Return		: None

--------------------------------------------------------------------------------*/

void Clear_SDA(void)

{

I2C_P3DIR  |= SDA;

}

/*--------------------------------------------------------------------------------

Function	: I2C_Init

Purpose		: Initialize I2C block

Parameters	: None

Return		: None

--------------------------------------------------------------------------------*/

void I2C_Init(void)

{

// Conffig SCL and SDA as GPIO

//I2C_P3SEL   &= ~(SCL + SDA);

//I2C_P3SEL2  &= ~(SCL + SDA);

// Set SCL and SDA is logic HIGH

I2C_P3DIR   &= ~(SCL + SDA);

I2C_P3OUT  	&= ~(SCL + SDA);

}

/*--------------------------------------------------------------------------------

Function	: I2C_Start

Purpose		: Send start signal

Parameters	: None

Return		: None

--------------------------------------------------------------------------------*/

void I2C_Start(void)

{

Read_SDA();				//set SDA to 1

//I2C_Delay();

Clear_SDA();			//set SDA to 0, currently SCL is 1

//I2C_Delay();

Clear_SCL();			//set SCL to 0

}

/*--------------------------------------------------------------------------------

Function	: I2C_Stop

Purpose		: Send Stop signal

Parameters	: None

Return		: None

--------------------------------------------------------------------------------*/

void I2C_Stop(void)

{

Clear_SDA();			//set SDA to 0

//I2C_Delay();

Read_SCL();				//set SCL to 1

//I2C_Delay();

Read_SDA();				//set SDA to 1

}

/*--------------------------------------------------------------------------------

Function	: I2C_Delay

Purpose		: time delay

Parameters	: break

Return		: None

--------------------------------------------------------------------------------*/

void I2C_Delay(void)

{

__delay_cycles(100);

}

/*--------------------------------------------------------------------------------

Function	: I2C_Writebit

Purpose		: Write bit to I2C bus

Parameters	: a bit need to write

Return		: None

--------------------------------------------------------------------------------*/

void I2C_Writebit(unsigned char bit)

{

if(bit)

{ Read_SDA();

}

else{

Clear_SDA();

//I2C_Delay();

Read_SCL();

//I2C_Delay();

Clear_SCL();

}

}

/*--------------------------------------------------------------------------------

Function	: I2C_Readbit

Purpose		: Read bit to I2C bus

Parameters	: None

Return		: unsigned char

--------------------------------------------------------------------------------*/

unsigned char I2C_Readbit(void)

{

unsigned char bit;

//Let the slave driver data

Read_SDA();

//I2C_Delay();

Read_SCL();

bit = Read_SDA();

//I2C_Delay();

Clear_SCL();

return bit;

}

/*--------------------------------------------------------------------------------

Function	: I2C_WriteByte

Purpose		: Write a Byte to I2C bus

Parameters	: unsigned char Data

Return		: None

--------------------------------------------------------------------------------*/

void I2C_WriteByte(unsigned char Data)

{

unsigned char nBit;

for(nBit = 0; nBit < 8; nBit++)

{

I2C_Writebit((Data & 0x80) != 0);

Data <<= 1;

}

I2C_Readbit(); // Waite NACK

}

/*--------------------------------------------------------------------------------

Function	: I2C_ReadByte

Purpose		: Read a Byte to I2C bus

Parameters	: None

Return		: unsigned char

--------------------------------------------------------------------------------*/

unsigned char I2C_ReadByte(void)

{

unsigned char Buff = 0;

unsigned char nBit;

for(nBit = 0; nBit < 8; nBit++)

{

Buff = (Buff << 1) | I2C_Readbit();

}

return Buff;

}

/*--------------------------------------------------------------------------------

Function	: I2C_WriteData

Purpose		: Write n Byte to I2C bus

Parameters	: Data 			- Pointer to Data need to write

DevideAddr	- Devide Address

Register		- Register Address

nLength		- Number of Byte need to write

Return		: None

--------------------------------------------------------------------------------*/

void I2C_WriteData(unsigned char *Data, unsigned char DevideAddr, unsigned char Register, unsigned char nLength)

{

unsigned char nIndex;

I2C_Start();

I2C_WriteByte(DevideAddr << 1);  // byDeviceAddr is 7 bit and command is write

I2C_WriteByte(Register);

for(nIndex = 0; nIndex < nLength; nIndex++)

{

I2C_WriteByte(*(Data + nIndex));

}

I2C_Readbit();

I2C_Stop();

}

/*--------------------------------------------------------------------------------

Function	: I2C_ReadData

Purpose		: Read n Byte from I2C bus

Parameters	: Buff 			- Pointer to Buffer store value

DevideAddr	- Devide Address

Register		- Register Address

nLength		- Number of Byte need to read

Return		: None

--------------------------------------------------------------------------------*/

void I2C_ReadData(unsigned char *Buff, unsigned char DevideAddr, unsigned char Register, unsigned char nLength)

{

unsigned char nIndex;

I2C_Start();

I2C_WriteByte(DevideAddr << 1);

I2C_WriteByte(Register);

I2C_Stop();

_NOP(); 								// Short Delay

I2C_Start();

_NOP();									// Short Delay

I2C_WriteByte((DevideAddr << 1) | 1);

for(nIndex = 0; nIndex < nLength; nIndex++)

{

*(Buff + nIndex) = I2C_ReadByte();

if(nIndex > 0)I2C_Writebit(ACK);

}

IE2 |= UCB0TXIE;                          // Enable TX interrupt

while (UCB0CTL1 & UCTXSTP);             // Ensure stop condition got sent

// x1 = (((int)Buff[1]) << 8) | Buff[0];

//  y1 = (((int)Buff[3]) << 8) | Buff[2];

//  z1 = (((int)Buff[5]) << 8) | Buff[4];

I2C_Writebit(NACK);

I2C_Stop();

}

#include<msp430f2274.h>

#include<stdio.h>

//#include<i2c_sw.h>

//#define ADXL_345   0x53

unsigned char RX;

int main(void)

{

WDTCTL = WDTPW + WDTHOLD;                 // Stop WDT

P1DIR |= BIT0;

P1OUT |= BIT0;

// UART

BCSCTL1 = CALBC1_1MHZ;            // Set DCO to 1MHz

DCOCTL = CALDCO_1MHZ;             // Set DCO to 1MHz

// ADXL345

P3SEL  |= BIT2 + BIT1;                     // Assign I2C pins to USCI_B0

P3DIR |= 0X02;

//P3OUT |= 0X06;

IE2 &= ~UCB0RXIE;

//  while (UCB0CTL1 & UCTXSTP);               // Ensure stop condition got sent// Disable RX interrupt

UCB0CTL1 |= UCSWRST;                      // Enable SW reset

UCB0CTL0 = UCMST + UCMODE_3 + UCSYNC;     // I2C Master, synchronous mode

UCB0CTL1 = UCSSEL_2 + UCSWRST;            // Use SMCLK, keep SW reset

UCB0BR0 = 12;                             // fSCL = SMCLK/12 = ~100kHz

UCB0BR1 = 0;

UCB0I2CSA = 0X53;                       // Slave Address is 048h

UCB0CTL1 &= ~UCSWRST;                     // Clear SW reset, resume operation

while(1)

{// Init sequence for ADXL345

I2C_WriteData(0x32,0x53,0x32,6);

//void I2C_WriteData(0X01,0x53,0x31,1);

//void I2C_WriteData(0X06,0x53,0x32,6);

// I2C_ReadData(0x32,0X53,0X32,6);

}

__bis_SR_register( LPM3_bits + GIE);

_no_operation();

}

// x1 = (((int)RxBuffer[1]) << 8) | RxBuffer[0];

//y1 = (((int)RxBuffer[3]) << 8) | RxBuffer[2];

//z1 = (((int)RxBuffer[5]) << 8) | RxBuffer[4];

/// __delay_cycles(1000);

/* Go into low power mode 3, general interrupts enabled */