#include "SPI.h"

#include "OneMsTaskTimer.h"

#include "LCD_SharpBoosterPack_SPI.h"

#include <Wire.h>

#define ADDR 0x40

#define TEMP_LOW 0x00

#define TEMP_HIGH 0x01

#define HUMID_LOW 0x02

#define HUMID_HIGH 0x03

#define INTERRUPT_DRDY 0x04

#define TEMP_MAX 0x05

#define HUMID_MAX 0x06

#define INTERRUPT_CONFIG 0x07

#define TEMP_OFFSET_ADJUST 0x08

#define HUM_OFFSET_ADJUST 0x09

#define TEMP_THR_L 0x0A

#define TEMP_THR_H 0x0B

#define HUMID_THR_L 0x0C

#define HUMID_THR_H 0x0D

#define CONFIG 0x0E

#define MEASUREMENT_CONFIG 0x0F

#define MID_L 0xFC

#define MID_H 0xFD

#define DEVICE_ID_L 0xFE

#define DEVICE_ID_H 0xFF

// Variables

LCD_SharpBoosterPack_SPI myScreen;

uint8_t myOrientation = 0;

uint16_t myCount = 0;

float hdc_temp = 22.0;

float temperature = 0, humidity = 0;

void openReg(uint8_t reg)

{

Wire.beginTransmission(ADDR);     // Connect to HDC2010

Wire.write(reg);            // point to specified register

Wire.endTransmission();         // Relinquish bus control

}

uint8_t readReg(uint8_t reg)

{

openReg(reg);

uint8_t reading;          // holds byte of read data

Wire.requestFrom(ADDR, 1);     // Request 1 byte from open register

Wire.endTransmission();       // Relinquish bus control

if (1 <= Wire.available())

{

reading = (Wire.read());      // Read byte

}

return reading;

}

float readTemp(void)

{

uint8_t byte[2];

uint16_t temp;

byte[0] = readReg(TEMP_LOW);

byte[1] = readReg(TEMP_HIGH);

temp = (unsigned int)byte[1] << 8 | byte[0];

return (float)(temp) * 165 / 65536 - 40;

}

void writeReg(uint8_t reg, uint8_t data)

{

Wire.beginTransmission(ADDR);    // Open Device

Wire.write(reg);            // Point to register

Wire.write(data);           // Write data to register

Wire.endTransmission();       // Relinquish bus control

}

// Add setup code

void setup() {

Serial.begin(9600);

Wire.begin();

myScreen.begin();

myScreen.clearBuffer();

myScreen.setFont(0);

myScreen.text(3, 5, "HIIIII");

myScreen.flush();

//for (uint8_t i=0; i<20; i++) delay(100);

delay(200);

//----------------------------------

}

void write_lcd(int setTemp, float curTemp, int power, float energy, float cost)

{

myScreen.clearBuffer();

myScreen.setFont(0);

myScreen.text(3, 5, "Set Temp:");

myScreen.setFont(1);

myScreen.text(60, 3, String(setTemp));

myScreen.setFont(0);

myScreen.text(85, 7, "C");

String temps;

temps = String(curTemp,2);

temps.remove(0,2);

myScreen.setFont(0);

//myScreen.text(3, 21, "Current");

myScreen.text(3, 21, "Temp:");

myScreen.text(9, 32, "(C)");

myScreen.setFont(1);

myScreen.text(32, 23, temps);

//myScreen.setFont(0);

//myScreen.text(85, 27, "C");

myScreen.setFont(0);

myScreen.text(3, 48, "Power:");

myScreen.setFont(1);

myScreen.text(37, 43, String(power));

myScreen.setFont(0);

myScreen.text(87, 48, "W");

myScreen.setFont(0);

myScreen.text(3, 63, "Today's Energy:");

//myScreen.text(3, 80, "Energy:");

myScreen.setFont(1);

String energys;

energys = String(energy,1);

energys.remove(0,2);

myScreen.text(3, 75, energys);

myScreen.setFont(0);

myScreen.text(41, 74, "kWh");

myScreen.text(55, 84, "(Rs.24)");

myScreen.flush();

}

// Add loop code

void loop()

{

temperature = readTemp();

Serial.println(temperature);

write_lcd(22, temperature, 1630, 6.7, 2.2);

delay(1000);

temperature = readTemp();

Serial.println(temperature);

write_lcd(25, temperature, 1260, 4.9, 2.2);

delay(1000);

}