#include <msp430.h>

#include <stdio.h>

#include"lcdLib.h"

#define keyport P6OUT//Keypad Port

#define COL1 (0x10 & P6IN)

#define COL2 (0x22 & P6IN)

#define COL3 (0x40 & P6IN)

#define COL4 (0x80 & P6IN)

const unsigned char Msg1[] = "MATRIX KEYPAD ";

unsigned char lsb,msb;

unsigned int a=1,j=0;

unsigned char key_press;

unsigned char i,k,key=0;

unsigned char Key_Val[] = {' ','C','8','4','0','D','9','5','1','E','A','6','2','F','B','7'};

unsigned char get_key(void);

void DelayMs(int Ms);

void main(void)

{

unsigned int count=0;

unsigned char Value=0;

WDTCTL = WDTPW + WDTHOLD; // Stop WDT

P6DIR = 0x0F; // Set P2.0 to 2.3 Output ,Set P2.4 to 2.7 Input

P7DIR = 0x0F; // Set P2.0 to 2.3 Output ,Set P2.4 to 2.7 Input

P6REN = 0xFF; // Set P2.0 to 2.7 Pull up Register enable

P7REN = 0xFF; // Set P2.0 to 2.7 Pull up Register enable

P6OUT = 0xF0; // Set P2.0 to 2.7 Out Register.

P7OUT = 0xF0; // Set P2.0 to 2.7 Out Register.

DelayMs(100);

lcdInit();

DelayMs(50);

lcdWriteCmd(0x80);

for(j=0;j<16;j++)

{

lcdWriteData(Msg1[j]);

DelayMs(5);

}

while(1)

{

while((count = get_key())==0); //Wait untill a key Pressed

Value = Key_Val[count];

lcdWriteCmd(0xC0);

lcdWriteData(Value);

//P3OUT = ( Value << 4 );

}

}

unsigned char get_key(void)

{

k=1;

for(i=0;i<4;i++)

{

keyport = ((0x01<<i) ^ 0xff); //Scan for a Key by sending '0' on ROWS

if(!COL1) //when a key pressed numbers 1--16 will be returned

{

key = k+0;

while(!COL1);

return key;

}

if(!COL2)

{

key = k+1;

while(!COL2);

return key;

}

if(!COL3)

{

key = k+2;

while(!COL3);

return key;

}

if(!COL4)

{

key = k+3;

while(!COL4);

return key;

}

k+=4;

keyport |= (0x01<<i);

}

return 0;

}

void DelayMs(int Ms)

{

int k;

while(Ms>0)

{

for(k=0;k<104;k++);

Ms--;

}

}