#include "lcdLib.h"

#define	LOWNIB(x)	P3OUT = (P3OUT & 0xF0) + (x & 0x0F)

void lcdInit() {

delay_ms(100);

// Wait for 100ms after power is applied.

P3DIR=EN+RS+DATA;  // Make pins outputs

P3OUT=0x03;  // Start LCD (send 0x03)

lcdTriggerEN(); // Send 0x03 3 times at 5ms then 100 us

delay_ms(5);

lcdTriggerEN();

delay_ms(5);

lcdTriggerEN();

delay_ms(5);

P3OUT = 0x02; // Switch to 4-bit mode

lcdTriggerEN();

delay_ms(5);

lcdWriteCmd(0x28); // 4-bit, 2 line, 5x8

lcdWriteCmd(0x08); // Instruction Flow

lcdWriteCmd(0x01); // Clear LCD

lcdWriteCmd(0x06); // Auto-Increment

lcdWriteCmd(0x0F); // Display On, No blink

}

void lcdTriggerEN() {

P3OUT |= EN;

P3OUT &= ~EN;

}

void lcdWriteData(unsigned char data) {

P3OUT |= RS; // Set RS to Data

LOWNIB(data >> 4); // Upper nibble

lcdTriggerEN();

LOWNIB(data); // Lower nibble

lcdTriggerEN();

delay_us(50); // Delay > 47 us

}

void lcdWriteCmd(unsigned char cmd) {

P3OUT&=~RS; // Set RS to Data

LOWNIB(cmd >> 4); // Upper nibble

lcdTriggerEN();

LOWNIB(cmd); // Lower nibble

lcdTriggerEN();

delay_ms(5); // Delay > 1.5ms

}

void lcdSetText(char* text, int x, int y) {

int i;

if (x < 16) {

x |= 0x80; // Set LCD for first line write

switch (y){

case 1:

x |= 0x40; // Set LCD for second line write

break;

case 2:

x |= 0x60; // Set LCD for first line write reverse

break;

case 3:

x |= 0x20; // Set LCD for second line write reverse

break;

}

lcdWriteCmd(x);

}

i = 0;

while (text[i] != '\0') {

lcdWriteData(text[i]);

i++;

}

}

void lcdSetInt(int val, int x, int y){

char number_string[16];

sprintf(number_string, "%d", val); // Convert the integer to character string

lcdSetText(number_string, x, y);

}

void lcdClear() {

lcdWriteCmd(CLEAR);

}

void lcd_string(char *p)

{

while(*p)

lcdWriteData(*p++);

}