/*********************************************************************

LCD Driver Functions

by: Elliott Gurrola

LCD Pinouts

Pin 1	Ground

Pin 2	VCC (+3.3 to +5V)

Pin 3	Contrast adjustment

Pin 4	Register Select (RS). 0: Command, 1: Data

Pin 5	Read/Write (R/W). 0: Write, 1: Read		-> Set to GND for constant write

Pin 6	Clock (Enable). Falling edge triggered

Pin 7	Bit 0 (Not used in 4-bit operation)

Pin 8	Bit 1 (Not used in 4-bit operation)

Pin 9	Bit 2 (Not used in 4-bit operation)

Pin 10	Bit 3 (Not used in 4-bit operation)

Pin 11	Bit 4

Pin 12	Bit 5

Pin 13	Bit 6

Pin 14	Bit 7

Pin 15	Backlight Anode (+)

Pin 16	Backlight Cathode (-)

Top level functions available

- lcd_init();					// Initialize LCD

- write_byte(char l);			// Send a single character (1 byte)

- write_string(char s[]);		// Print a character string

- write_int(int num);			// Write an integer

- gotoXy(x, y);					// Move cursor to (x, y) location (Lines and columns start at 0)

- clear_lcd();					// Clear LCD and move cursor to (0, 0)

Not important to the user:

- send_cmd(char cmd);			// Send a command to the LCD

- write_nibble(char l);			// Send a nibble to the LCD

- void trigger_EN();

*** DELAY FUNCTION ASSUMES 1 MHz CLOCK ***

Modified on: Apr 15, 2013

by: Luiz (Luis Carlos Ba?uelos-Chacon)

Modified on: May 24, 2013

by: Elias N Jaquez

*********************************************************************/

#ifndef MINI_PROJECT_CIRCULAR_DISPLAY_LCDLIB_H_

#define MINI_PROJECT_CIRCULAR_DISPLAY_LCDLIB_H_

#include <msp430.h>

#include <string.h>

#include <stdio.h>

// Delay Functions

#define delay_ms(x)		__delay_cycles((long) x* 1000)

#define delay_us(x)		__delay_cycles((long) x)

// Pins

#define EN		BIT4

#define RS		BIT5

#define DATA	0x0F

// Commands

#define CLEAR	0x01

// Functions

void lcdInit();									// Initialize LCD

void lcdTriggerEN();							// Trigger Enable

void lcdWriteData(unsigned char data);			// Send Data (Characters)

void lcdWriteCmd(unsigned char cmd);			// Send Commands

void lcdClear();								// Clear LCD

void lcdSetText(char * text, int x, int y);		// Write string

void lcdSetInt(int val, int x, int y);			// Write integer

void lcd_string(char *t);

#endif /* MINI_PROJECT_CIRCULAR_DISPLAY_LCDLIB_H_ */