#include "msp432p401r.h"

#include <stdint.h>

int main(void)

{

WDT_A->CTL = WDT_A_CTL_PW + WDT_A_CTL_HOLD;            // Stop WDT

/* NOTE: This example assumes the default power state is AM0_LDO.

* Refer to  msp432p401x_pcm_0x code examples for more complete PCM

* operations to exercise various power state transitions between active

* modes.

*/

/* Step 1: Transition to VCORE Level 1: AM0_LDO --> AM1_LDO */

while ((PCM->CTL1 & PCM_CTL1_PMR_BUSY));

PCM->CTL0 = PCM_CTL0_KEY_VAL | PCM_CTL0_AMR_1;

while ((PCM->CTL1 & PCM_CTL1_PMR_BUSY));

/* Step 2: Configure Flash wait-state to 1 for both banks 0 & 1 */

FLCTL->BANK0_RDCTL = (FLCTL->BANK0_RDCTL & ~(FLCTL_BANK0_RDCTL_WAIT_MASK)) |

FLCTL_BANK0_RDCTL_WAIT_1;

FLCTL->BANK1_RDCTL  = (FLCTL->BANK0_RDCTL & ~(FLCTL_BANK1_RDCTL_WAIT_MASK)) |

FLCTL_BANK1_RDCTL_WAIT_1;

/* Step 3: Configure DCO to 48MHz, ensure MCLK uses DCO as source*/

CS->KEY = CS_KEY_VAL ;                  // Unlock CS module for register access

CS->CTL0 = 0;                           // Reset tuning parameters

CS->CTL0 = CS_CTL0_DCORSEL_5;           // Set DCO to 48MHz

// Select MCLK = DCO=48MHz and Configure SMCLK=MCLK/4= 12MHz(SMCLK source divider)

CS->CTL1 = CS->CTL1 & ~(CS_CTL1_SELM_MASK | CS_CTL1_DIVM_MASK) | CS_CTL1_SELM_3 | CS_CTL1_SELS_3 | CS_CTL1_DIVS_2;

CS->KEY = 0;                            // Lock CS module from unintended accesses

// Enable global interrupt

__enable_irq();

volatile uint32_t i;

P1->DIR |= BIT0;                        // P1.0 set as output

while (1)                               // continuous loop

{

P1->OUT ^= BIT0;                    // Blink P1.0 LED

for (i = 20000; i > 0; i--);        // Delay

}

}