Description: Initializes ADC-A&B on the F28x7x

**********************************************************************/

void InitAdca(void)

{

asm(" EALLOW");						// Enable EALLOW protected register access

//--- Reset the ADC.  This is good programming practice.

DevCfgRegs.SOFTPRES13.bit.ADC_A = 1;	// ADC is reset

DevCfgRegs.SOFTPRES13.bit.ADC_A = 0;	// ADC is released from reset

//MZ--- Reset the ADC.  This is good programming practice.

DevCfgRegs.SOFTPRES13.bit.ADC_B = 1;    // ADC is reset

DevCfgRegs.SOFTPRES13.bit.ADC_B = 0;    // ADC is released from reset

//*****END MZ

//--- Configure the ADC base registers

AdcaRegs.ADCCTL1.all = 0x0004;		// Main ADC configuration

AdcbRegs.ADCCTL1.all = 0x0004;

// bit 15-14     00:     reserved

// bit 13        0:      ADCBSY, ADC busy, read-only

// bit 12        0:      reserved

// bit 11-8      0's:    ADCBSYCHN, ADC busy channel, read-only

// bit 7         0:      ADCPWDNZ, ADC power down, 0=powered down, 1=powered up

// bit 6-3       0000:   reserved

// bit 2         1:      INTPULSEPOS, INT pulse generation, 0=start of conversion, 1=end of conversion

// bit 1-0       00:     reserved

AdcaRegs.ADCCTL2.all = 0x0006;		// ADC clock configuration

AdcbRegs.ADCCTL2.all = 0x0006;

// bit 15-8      0's:    reserved

// bit 7         0:      SIGNALMODE, configured by AdcSetMode() below to get calibration correct

// bit 6         0:      RESOLUTION, configured by AdcSetMode() below to get calibration correct

// bit 5-4       00:     reserved

// bit 3-0       0110:   PRESCALE, ADC clock prescaler.  0110=CPUCLK/4

AdcaRegs.ADCBURSTCTL.all = 0x0000;

AdcbRegs.ADCBURSTCTL.all = 0x0000;

//--- Call AdcSetMode() to configure the resolution and signal mode.

//    This also performs the correct ADC calibration for the configured mode.

AdcSetMode(ADC_ADCA, ADC_RESOLUTION_12BIT, ADC_SIGNALMODE_SINGLE);

AdcSetMode(ADC_ADCB, ADC_RESOLUTION_12BIT, ADC_SIGNALMODE_SINGLE);

//--- SOC0 configuration

AdcaRegs.ADCSOC0CTL.bit.TRIGSEL = 7;		// Trigger using ePWM2-ADCSOCA

AdcaRegs.ADCSOC0CTL.bit.CHSEL = 0;			// Convert channel ADCINA0 (Ch. 0)

AdcaRegs.ADCSOC0CTL.bit.ACQPS = 19;			// Acquisition window set to (19+1)=20 cycles (100 ns with 200 MHz SYSCLK)

AdcaRegs.ADCINTSOCSEL1.bit.SOC0 = 0;		// No ADC interrupt triggers SOC0 (TRIGSEL field determines trigger)**MZ; i do not want the interrupt generated by another ADC to trigger soc0

AdcaRegs.ADCSOCPRICTL.bit.SOCPRIORITY = 0;	// All SOCs handled in round-robin mode

//***SOC0 configuration ADCB

AdcbRegs.ADCSOC0CTL.bit.TRIGSEL = 7;

AdcbRegs.ADCSOC0CTL.bit.CHSEL = 2;       // MZ***:used ch 2 (ADCINB2)of B port because it is what i have available in the lunch pad

//SOC0 will convert ADCINB2

AdcbRegs.ADCSOC0CTL.bit.ACQPS = 19;

AdcbRegs.ADCINTSOCSEL1.bit.SOC0 = 0;

AdcbRegs.ADCSOCPRICTL.bit.SOCPRIORITY = 0;

//--- ADCA1 interrupt configuration

AdcaRegs.ADCINTSEL1N2.bit.INT1CONT = 1;		// Interrupt pulses regardless of flag state

AdcaRegs.ADCINTSEL1N2.bit.INT1E = 1;		// Enable the interrupt in the ADC

AdcaRegs.ADCINTSEL1N2.bit.INT1SEL = 0;		// EOC0 triggers the interrupt

//---*** ADCB1 interrupt configuration

AdcaRegs.ADCINTSEL1N2.bit.INT1CONT = 1;

AdcaRegs.ADCINTSEL1N2.bit.INT1E = 1;

AdcaRegs.ADCINTSEL1N2.bit.INT1SEL = 0;

//--- Enable the ADC interrupt FOR ADCa

PieCtrlRegs.PIEIER1.bit.INTx1 = 1;			// Enable ADCA1 interrupt in PIE group 1

PieCtrlRegs.PIEIER1.bit.INTx2 = 1;

IER |= 0x0001;								// Enable INT1 in IER to enable PIE group1

//--- Finish up

AdcaRegs.ADCCTL1.bit.ADCPWDNZ = 1;			// Power up the ADC

DelayUs(1000);								// Wait 1 ms after power-up before using the ADC

asm(" EDIS");								// Disable EALLOW protected register access

//-** Finish up FOR ADCB

AdcbRegs.ADCCTL1.bit.ADCPWDNZ = 1;

DelayUs(1000);

asm(" EDIS");

//-END Finish up FOR ADCB

} // end InitAdc()

//--- end of file -----------------------------------------------------

**********************************************************************

* File: Main_6.c -- Solution File for Lab 6

* Devices: TMS320F28x7x

* Author: C2000 Technical Training, Texas Instruments

**********************************************************************/

#include "Lab.h"                        // Main include file

//--- Global Variables

Uint16 DEBUG_TOGGLE = 1;                // Used for realtime mode investigation test

Uint16 SINE_ENABLE = 0;                 // Used for DAC waveform generation

Uint16 AdcBuf[ADC_BUF_LEN];             // ADC buffer allocation

Uint16 AdcBuf1[ADC_BUF_LEN];            // ADC buffer1 allocation               ****mz

Uint16 DacOffset;                       // DAC offset

Uint16 DacOutput;                       // DAC output

/**********************************************************************

* Function: main()

*

* Description: Main function for C28x workshop labs

**********************************************************************/

void main(void)

{

//--- CPU Initialization

InitSysCtrl();						// Initialize the CPU (FILE: SysCtrl.c)

InitGpio();							// Initialize the shared GPIO pins (FILE: Gpio.c)

InitXbar();							// Initialize the input, output & ePWM X-Bar (FILE: Xbar.c)

InitPieCtrl();						// Initialize and enable the PIE (FILE: PieCtrl.c)

InitWatchdog();						// Initialize the Watchdog Timer (FILE: WatchDog.c)

//--- Peripheral Initialization

InitAdca();							// Initialize the ADC-A (FILE: Adc.c)

InitDacb();                         // Initialize the DAC-B (File: Dac.c)

InitEPwm();							// Initialize the EPwm (FILE: EPwm.c)

//--- Enable global interrupts

asm(" CLRC INTM, DBGM");			// Enable global interrupts and realtime debug

//--- Main Loop

while(1)							// endless loop - wait for an interrupt

{

asm(" NOP");

}

} //end of main()

/*** end of file *****************************************************/

ISRs for PIE vectors x.1 - x.8

//=====================================================================

//---------------------------------------------------------------------

interrupt void ADCA1_ISR(void)						// PIE1.1 @ 0x000D40  ADC-A interrupt #1

{

static Uint16 *AdcBufPtr = AdcBuf;					// Pointer to buffer

static Uint16 iQuadratureTable = 0;					// Quadrature table index

static volatile Uint16 GPIO34_count = 0;			// Counter for pin toggle

PieCtrlRegs.PIEACK.all = PIEACK_GROUP1;			// Must acknowledge the PIE group

//--- Manage the ADC registers

AdcaRegs.ADCINTFLGCLR.bit.ADCINT1 = 1;			// Clear ADCINT1 flag

//--- Read the ADC result

*AdcBufPtr++ = 	AdcaResultRegs.ADCRESULT0;			// Read the result

//--- Brute-force the circular buffer

if( AdcBufPtr == (AdcBuf + ADC_BUF_LEN) )      // ***mz: adc buffer + adc buffer length

{

AdcBufPtr = AdcBuf;						// Rewind the pointer to beginning

}

//--- Example: Toggle GPIO18 so we can read it with the ADC ***/

if(DEBUG_TOGGLE == 1)

{

GpioDataRegs.GPATOGGLE.bit.GPIO18 = 1;		// Toggle the pin

}

//--- Example: Toggle GPIO34 at a 0.5 sec rate (connected to the LED).

//             (1/50000 sec/sample)*(1 samples/int)*(x interrupts/toggle) = (0.5 sec/toggle)

//             ==> x = 25000

if(GPIO34_count++ > 25000)					// Toggle slowly to see the LED blink

{

GpioDataRegs.GPBTOGGLE.bit.GPIO34 = 1;	// Toggle the pin

GPIO34_count = 0;						// Reset the counter

}

//--- Write to DAC-B to create input to ADC-A0

if(SINE_ENABLE == 1)

{

DacOutput = DacOffset + ((QuadratureTable[iQuadratureTable++] ^ 0x8000) >> 5);

}

else

{

DacOutput = DacOffset;

}

if(iQuadratureTable > (SINE_PTS - 1))       // Wrap the index

{

iQuadratureTable = 0;

}

DacbRegs.DACVALS.all = DacOutput;

}

//---------------------------------------------------------------------

//interrupt void ADCB1_ISR(void)						// PIE1.2 @ 0x000D42  ADC-B interrupt #1

//{

//PieCtrlRegs.PIEACK.all = PIEACK_GROUP1;			// Must acknowledge the PIE group

// Next two lines for debug only - remove after inserting your ISR

//	asm (" ESTOP0");								// Emulator Halt instruction

//	while(1);

//}

//********MZ*****

interrupt void ADCB1_ISR(void)                      // PIE1.1 @ 0x000D40  ADC-B4] interrupt #1

{

static Uint16 *AdcBufPtr1 = AdcBuf1;                  // Pointer to buffer

//static Uint16 iQuadratureTable = 0;                  // Quadrature table index

static volatile Uint16 GPIO34_count = 0;            // Counter for pin toggle

PieCtrlRegs.PIEACK.all = PIEACK_GROUP1;         // Must acknowledge the PIE group

//--- Manage the ADC registers

AdcbRegs.ADCINTFLGCLR.bit.ADCINT1 = 1;          // Clear ADCINT1 flag

//--- Read the ADC result

*AdcBufPtr1++ =  AdcbResultRegs.ADCRESULT0;          // Read the result

//--- Brute-force the circular buffer

if( AdcBufPtr1 == (AdcBuf1 + ADC_BUF_LEN) )

{

AdcBufPtr1 = AdcBuf1;                     // Rewind the pointer to beginning

}

//--- Example: Toggle GPIO18 so we can read it with the ADC ***/

if(DEBUG_TOGGLE == 1)

{

GpioDataRegs.GPATOGGLE.bit.GPIO18 = 1;      // Toggle the pin

}

}

//------------------------------------