## Steps to allow slave select in motorware

Based on Example_2806xExternalInterrupt.c from ControlSuite

Use XINT1 for SPISTEA falling edge = start communication with that slave
Use XINT2 for SPISTEA rising edge = stop communication with that slave

Interrupt prioritization based on https://e2e.ti.com/support/microcontrollers/c2000/f/902/t/462460

All additional code is indicated by :
- START : // ADDED CODE FOR 2 SPI SLAVES - BEGIN
- END : // ADDED CODE FOR 2 SPI SLAVES - END

## proj_lab05e.c (in my case)

(1) Add:

#ifdef FLASH

#pragma CODE_SECTION(mainISR,"ramfuncs");

// ADDED CODE FOR 2 SPI SLAVES - BEGIN

#pragma CODE_SECTION(Interrupt_SPI_enableTx,"ramfuncs");

#pragma CODE_SECTION(Interrupt_SPI_disableTx,"ramfuncs");

// ADDED CODE FOR 2 SPI SLAVES - END

#endif

(2) Add in void main(void):

// set the hardware abstraction layer parameters

HAL_setParams(halHandle,&gUserParams);

// ADDED CODE FOR 2 SPI SLAVES - BEGIN

// Disable slave output to allow multiple slaves

// was enabled in HAL_setParams

SPI_disableTx(halHandle->spiAHandle);

// ADDED CODE FOR 2 SPI SLAVES - END

…

// initialize the interrupt vector table

HAL_initIntVectorTable(halHandle);

// ADDED CODE FOR 2 SPI SLAVES - BEGIN

// enable the External interrupts

HAL_enableExtInts(halHandle);

// ADDED CODE FOR 2 SPI SLAVES - END

// enable the ADC interrupts

HAL_enableAdcInts(halHandle);

(3) Add at end of mainISR:

interrupt void mainISR(void)

{

static uint16_t stCnt = 0;

// acknowledge the ADC interrupt

HAL_acqAdcInt(halHandle,ADC_IntNumber_1);

// ADDED CODE FOR 2 SPI SLAVES - BEGIN

// When entering ISR, all interrupt requests are put on hold

// Store interrupt settings for ADCINT1 PIE_Group

volatile unsigned short TempPIEIER = PIE_getIntEnables(halHandle->pieHandle,PIE_GroupNumber_10);

// Re-enable CPU group with XINT1 & XINT2

CPU_enableInt(halHandle->cpuHandle,CPU_IntNumber_1);

// Disable CPU group with ADCINT1

CPU_disableInt(halHandle->cpuHandle,CPU_IntNumber_10);

// Disable PIE channel ADCINT1

PIE_disableInt(halHandle->pieHandle,PIE_GroupNumber_10,PIE_InterruptSource_ADCINT_10_1);

PIE_clearAllInts(halHandle->pieHandle);

__asm("  NOP");

ENABLE_INTERRUPTS; //EINT;

// ADDED CODE FOR 2 SPI SLAVES - END

(4) Add at beginning of mainISR:

// ADDED CODE FOR 2 SPI SLAVES - BEGIN

// Restore registers saved:

// Disable interrupts before they will be automatically re-enabled after closing mainISR

CPU_disableGlobalInts(halHandle->cpuHandle); //DINT;

// Re-enable PIE channel ADCINT1

PIE_enableInt(halHandle->pieHandle,PIE_GroupNumber_10,(PIE_InterruptSource_e)TempPIEIER); //PieCtrlRegs.PIEIER1.all = TempPIEIER;

// ADDED CODE FOR 2 SPI SLAVES - END

return;

} // end of mainISR() function

(5) Add after mainISR:

// ADDED CODE FOR 2 SPI SLAVES - BEGIN

interrupt void Interrupt_SPI_enableTx(void)

{

// acknowledge the External interrupt

HAL_acqExtInt(halHandle);

// enable SPI SOMI

SPI_enableTx(halHandle->spiAHandle);

return;

} // end of Interrupt_SPI_enableTx() function

interrupt void Interrupt_SPI_disableTx(void)

{

// acknowledge the External interrupt

HAL_acqExtInt(halHandle);

// disable SPI SOMI

SPI_disableTx(halHandle->spiAHandle);

return;

} // end of Interrupt_SPI_disableTx() function

// ADDED CODE FOR 2 SPI SLAVES - END

## hal.h

(1) Add:

extern interrupt void mainISR(void);

// ADDED CODE FOR 2 SPI SLAVES - BEGIN

extern interrupt void Interrupt_SPI_enableTx(void);

extern interrupt void Interrupt_SPI_disableTx(void);

// ADDED CODE FOR 2 SPI SLAVES - END

(2) Add (why not only prototype here and definition in hal.c?) :

// ADDED CODE FOR 2 SPI SLAVES - BEGIN

//! \brief     Acknowledges an External interrupt so that another External interrupt can

//!            happen again.

//! \param[in] handle     The hardware abstraction layer (HAL) handle

//! \param[in] intNumber  The interrupt number

//static inline void HAL_acqExtInt(HAL_Handle handle,const ADC_IntNumber_e intNumber)

static inline void HAL_acqExtInt(HAL_Handle handle)

{

HAL_Obj *obj = (HAL_Obj *)handle;

// Acknowledge interrupt from PIE group 1

PIE_clearInt(obj->pieHandle,PIE_GroupNumber_1);

return;

} // end of HAL_acqExtInt() function

// ADDED CODE FOR 2 SPI SLAVES - END

(3) Add:

// ADDED CODE FOR 2 SPI SLAVES - BEGIN

//! \brief      Enables the SPI-STEA interrupts

//! \details    Enables the SPI-STEA interrupt in the PIE, and CPU.  Enables the

//!             interrupt to be sent from the SPI-STEA peripheral.

//! \param[in]  handle  The hardware abstraction layer (HAL) handle

extern void HAL_enableExtInts(HAL_Handle handle);

// ADDED CODE FOR 2 SPI SLAVES - END

(4) Add in static inline void HAL_initIntVectorTable(HAL_Handle handle):

pie->ADCINT1 = &mainISR;

// ADDED CODE FOR 2 SPI SLAVES - BEGIN

pie->XINT1 = &Interrupt_SPI_enableTx;

pie->XINT2 = &Interrupt_SPI_disableTx;

// ADDED CODE FOR 2 SPI SLAVES - END

## hal.c

// ADDED CODE FOR 2 SPI SLAVES - BEGIN

void HAL_enableExtInts(HAL_Handle handle)

{

HAL_Obj *obj = (HAL_Obj *)handle;

// enable the PIE interrupts associated with the External interrupts

PIE_enableExtInt(obj->pieHandle,CPU_ExtIntNumber_1);

PIE_enableExtInt(obj->pieHandle,CPU_ExtIntNumber_2);

PIE_setExtIntPolarity(obj->pieHandle,CPU_ExtIntNumber_1,PIE_ExtIntPolarity_FallingEdge);

PIE_setExtIntPolarity(obj->pieHandle,CPU_ExtIntNumber_2,PIE_ExtIntPolarity_RisingEdge);

ENABLE_PROTECTED_REGISTER_WRITE_MODE;

GPIO_setExtInt(obj->gpioHandle,GPIO_Number_19,CPU_ExtIntNumber_1);

GPIO_setExtInt(obj->gpioHandle,GPIO_Number_19,CPU_ExtIntNumber_2);

DISABLE_PROTECTED_REGISTER_WRITE_MODE;

// enable the cpu interrupt for external interrupts

CPU_enableInt(obj->cpuHandle,CPU_IntNumber_1);

return;

} // end of HAL_enableExtInts() function

// ADDED CODE FOR 2 SPI SLAVES - END