//Dedicated pin on the board

//GPIO0 to GPIO11 - PWM output pins

//GPIO17 - switch SW2

//GPIO18, GPIO19 - uart communication

//GPIO22 - connected to slide switch

//GPIO20, GPIO21, GPIO23 - encoder connected to j2 or rotary switch connected to j7

//GPIO24, GPIO25, GPIO26 - encoder connected to J3

//GPIO27 - LED D8

//GPIO32, GPIO33 - i2c bus for dac

//GPIO76 - /load dac output, GPIO77 - RDY input

//parallel dac

//GPIO61, GPIO62- a0,a1 address selection

//GPIO63 - /wr

//GPIO64 to GPIO71 - 8BIT data to

//GPIO72 - /load dac

//ADCINB7 connected to 10K pot

// do not connect GPIO87, GPIO86, GPIO85 and GPIO84 to boot from Flash

//===============================================================================

#include "DSP28x_Project.h"

#include "DSP2833x_Device.h"

#include "DSP2833x_Examples.h"

#include "math.h"

#define LED_D8 GpioDataRegs.GPADAT.bit.GPIO27

#define SW2 GpioDataRegs.GPADAT.bit.GPIO17

#define SW4 GpioDataRegs.GPADAT.bit.GPIO22

#define ADC_MODCLK 0x3

#define ADC_CKPS   0x1 //ADC module clock= HSPCLK/2*ADC_CKPS = 25MHz/(1*2)= 12.5MHz

#define ADC_SHCLK  0xf   // S/H width in ADC module periods

// 2833x ADC requires a 5-ms delay after all of the circuits are powered up

#define ADC_usDELAY 5000L

#define N 5000 // integration of actual stator-flux

float Vas,Vbs,Vcs,Vds,Vqs;

float ias,ibs,ics;

float ids,iqs;

float pi=3.142857;

int i;

float D_ref=0.632;

double D_act;

float square;

float D_ds,D_qs;

float theta,theta_r,theta_m,wr,wm,Te,Tm,p,J;

float err_speed,err_t,err_flux;

float flux_ref=0.632;

float speed_ref=150;

float ramplowlimit=-1;

float ramphighlimit=1;

float ramp_count=0;

float ramp_final;

float T_load=1;

float Ht,Hd;

unsigned int sec,vec,out;

float Sa1,Sa2,Sb1,Sb2,Sc1,Sc2;

int temp[6];

int pulse[3];

float Lm,Ls,Rs;

float Rr=0;

float Lr=0;

float Kp=50;

float Ki=0.05;

float i_max=40;

float i_min=-40;

float prop,i_temp,integral,PI_out;

Uint16 ReceivedChar;

void clarke(void);

void inv_clarke(void);

void actual_flux(void);

void motor_torque(void);

void sector(void);

void Calc_SPEED(void);

void speed_reference(void);

void pi_controller(void);

void T_hysteresis(void);

void f_hysteresis(void);

void lookup_table(void);

void delay_loop_int(unsigned int dly_int);

void delay_loop(void);

void Gpio_select(void);

void Gpio_example1(void);

void Gpio_example2(void);

void Gpio_example3(void);

void init28335(void);

Uint16 update_MCP4728(void);

void update_AD7305(void);

Uint16 config_MCP4728(void);

void switch_led_example(void);

void I2CA_Init(void);

void Init_QEP(void);

void pwm_example(void);

void init_pwm(void);

interrupt void epwm1_isr(void);

void adc_example(void);

void init_adc(void);

void test_rs232(void);

void scib_echoback_init(void);

void scib_xmit(int a);

void scib_fifo_init(void);

unsigned int MCP4728_CH_A=0, MCP4728_CH_B=0, MCP4728_CH_C=0, MCP4728_CH_D=0;

unsigned int AD7305_CH_A=0, AD7305_CH_B=0, AD7305_CH_C=0, AD7305_CH_D=0;

unsigned int ADCINA0IA,ADCINA1N,ADCINA2,ADCINA3,ADCINA4,ADCINA5,ADCINA6,ADCINA7;

unsigned int  ADCINB0IB, ADCINB1VDC, ADCINB2, ADC_POT;;

int ADCINB3, ADCINB4, ADCINB5, ADCINB6;

void main(void)

{

init28335();

//un-comment any one of the below examples and test

//Gpio_example1();

//parallel_dac_example();

//i2c_dac_exapmle();

//adc_example();

//switch_led_example();

//pwm_example();

//QEP_example();

//test_rs232();

for(;;);

}

//===============================================================================//

void delay_loop_int(unsigned int dly_int)

{

unsigned int k2;

unsigned int k1=10;

for (; k1 >0; k1--);

{

k2=dly_int;

for (; k2 >0; k2--);

}

}

//===============================================================================

void delay_loop()

{

long i;

for (i = 0; i < 1000000; i++) {}

}

//===============================================================================

void switch_led_example(void)

{

for(;;)

{

//if(SW2==1) LED_D8=1;		else LED_D8=0;

if(SW4==1) LED_D8=1;		else LED_D8=0;

}

}

//===============================================================================

void Gpio_example1(void)

{

// Example 1:

// Toggle all output GPIO on using DATA registers

for(;;)

{

GpioDataRegs.GPADAT.all    =0xAAAAAAAA;

GpioDataRegs.GPBDAT.all    =0xAAAAAAAA;

//    GpioDataRegs.GPBDAT.all    =0xAAAAAAAA; earlier code

GpioDataRegs.GPCDAT.all    =0xAAAAAAAA;

delay_loop();

GpioDataRegs.GPADAT.all    =0x55555555;

GpioDataRegs.GPBDAT.all    =0x55555555;

//GpioDataRegs.GPBDAT.all  =0x55555555; earlier code

GpioDataRegs.GPCDAT.all    =0x55555555;

delay_loop();

}

}

//===============================================================================

void Gpio_example2(void)

{

// Example 2:

// Toggle I/Os using SET/CLEAR registers

for(;;)

{

GpioDataRegs.GPASET.all    =0xAAAAAAAA;

GpioDataRegs.GPACLEAR.all  =0x55555555;

GpioDataRegs.GPBSET.all    =0x0000000A;

GpioDataRegs.GPBCLEAR.all  =0x00000005;

delay_loop();

GpioDataRegs.GPACLEAR.all    =0xAAAAAAAA;

GpioDataRegs.GPASET.all      =0x55555555;

GpioDataRegs.GPBCLEAR.all    =0x0000000A;

GpioDataRegs.GPBSET.all      =0x00000005;

delay_loop();

}

}

//===============================================================================

void Gpio_example3(void)

{

// Example 2:

// Toggle I/Os using TOGGLE registers

// Set pins to a known state

GpioDataRegs.GPASET.all    =0xAAAAAAAA;

GpioDataRegs.GPACLEAR.all  =0x55555555;

GpioDataRegs.GPBSET.all    =0x0000000A;

GpioDataRegs.GPBCLEAR.all  =0x00000005;

// Use TOGGLE registers to flip the state of

// the pins.

// Any bit set to a 1 will flip state (toggle)

// Any bit set to a 0 will not toggle.

for(;;)

{

GpioDataRegs.GPATOGGLE.all =0xFFFFFFFF;

GpioDataRegs.GPBTOGGLE.all =0x0000000F;

delay_loop();

}

}

//===============================================================================

void Gpio_select(void)

{

EALLOW;

GpioCtrlRegs.GPAMUX1.all = 0x00555555;

GpioCtrlRegs.GPAMUX2.all = 0x002A55A0;

GpioCtrlRegs.GPBMUX1.all = 0x00000005;

GpioCtrlRegs.GPBMUX2.all = 0x00000000;

GpioCtrlRegs.GPCMUX1.all = 0x00000000;

GpioCtrlRegs.GPCMUX2.all = 0x00000000;

GpioCtrlRegs.GPADIR.all = 0xFFBDFFFF;

GpioCtrlRegs.GPBDIR.all = 0xFFFFFFFF;

GpioCtrlRegs.GPCDIR.all = 0xFFFFDFFF;

EDIS;

}

//===============================================================================

void update_AD7305(void)

{

unsigned int temp;

//read existing data and mask 8 LSB bits

temp=GpioDataRegs.GPCDAT.all;

temp&=0xFFFFFF00;

GpioDataRegs.GPBCLEAR.all  =0x60000000;		//select channel A

GpioDataRegs.GPCDAT.all=temp|AD7305_CH_A;	//output 8bit data

GpioDataRegs.GPBCLEAR.bit.GPIO63=1;

GpioDataRegs.GPBSET.bit.GPIO63=1;		//latch A register

GpioDataRegs.GPBSET.bit.GPIO61=1;		//select channel B

GpioDataRegs.GPCDAT.all=temp|AD7305_CH_B;	//output 8bit data

GpioDataRegs.GPBCLEAR.bit.GPIO63=1;

GpioDataRegs.GPBSET.bit.GPIO63=1;		//latch B register

GpioDataRegs.GPBSET.bit.GPIO62=1;		//select channel D

GpioDataRegs.GPCDAT.all=temp|AD7305_CH_D;	//output 8bit data

GpioDataRegs.GPBCLEAR.bit.GPIO63=1;

GpioDataRegs.GPBSET.bit.GPIO63=1;		//latch D register

GpioDataRegs.GPBCLEAR.bit.GPIO61=1;		//select channel C

GpioDataRegs.GPCDAT.all=temp|AD7305_CH_C;	//output 8bit data

GpioDataRegs.GPBCLEAR.bit.GPIO63=1;

GpioDataRegs.GPBSET.bit.GPIO63=1;		//latch C register

//update output register simultaneously

GpioDataRegs.GPCCLEAR.bit.GPIO72=1;

GpioDataRegs.GPCSET.bit.GPIO72=1;

}

//===============================================================================

void adc_example(void)

{

//12-bit ADC core with built-in dual S/H

//Continuously converts all 16 ADC channels

//apply result from potentiometer to DAC channel - pin8, J6

for(;;)

{

//for testing conversion speed - J30, pin1

GpioDataRegs.GPCDAT.bit.GPIO75=1;

GpioDataRegs.GPCDAT.bit.GPIO75=0;

//if continuous conversion is not used, start conversion

//ADC has two independent 8-state sequencers (SEQ1 and SEQ2)

AdcRegs.ADCTRL2.bit.RST_SEQ1 = 1; // Reset SEQ1

AdcRegs.ADCTRL2.bit.SOC_SEQ1=1;	 //start of conversion on SEQ1

while (AdcRegs.ADCST.bit.INT_SEQ1== 0) {} // Wait for interrupt flag

AdcRegs.ADCST.bit.INT_SEQ1_CLR = 1;

// result of S/H conversion is placed in the ADCRESULT register

// lower 8-bit Digital result displayed using 8-bit dac

ADCINA0IA=AdcRegs.ADCRESULT0>>4;

ADCINB0IB=AdcRegs.ADCRESULT1>>4;

ADCINA1N=AdcRegs.ADCRESULT2>>4;

ADCINB1VDC=AdcRegs.ADCRESULT3>>4;

ADCINA2=AdcRegs.ADCRESULT4>>4;

ADCINB2=AdcRegs.ADCRESULT5>>4;

ADCINA3=AdcRegs.ADCRESULT6>>4;

ADCINB3=AdcRegs.ADCRESULT7>>4;

ADCINA4=AdcRegs.ADCRESULT8>>4;

ADCINB4=AdcRegs.ADCRESULT9>>4;

ADCINA5=AdcRegs.ADCRESULT10>>4;

ADCINB5=AdcRegs.ADCRESULT11>>4;

ADCINA6=AdcRegs.ADCRESULT12>>4;

ADCINB6=AdcRegs.ADCRESULT13>>4;

ADCINA7=AdcRegs.ADCRESULT14>>4;

ADC_POT=AdcRegs.ADCRESULT15>>4;

AD7305_CH_D=ADC_POT&0xff;

//display phase A on channel A and B current on channel B

AD7305_CH_A=

AD7305_CH_B=

// display speed on channel C and stator voltage Vs on channel D

AD7305_CH_C=

AD7305_CH_D=

update_AD7305();

//delay_loop_int(10000);

//scib_xmit(AD7305_CH_A);

/*// check for serial data request

if(ScibRegs.SCIFFRX.bit.RXFFST ==1)

{

// Get character

ReceivedChar = ScibRegs.SCIRXBUF.all;

scib_xmit(AD7305_CH_A);

}

*/

}

}

//===============================================================================

void init_adc(void)

{

EALLOW;

// enable ADC clocks

SysCtrlRegs.PCLKCR0.bit.ADCENCLK = 1;

EDIS;

// Call the ADC_cal function before using the ADC

ADC_cal();

// ADC Control Register 3

// Powerup the ADC band gap and reference circuitry

// Sequential sampling mode is selected

AdcRegs.ADCTRL3.all = 0x00E0;

// 2833x ADC requires a 5-ms delay after all the circuits are powered up

DELAY_US(ADC_usDELAY);

// Specific clock setting for this example:

EALLOW;

SysCtrlRegs.HISPCP.all = ADC_MODCLK;

// HSPCLK = SYSCLKOUT/ADC_MODCLK

EDIS;

// ADC External Reference Select Register is enabled

//ADC can accept external reference =2.048 V on ADCREFIN pin

AdcRegs.ADCREFSEL.bit.REF_SEL=1;

//Acquisition window size ACQ_PS is set from 0 to 15

// ACQ_PS controls the width of SOC pulse

//ADC_SHCLK; 1- fast, 15-very less noise

AdcRegs.ADCTRL1.bit.ACQ_PS = 15;

// Sequential mode: Sample rate   = 1/[(2+ACQ_PS)*ADC clock in ns]

//                                = 1/(3*40ns) =8.3MHz

// If Simultaneous mode enabled: Sample rate = 1/[(3+ACQ_PS)*ADC clock in ns]

//ADC CLOCK PRESCALER

// Core clock divider ADCCLKPS [ 0-fast, 1-very less noise]

AdcRegs.ADCTRL3.bit.ADCCLKPS = 1;

// CONTINUOUS RUN BIT disabled

// If CONT_RUN is set, conversion sequence starts all over again automatically

// If CONT_RUN is not set, the sequencer stays in the last state

AdcRegs.ADCTRL1.bit.CONT_RUN = 0;

// SIMULTANEOUS SAMPLING MODE

// one input is from ADCINA0-ADCINA7 and the other input is from ADCINB0-ADCINB7

AdcRegs.ADCTRL3.bit.SMODE_SEL = 0x1;

// CASCADED SEQUENCER MODE

AdcRegs.ADCTRL1.bit.SEQ_CASC = 0x1;

// Maximum number of conversions executed in an autoconversion session

// 8 double conversions (16 total)

AdcRegs.ADCMAXCONV.all = 0x0007;

// ADC INPUT CHANNEL SELECT SEQUENCING CONTROL REGISTERS 16-bit

// Setup conversion from ADCINA0 &amp; ADCINB0

AdcRegs.ADCCHSELSEQ1.bit.CONV00 = 0x0;

// Setup conv from ADCINA1 &amp; ADCINB1

AdcRegs.ADCCHSELSEQ1.bit.CONV01 = 0x1;

// Setup conv from ADCINA2 &amp; ADCINB2

AdcRegs.ADCCHSELSEQ1.bit.CONV02 = 0x2;

// Setup conv from ADCINA3 &amp; ADCINB3

AdcRegs.ADCCHSELSEQ1.bit.CONV03 = 0x3;

// Setup conv from ADCINA4 &amp; ADCINB4

AdcRegs.ADCCHSELSEQ2.bit.CONV04 = 0x4;

// Setup conv from ADCINA5 &amp; ADCINB5

AdcRegs.ADCCHSELSEQ2.bit.CONV05 = 0x5;

// Setup conv from ADCINA6 &amp; ADCINB6

AdcRegs.ADCCHSELSEQ2.bit.CONV06 = 0x6;

// Setup conv from ADCINA7 &amp; ADCINB7

AdcRegs.ADCCHSELSEQ2.bit.CONV07 = 0x7;

AdcRegs.ADCTRL2.bit.RST_SEQ1 = 1;  // Reset SEQ1

AdcRegs.ADCTRL2.bit.SOC_SEQ1=1;    //start of conversion on seq1

}

//===============================================================================

void init28335(void)

{

// Step 1. Initialize System Control:

// PLL, WatchDog, enable Peripheral Clocks

// This example function is found in the DSP2833x_SysCtrl.c file.

InitSysCtrl();

// Step 2. Initalize GPIO:

// This example function is found in the DSP2833x_Gpio.c file and

// illustrates how to set the GPIO to it's default state.

// InitGpio();  // Skipped for this example

// For this example use the following configuration:

Gpio_select();

// Step 3. Clear all interrupts and initialize PIE vector table:

// Disable CPU interrupts

DINT;

// Initialize PIE control registers to their default state.

// The default state is all PIE interrupts disabled and flags

// are cleared.

// This function is found in the DSP2833x_PieCtrl.c file.

InitPieCtrl();

// Disable CPU interrupts and clear all CPU interrupt flags:

IER = 0x0000;

IFR = 0x0000;

// Initialize the PIE vector table with pointers to the shell Interrupt

// Service Routines (ISR).

// This will populate the entire table, even if the interrupt

// is not used in this example.  This is useful for debug purposes.

// The shell ISR routines are found in DSP2833x_DefaultIsr.c.

// This function is found in DSP2833x_PieVect.c.

InitPieVectTable();

// Step 4. Initialize all the Device Peripherals:

// This function is found in DSP2833x_InitPeripherals.c

// InitPeripherals(); // Not required for this example

Init_QEP();

I2CA_Init();

config_MCP4728();

init_adc();

scib_fifo_init();	   // Initialize the SCI FIFO

scib_echoback_init();  // Initalize SCI for echoback

}

//===============================================================================

Uint16 update_MCP4728(void)

{

//NOTE

// Wait until the STP bit is cleared from any previous master communication.

// Clearing of this bit by the module is delayed until after the SCD bit is

// set. If this bit is not checked prior to initiating a new message, the

// I2C could get confused.

/*

if (I2caRegs.I2CMDR.bit.STP == 1)

{

return I2C_STP_NOT_READY_ERROR;

}

// Check if bus busy

if (I2caRegs.I2CSTR.bit.BB == 1)

{

return I2C_BUS_BUSY_ERROR;

}*/

//wait until bus is free - upto 6k updates per second is possible

while(I2caRegs.I2CMDR.bit.STP == 1);

while(I2caRegs.I2CSTR.bit.BB == 1);

// Setup slave address

I2caRegs.I2CSAR = 0x60;

// Setup number of bytes to send

I2caRegs.I2CCNT = 8;

// Setup data to send

//  I2caRegs.I2CDXR = 0x50;

I2caRegs.I2CDXR = (MCP4728_CH_A>>8)&0x0F;

I2caRegs.I2CDXR = MCP4728_CH_A&0xff;

I2caRegs.I2CDXR = (MCP4728_CH_B>>8)&0x0F;

I2caRegs.I2CDXR = MCP4728_CH_B&0xff;

I2caRegs.I2CDXR = (MCP4728_CH_C>>8)&0x0F;

I2caRegs.I2CDXR = MCP4728_CH_C&0xff;

I2caRegs.I2CDXR = ((MCP4728_CH_D>>8)&0x0F)|0x80;

I2caRegs.I2CDXR = MCP4728_CH_D&0xff;

// Send start as master transmitter

I2caRegs.I2CMDR.all = 0x6E20;

return I2C_SUCCESS;

}

//===============================================================================

void parallel_dac_exapmle(void)

{

for(;;)

{

update_AD7305();

//update ramp data

AD7305_CH_A =

//AD7305_CH_A&=0xff;

i++;

if(i>1024)

{

i=0;

}

AD7305_CH_B=

AD7305_CH_C=

AD7305_CH_D=

//apply delay

delay_loop_int(2);

}

}

Uint16 config_MCP4728(void)

{

//update some of the EEPRO locations to set gain, reference voltage etc

//wait until bus is free

while(I2caRegs.I2CMDR.bit.STP == 1);

while(I2caRegs.I2CSTR.bit.BB == 1);

// Setup slave address

I2caRegs.I2CSAR = 0x60;

// Setup number of bytes to send

I2caRegs.I2CCNT = 9;

// Setup data to send

I2caRegs.I2CDXR = 0x50;

//For setting internal reference with gain 1.

I2caRegs.I2CDXR = 0x80;

I2caRegs.I2CDXR = 0;

I2caRegs.I2CDXR = 0x80;

I2caRegs.I2CDXR = 0;

I2caRegs.I2CDXR = 0x80;

I2caRegs.I2CDXR = 0;

I2caRegs.I2CDXR = 0x80;

I2caRegs.I2CDXR = 0;

// Send start as master transmitter

I2caRegs.I2CMDR.all = 0x6E20;

return I2C_SUCCESS;

}

//===============================================================================

void I2CA_Init(void)

{

EALLOW;

/* Enable internal pull-up for the selected pins */

// Pull-ups can be enabled or disabled disabled by the user.

// This will enable the pullups for the specified pins.

// Comment out other unwanted lines.

GpioCtrlRegs.GPBPUD.bit.GPIO32 = 0;    // Enable pull-up for GPIO32 (SDAA)

GpioCtrlRegs.GPBPUD.bit.GPIO33 = 0;   // Enable pull-up for GPIO33 (SCLA)

GpioDataRegs.GPCCLEAR.bit.GPIO76=1;

/* Set qualification for selected pins to asynch only */

// This will select asynch (no qualification) for the selected pins.

// Comment out other unwanted lines.

GpioCtrlRegs.GPBQSEL1.bit.GPIO32 = 3;  // Asynch input GPIO32 (SDAA)

GpioCtrlRegs.GPBQSEL1.bit.GPIO33 = 3;  // Asynch input GPIO33 (SCLA)

/* Configure SCI pins using GPIO regs*/

// This specifies which of the possible GPIO pins will be I2C functional pins.

// Comment out other unwanted lines.

GpioCtrlRegs.GPBMUX1.bit.GPIO32 = 1;   // Configure GPIO32 for SDAA operation

GpioCtrlRegs.GPBMUX1.bit.GPIO33 = 1;   // Configure GPIO33 for SCLA operation

EDIS;

// Initialise I2C

I2caRegs.I2CSAR = 0x60;		// Slave address - EEPROM control code

I2caRegs.I2CPSC.all = 14; // Prescaler-need 7-12 MHz on module clk(150/15=10MHz)

I2caRegs.I2CCLKL = 4;			// NOTE: must be non zero

I2caRegs.I2CCLKH = 4;			// NOTE: must be non zero

//I2caRegs.I2CIER.all = 0x24;		// Enable SCD & ARDY interrupts

I2caRegs.I2CMDR.all = 0x0020;	// Take I2C out of reset

// Stop I2C when suspended

I2caRegs.I2CFFTX.all = 0x6000;	// Enable FIFO mode and TXFIFO

I2caRegs.I2CFFRX.all = 0x2040;	// Enable RXFIFO, clear RXFFINT,

return;

}

//===============================================================================

void Init_QEP(void)

{

//Encoder 1

EQep1Regs.QDECCTL.bit.QSRC=00;		// QEP quadrature count mode

EQep1Regs.QEPCTL.bit.FREE_SOFT=2;

EQep1Regs.QEPCTL.bit.PCRM=00;// PCRM=00 mode - QPOSCNT reset on index event

EQep1Regs.QEPCTL.bit.UTE=1; 		// Unit Timeout Enable

EQep1Regs.QEPCTL.bit.QCLM=1; 		// Latch on unit time out

EQep1Regs.QPOSMAX=0xffffffff;

EQep1Regs.QEPCTL.bit.QPEN=1; 		// QEP enable

EQep1Regs.QCAPCTL.bit.UPPS=5;   	// 1/32 for unit position

EQep1Regs.QCAPCTL.bit.CCPS=7;		// 1/128 for CAP clock

EQep1Regs.QCAPCTL.bit.CEN=1; 		// QEP Capture Enable

//Encoder 2

EQep2Regs.QDECCTL.bit.QSRC=00;		// QEP quadrature count mode

EQep2Regs.QEPCTL.bit.FREE_SOFT=2;

EQep2Regs.QEPCTL.bit.PCRM=00; // PCRM=00 mode - QPOSCNT reset on index event

EQep2Regs.QEPCTL.bit.UTE=1; 		// Unit Timeout Enable

EQep2Regs.QEPCTL.bit.QCLM=1; 		// Latch on unit time out

EQep2Regs.QPOSMAX=0xffffffff;

EQep2Regs.QEPCTL.bit.QPEN=1; 		// QEP enable

EQep2Regs.QCAPCTL.bit.UPPS=5;   	// 1/32 for unit position

EQep2Regs.QCAPCTL.bit.CCPS=7;		// 1/128 for CAP clock

EQep2Regs.QCAPCTL.bit.CEN=1; 		// QEP Capture Enable

}

//===============================================================================

void pwm_example(void)

{

// Enhanced PWM module

// epwm1a, epwm1b, epwm2a, epwm2b… to epwm6a,epwm6b

//suitable for the control of two three-phase induction motors

//pwm frequency approximately equal to 4.577 KHz

init_pwm();

for(;;);	//pwm updated during interrupt service subroutine

}

//===============================================================================

void init_pwm(void)

{

// CONTROL OF 3-PHASE INVERTER -COMMONLY USED IN MOTOR CONTROL

// Supports 16-bit Time-Base Counter with period & frequency control

// ePWM MODULE 1 CONFIG

// System Clock frequency = 150 MHz

// Period = 1600 TBCLK counts

EPwm1Regs.TBPRD = 0x1E8;

// Time-base clock is a pre-scaled version of the system clock

// TB clock is used by all sub-modules within the ePWM.

// Determines the rate at which time-base counter increments or decrements

EPwm1Regs.TBCTL.bit.HSPCLKDIV = TB_DIV4; // Clock ratio to SYSCLKOUT

EPwm1Regs.TBCTL.bit.CLKDIV = TB_DIV4;

EPwm1Regs.TBPHS.half.TBPHS = 0; // Set Phase register to zero

// Symmetrical mode for Time Base counter

EPwm1Regs.TBCTL.bit.CTRMODE = TB_COUNT_UPDOWN;

// PHASE REGISTER ENABLE PHSEN

// To set up a Master TB module

// To ignore the synchronization input pulse

EPwm1Regs.TBCTL.bit.PHSEN = TB_DISABLE;

// Time-base period register (TBPRD) has a shadow register

// Shadow register provides a temporary holding location for the active register

EPwm1Regs.TBCTL.bit.PRDLD = TB_SHADOW;

// Synchronization Output Select SYNCOSEL

// Time-base counter equal to zero (TB_CTR = 0x0000)

EPwm1Regs.TBCTL.bit.SYNCOSEL = TB_CTR_ZERO; // Sync down-stream module

EPwm1Regs.CMPCTL.bit.SHDWAMODE = CC_SHADOW;

EPwm1Regs.CMPCTL.bit.SHDWBMODE = CC_SHADOW;

EPwm1Regs.CMPCTL.bit.LOADAMODE = CC_CTR_ZERO; // load on CTR=Zero

EPwm1Regs.CMPCTL.bit.LOADBMODE = CC_CTR_ZERO; // load on CTR=Zero

// force ePWMA output high (counter incrementing)

EPwm1Regs.AQCTLA.bit.CAU = AQ_SET;

// force ePWMB output low (counter decrementing)

EPwm1Regs.AQCTLA.bit.CAD = AQ_CLEAR;

// Dead-band generation with independent rising and falling edge delay control

EPwm1Regs.DBCTL.bit.OUT_MODE = DB_FULL_ENABLE; // enable Dead-band module

EPwm1Regs.DBCTL.bit.POLSEL = DB_ACTV_HIC; // Active Hi complementary

EPwm1Regs.DBFED = 50; // FED = 50 TBCLKs

EPwm1Regs.DBRED = 50; // RED = 50 TBCLKs

// ePWM MODULE 2 CONFIGURE

EPwm2Regs.TBPRD = 0xEA6; // Period = 1600 TBCLK counts

EPwm2Regs.TBCTL.bit.HSPCLKDIV = TB_DIV4;       // Clock ratio to SYSCLKOUT

EPwm2Regs.TBCTL.bit.CLKDIV = TB_DIV4;

EPwm2Regs.TBPHS.half.TBPHS = 0; // Set Phase register to zero

EPwm2Regs.TBCTL.bit.CTRMODE = TB_COUNT_UPDOWN; // Symmetrical mode

EPwm2Regs.TBCTL.bit.PHSEN = TB_ENABLE; // Slave module

EPwm2Regs.TBCTL.bit.PRDLD = TB_SHADOW;

// Synchronization Output Select SYNCOSEL

// sync flow-through

EPwm2Regs.TBCTL.bit.SYNCOSEL = TB_SYNC_IN;

EPwm2Regs.CMPCTL.bit.SHDWAMODE = CC_SHADOW;

EPwm2Regs.CMPCTL.bit.SHDWBMODE = CC_SHADOW;

EPwm2Regs.CMPCTL.bit.LOADAMODE = CC_CTR_ZERO; // load on CTR=Zero

EPwm2Regs.CMPCTL.bit.LOADBMODE = CC_CTR_ZERO; // load on CTR=Zero

EPwm2Regs.AQCTLA.bit.CAU = AQ_SET; // set actions for EPWM2A

EPwm2Regs.AQCTLA.bit.CAD = AQ_CLEAR;

EPwm2Regs.DBCTL.bit.OUT_MODE = DB_FULL_ENABLE; // enable Dead-band module

EPwm2Regs.DBCTL.bit.POLSEL = DB_ACTV_HIC; // Active Hi complementary

EPwm2Regs.DBFED = 50; // FED = 50 TBCLKs

EPwm2Regs.DBRED = 50; // RED = 50 TBCLKs

// EPWM Module 3 config

EPwm3Regs.TBPRD = 0xEA6; // Period = 1600 TBCLK counts

EPwm3Regs.TBCTL.bit.HSPCLKDIV = TB_DIV4;       // Clock ratio to SYSCLKOUT

EPwm3Regs.TBCTL.bit.CLKDIV = TB_DIV4;

EPwm3Regs.TBPHS.half.TBPHS = 0; // Set Phase register to zero

EPwm3Regs.TBCTL.bit.CTRMODE = TB_COUNT_UPDOWN; // Symmetrical mode

EPwm3Regs.TBCTL.bit.PHSEN = TB_ENABLE; // Slave module

EPwm3Regs.TBCTL.bit.PRDLD = TB_SHADOW;

EPwm3Regs.TBCTL.bit.SYNCOSEL = TB_SYNC_IN; // sync flow-through

EPwm3Regs.CMPCTL.bit.SHDWAMODE = CC_SHADOW;

EPwm3Regs.CMPCTL.bit.SHDWBMODE = CC_SHADOW;

EPwm3Regs.CMPCTL.bit.LOADAMODE = CC_CTR_ZERO; // load on CTR=Zero

EPwm3Regs.CMPCTL.bit.LOADBMODE = CC_CTR_ZERO; // load on CTR=Zero

EPwm3Regs.AQCTLA.bit.CAU = AQ_SET; // set actions for EPWM3A

EPwm3Regs.AQCTLA.bit.CAD = AQ_CLEAR;

EPwm3Regs.DBCTL.bit.OUT_MODE = DB_FULL_ENABLE; // enable Dead-band module

EPwm3Regs.DBCTL.bit.POLSEL = DB_ACTV_HIC; // Active Hi complementary

EPwm3Regs.DBFED = 50; // FED = 50 TBCLKs

EPwm3Regs.DBRED = 50; // RED = 50 TBCLKs

// EPWM Module 4 config

EPwm4Regs.TBPRD = 0x1E8; // Period = 1600 TBCLK counts

EPwm4Regs.TBCTL.bit.HSPCLKDIV = TB_DIV4;       // Clock ratio to SYSCLKOUT

EPwm4Regs.TBCTL.bit.CLKDIV = TB_DIV4;

EPwm4Regs.TBPHS.half.TBPHS = 0; // Set Phase register to zero

EPwm4Regs.TBCTL.bit.CTRMODE = TB_COUNT_UPDOWN; // Symmetrical mode

EPwm4Regs.TBCTL.bit.PHSEN = TB_ENABLE; // Slave module

EPwm4Regs.TBCTL.bit.PRDLD = TB_SHADOW;

EPwm4Regs.TBCTL.bit.SYNCOSEL = TB_SYNC_IN; // sync flow-through

EPwm4Regs.CMPCTL.bit.SHDWAMODE = CC_SHADOW;

EPwm4Regs.CMPCTL.bit.SHDWBMODE = CC_SHADOW;

EPwm4Regs.CMPCTL.bit.LOADAMODE = CC_CTR_ZERO; // load on CTR=Zero

EPwm4Regs.CMPCTL.bit.LOADBMODE = CC_CTR_ZERO; // load on CTR=Zero

EPwm4Regs.AQCTLA.bit.CAU = AQ_SET; // set actions for EPWM4A

EPwm4Regs.AQCTLA.bit.CAD = AQ_CLEAR;

EPwm4Regs.DBCTL.bit.OUT_MODE = DB_FULL_ENABLE; // enable Dead-band module

EPwm4Regs.DBCTL.bit.POLSEL = DB_ACTV_HIC; // Active Hi complementary

EPwm4Regs.DBFED = 50; // FED = 50 TBCLKs

EPwm4Regs.DBRED = 50; // RED = 50 TBCLKs

// EPWM Module 5 config

EPwm5Regs.TBPRD = 0x1E8; // Period = 1600 TBCLK counts

EPwm5Regs.TBCTL.bit.HSPCLKDIV = TB_DIV4;       // Clock ratio to SYSCLKOUT

EPwm5Regs.TBCTL.bit.CLKDIV = TB_DIV4;

EPwm5Regs.TBPHS.half.TBPHS = 0; // Set Phase register to zero

EPwm5Regs.TBCTL.bit.CTRMODE = TB_COUNT_UPDOWN; // Symmetrical mode

EPwm5Regs.TBCTL.bit.PHSEN = TB_ENABLE; // Slave module

EPwm5Regs.TBCTL.bit.PRDLD = TB_SHADOW;

EPwm5Regs.TBCTL.bit.SYNCOSEL = TB_SYNC_IN; // sync flow-through

EPwm5Regs.CMPCTL.bit.SHDWAMODE = CC_SHADOW;

EPwm5Regs.CMPCTL.bit.SHDWBMODE = CC_SHADOW;

EPwm5Regs.CMPCTL.bit.LOADAMODE = CC_CTR_ZERO; // load on CTR=Zero

EPwm5Regs.CMPCTL.bit.LOADBMODE = CC_CTR_ZERO; // load on CTR=Zero

EPwm5Regs.AQCTLA.bit.CAU = AQ_SET; // set actions for EPWM4A

EPwm5Regs.AQCTLA.bit.CAD = AQ_CLEAR;

EPwm5Regs.DBCTL.bit.OUT_MODE = DB_FULL_ENABLE; // enable Dead-band module

EPwm5Regs.DBCTL.bit.POLSEL = DB_ACTV_HIC; // Active Hi complementary

EPwm5Regs.DBFED = 50; // FED = 50 TBCLKs

EPwm5Regs.DBRED = 50; // RED = 50 TBCLKs

// EPWM Module 6 config

EPwm6Regs.TBPRD = 0x1E8; // Period = 1600 TBCLK counts

EPwm6Regs.TBCTL.bit.HSPCLKDIV = TB_DIV4;       // Clock ratio to SYSCLKOUT

EPwm6Regs.TBCTL.bit.CLKDIV = TB_DIV4;

EPwm6Regs.TBPHS.half.TBPHS = 0; // Set Phase register to zero

EPwm6Regs.TBCTL.bit.CTRMODE = TB_COUNT_UPDOWN; // Symmetrical mode

EPwm6Regs.TBCTL.bit.PHSEN = TB_ENABLE; // Slave module

EPwm6Regs.TBCTL.bit.PRDLD = TB_SHADOW;

EPwm6Regs.TBCTL.bit.SYNCOSEL = TB_SYNC_IN; // sync flow-through

EPwm6Regs.CMPCTL.bit.SHDWAMODE = CC_SHADOW;

EPwm6Regs.CMPCTL.bit.SHDWBMODE = CC_SHADOW;

EPwm6Regs.CMPCTL.bit.LOADAMODE = CC_CTR_ZERO; // load on CTR=Zero

EPwm6Regs.CMPCTL.bit.LOADBMODE = CC_CTR_ZERO; // load on CTR=Zero

EPwm6Regs.AQCTLA.bit.CAU = AQ_SET; // set actions for EPWM4A

EPwm6Regs.AQCTLA.bit.CAD = AQ_CLEAR;

EPwm6Regs.DBCTL.bit.OUT_MODE = DB_FULL_ENABLE; // enable Dead-band module

EPwm6Regs.DBCTL.bit.POLSEL = DB_ACTV_HIC; // Active Hi complementary

EPwm6Regs.DBFED = 50; // FED = 50 TBCLKs

EPwm6Regs.DBRED = 50; // RED = 50 TBCLKs

// initializing the CMPA register

EPwm1Regs.CMPA.half.CMPA = 10; // adjust duty for output EPWM1A

EPwm2Regs.CMPA.half.CMPA = 10; // adjust duty for output EPWM2A

EPwm3Regs.CMPA.half.CMPA = 10; // adjust duty for output EPWM3A

//EPwm4Regs.CMPA.half.CMPA = 10;

//EPwm5Regs.CMPA.half.CMPA = 10;

//EPwm6Regs.CMPA.half.CMPA = 10;

//configure interrupt from the pwm module

EPwm1Regs.ETSEL.bit.INTSEL = ET_CTR_ZERO;     // Select INT on Zero event

EPwm1Regs.ETSEL.bit.INTEN = 1;                // Enable INT

EPwm1Regs.ETPS.bit.INTPRD = ET_1ST;           // Generate INT on 1st event

// Interrupts that are used in this example are re-mapped to

// ISR functions found within this file.

EALLOW;  // This is needed to write to EALLOW protected registers

PieVectTable.EPWM1_INT = &epwm1_isr();

EDIS;    // This is needed to disable write to EALLOW protected registers

// Enable CPU INT3 which is connected to EPWM1-3 INT:

IER |= M_INT3;

// Enable EPWM INTn in the PIE: Group 3 interrupt 1-3

PieCtrlRegs.PIEIER3.bit.INTx1 = 1;

// PieCtrlRegs.PIEIER3.bit.INTx2 = 1;

// PieCtrlRegs.PIEIER3.bit.INTx3 = 1;

// Enable global Interrupts and higher priority real-time debug events:

EINT;   // Enable Global interrupt INTM

ERTM;   // Enable Global realtime interrupt DBGM

}

//===============================================================================

interrupt void epwm1_isr(void)

{

//interrupt generated at the zero event of the pwm counter

// Clear INT flag for this timer

EPwm1Regs.ETCLR.bit.INT = 1;

//Position & Speed calculation from rotary encoder

Calc_SPEED();

// To generate reference speed

ramp_final=speed_ref;

speed_reference();

//acquire phase A and B currents and phase voltages

adc_example();

//do clarke transformation

clarke();

// actual stator flux calculation

actual_flux();

// Electromagnetic torque produced is calculated

motor_torque();

// Sector segregation

sector();

//pi controller for speed

pi_controller();

// Three level hysteresis controller for Torque error

T_hysteresis();

// Two level hysteresis controller for flux error

f_hysteresis();

// Switching Table for inverter voltage vectors

lookup_table();

// Acknowledge this interrupt to receive more interrupts from group 3

PieCtrlRegs.PIEACK.all = PIEACK_GROUP3;

EPwm1Regs.CMPA.half.CMPA =

EPwm2Regs.CMPA.half.CMPA =

EPwm3Regs.CMPA.half.CMPA =

EPwm4Regs.CMPA.half.CMPA =

EPwm5Regs.CMPA.half.CMPA =

EPwm6Regs.CMPA.half.CMPA =

}

//===============================================================================

void clarke(void)

{

// abc to dq transformation

ids=ias-(0.5*ibs)-(0.5*ics);

iqs=0.5*sqrt(3)*(ibs-ics);

Vds=Vas-(0.5*Vbs)-(0.5*Vcs);

Vqs=0.5*sqrt(3)*(Vbs-Vcs);

}

//===========================================================================

void inv_clarke(void)

{

//dc to abc transformation

ias=(2/3)*ids;

ibs=(-1/3)*ids+(1/sqrt(3))*iqs;

ics=(-1/3)*ids-(1/sqrt(3))*iqs;

}

//===========================================================================

void actual_flux(void)

{

// integrating (Vs-is*Rs) to obtain stator flux

float i,j,low,up,d_flux,q_flux;

float d_sum=0;

float q_sum=0;

up=0.632; // upper limit of integration

if(low>up)

{

i=low;

low=up;

up=i;

}

for(i=low;i<up;i=i+(up-low)/N)

{

d_flux=Vds-ids*Rs;

d_sum=d_sum+d_flux*(up-low)/N;

}

D_ds=d_sum; // Direct component of stator flux

for(j=low;j<up;i=j+(up-low)/N)

{

q_flux=Vqs-iqs*Rs;

q_sum=q_sum+q_flux*(up-low)/N;

}

D_qs=q_sum; // Quadrature component of stator flux

square=pow(D_ds,2)+pow(D_qs,2);

D_act=sqrt(square); // Actual stator flux

theta=atan2(D_qs,D_ds);

}

//===========================================================================

void motor_torque(void)

{

// Tm= (2/3)*(p/2)*(iqs*D_ds-ids*D_qs) but D_qs=0 for stator flux orientation

Tm= (2/3)*(p/2)*iqs*D_ds;

Te=Tm-T_load;

}

//===========================================================================

void sector(void)

{

theta=theta*(180/pi);

if((theta>-30)&&(theta<30))

sec = 1;

else if((theta>30)&&(theta<90))

sec = 2;

else if((theta>90)&&(theta<150))

sec = 3;

else if((theta>150)&&(theta<=180)||(theta<-150)&&(theta>=-180))

sec = 4;

else if((theta>-150)&&(theta<-90))

sec = 5;

else if((theta>-90)&&(theta<-30))

sec = 6;

}

//===========================================================================

void Calc_SPEED(void)

{

// Read from the on board rotary encoder (short J9, J10)

// Or external rotary encoder connected to J2 (open J9, J10)

// For forward or clockwise rotation, QEPA signal leads QEPB signal

// QEPA pin provides the clock input

// QEPB pin provides the direction input

}

void speed_reference(void)

{

int i=0;

for(i=0;i<ramp_final;i++)

{

ramp_count++;

}

}

//===========================================================================

void pi_controller(void)

{

err_speed=speed_ref-wr;

prop=Kp*err_speed;

i_temp=i_temp+err_speed;

// if-else statements are used to limit the accumulated integral term by setting upper and lower thresholds

if(i_temp>i_max)

i_temp=i_max;

else if(i_temp<i_min)

i_temp=i_min;

// Integral term is then calculated by multiplying the Ki constant with the accumulated integral value

integral=Ki*i_temp;

PI_out=prop+integral;

}

//===========================================================================

void T_hysteresis(void)

{

err_t=PI_out-Te;

if(err_t>0.1)

Ht=1;

else if(err_t<-0.1)

Ht=-1;

else if((Ht==1)&&(0<err_t)&&(err_t<0.1))

Ht=1;

else if((Ht==-1)&&(0<err_t)&&(err_t<0.1))

Ht=0;

else if((Ht==-1)&&(-0.1<err_t)&&(err_t<0))

Ht=-1;

else if((Ht==1)&&(-0.1<err_t)&&(err_t<0))

Ht=0;

}

//===========================================================================

void f_hysteresis(void)

{

err_flux=D_ref-D_act;

if(err_flux>0.000316)

Hd=1;

else if(err_flux<-0.000316)

Hd=-1;

}

//===========================================================================

void lookup_table(void)

{

out=0;

// Switching of 9 voltage vectors V0,V1,V2,V3,V4,V5,V6,V7

if ((Hd==1) && (Ht==1))

{

temp[0]=2;

temp[1]=3;

temp[2]=4;

temp[3]=5;

temp[4]=6;

temp[5]=1;

}

// active voltage vectors = {V2,V3,V4,V5,V6,V1};

else if ((Hd==1) && (Ht==0))

{

temp[0]=7;

temp[1]=0;

temp[2]=7;

temp[3]=0;

temp[4]=7;

temp[5]=0;

}

// zero voltage vectors = {V7,V0,V7,V0,V7,V0};

else if ((Hd==1) && (Ht==-1))

{

temp[0]=6;

temp[1]=1;

temp[2]=2;

temp[3]=3;

temp[4]=4;

temp[5]=5;

}

// active voltage vectors = {V6,V1,V2,V3,V4,V5};

else if ((Hd==-1) && (Ht==1))

{

temp[0]=3;

temp[1]=4;

temp[2]=5;

temp[3]=6;

temp[4]=1;

temp[5]=2;

}

// active voltage vectors = {V3,V4,V5,V6,V1,V2};

else if ((Hd==-1) && (Ht==0))

{

temp[0]=0;

temp[1]=7;

temp[2]=0;

temp[3]=7;

temp[4]=0;

temp[5]=7;

}

// zero voltage vectors = {V0,V7,V0,V7,V0,V7};

else if ((Hd==-1) && (Ht==-1))

{

temp[0]=5;

temp[1]=6;

temp[2]=1;

temp[3]=2;

temp[4]=3;

temp[5]=4;

}

// active voltage vectors = {V5,V6,V1,V2,V3,V4};

else

{

temp[0]=7;

temp[1]=0;

temp[2]=7;

temp[3]=0;

temp[4]=7;

temp[5]=0;

}

// zero voltage vectors = {V7,V0,V7,V0,V7,V0};

// According to the sector, the voltage vector is selected using look-up table

// The selected voltage vector is given to variable 'out'

switch (sec)

{

case 1:

{   out=temp[0];

break;

}

case 2:

{   out=temp[1];

break;

}

case 3:

{   out=temp[2];

break;

}

case 4:

{   out=temp[3];

break;

}

case 5:

{   out=temp[4];

break;

}

default:

out=temp[5];

}

// 'out' is a variable storing each element of array 'temp'.

switch(out)

{

case 1:

{   // pulses for phases A,B,C = {1,0,0};

pulse[0]=1;

pulse[1]=0;

pulse[2]=0;

break;

}

case 2:

{   // pulses for phases A,B,C = {1,1,0};

pulse[0]=1;

pulse[1]=1;

pulse[2]=0;

break;

}

case 3:

{   // pulses for phases A,B,C = {0,1,0};

pulse[0]=0;

pulse[1]=1;

pulse[2]=0;

break;

}

case 4:

{   // pulses for phases A,B,C = {0,1,1};

pulse[0]=0;

pulse[1]=1;

pulse[2]=1;

break;

}

case 5:

{   // pulses for phases A,B,C = {0,0,1};

pulse[0]=0;

pulse[1]=0;

pulse[2]=1;

break;

}

case 6:

{    // pulses for phases A,B,C = {1,0,1};

pulse[0]=1;

pulse[1]=0;

pulse[2]=1;

break;

}

case 7:

{    // pulses for all the phases = {1,1,1};

pulse[0]=1;

pulse[1]=1;

pulse[2]=1;

break;

}

default:

{    // no pulse given

pulse[0]=0;

pulse[1]=0;

pulse[2]=0;

}

}

// Switching pulses for phase-A

Sa1=pulse[0];

Sa2=1-Sa1;

// Switching pulses for phase-B

Sb1=pulse[1];

Sb2=1-Sb1;

// Switching pulses for phase-C

Sc1=pulse[2];

Sc2=1-Sc1;

}

//===============================================================================

// Initalize the SCI FIFO

void scib_fifo_init(void)

{

ScibRegs.SCIFFTX.all=0xE040;

ScibRegs.SCIFFRX.all=0x204f;

ScibRegs.SCIFFCT.all=0x0;

}

//===============================================================================

// Transmit a character from the SCI

void scib_xmit(int a)

{

while (ScibRegs.SCIFFTX.bit.TXFFST != 0) {}

ScibRegs.SCITXBUF=a;

}

//===============================================================================

void scib_echoback_init(void)

{

//Note: Clocks were turned on to the SCIA peripheral

//in the InitSysCtrl() function

ScibRegs.SCICCR.all =0x0007;   // 1 stop bit,  No loopback

// No parity,8 char bits,

// async mode, idle-line protocol

ScibRegs.SCICTL1.all =0x0003;  // enable TX, RX, internal SCICLK,

// Disable RX ERR, SLEEP, TXWAKE

ScibRegs.SCICTL2.all =0x0003;

ScibRegs.SCICTL2.bit.TXINTENA =1;

ScibRegs.SCICTL2.bit.RXBKINTENA =1;

/*

ScibRegs.SCIHBAUD    =0x0001;  // 9600 baud @LSPCLK = 37.5MHz.

ScibRegs.SCILBAUD    =0x00E7;*/

ScibRegs.SCIHBAUD    =0;  // 38700 baud @LSPCLK = 37.5MHz.

ScibRegs.SCILBAUD    =121;

ScibRegs.SCICTL1.all =0x0023;  // Relinquish SCI from Reset

}

//===============================================================================

void test_rs232(void)

{

//add 1 to the received data and send back

for(;;)

{

// Wait forinput data

while(ScibRegs.SCIFFRX.bit.RXFFST !=1) { }// wait for XRDY =1 for empty state

// Get character

ReceivedChar = ScibRegs.SCIRXBUF.all;

//add 1 and send back

scib_xmit(ReceivedChar+1);

}

}

//===============================================================================