Below function can be used to calculate ECC for a given address (no need to left shift the address) and 64-bit data for F2837xS device.

//Calculate the ECC for an address/data pair

uint16 CalcEcc(uint32 address, uint64 data)

{

const uint32 addrSyndrome[8] =  {0x554ea, 0x0bad1, 0x2a9b5, 0x6a78d,

0x19f83, 0x07f80, 0x7ff80, 0x0007f};

const uint64 dataSyndrome[8] =  {0xb4d1b4d14b2e4b2e, 0x1557155715571557,

0xa699a699a699a699, 0x38e338e338e338e3,

0xc0fcc0fcc0fcc0fc, 0xff00ff00ff00ff00,

0xff0000ffff0000ff, 0x00ffff00ff0000ff};

const uint16 parity = 0xfc;

uint64 xorData;

uint32 xorAddr;

uint16 bit, eccBit, eccVal;

//Extract bits "20:2" of the address

address = (address >> 2) & 0x7ffff;

//Compute the ECC one bit at a time.

eccVal = 0;

for (bit = 0; bit < 8; bit++)

{

//Apply the encoding masks to the address and data

xorAddr = address & addrSyndrome[bit];

xorData = data & dataSyndrome[bit];

//Fold the masked address into a single bit for parity calculation.

//The result will be in the LSB.

xorAddr = xorAddr ^ (xorAddr >> 16);

xorAddr = xorAddr ^ (xorAddr >> 8);

xorAddr = xorAddr ^ (xorAddr >> 4);

xorAddr = xorAddr ^ (xorAddr >> 2);

xorAddr = xorAddr ^ (xorAddr >> 1);

//Fold the masked data into a single bit for parity calculation.

//The result will be in the LSB.

xorData = xorData ^ (xorData >> 32);

xorData = xorData ^ (xorData >> 16);

xorData = xorData ^ (xorData >> 8);

xorData = xorData ^ (xorData >> 4);

xorData = xorData ^ (xorData >> 2);

xorData = xorData ^ (xorData >> 1);

//Merge the address and data, extract the ECC bit, and add it in

eccBit = ((uint16)xorData ^ (uint16)xorAddr) & 0x0001;

eccVal |= eccBit << bit;

}

//Handle the bit parity. For odd parity, XOR the bit with 1

eccVal ^= parity;

return eccVal;

}