EMIF Timing Calculation:

tDn & tWn are the timing characteristics values which are taken from ADS8365 data sheet.

a) Read calculations:

- R_STROBE >= (tELQV + tSU) × fEM1CLK – 1

tELQV = td4+td6 = 0+60 or tw6=70(for 3v)

R_STROBE >= (70 ns + 15 ns) × 200 MHz – 1

R_STROBE = 17 clock cycles

- The R_HOLD field must be large enough to satisfy EMIF Data hold time, tH:

R_HOLD > = tH × fEM1CLK - 1

R_HOLD >= 0 ns × 200 MHz - 1

R_HOLD >= -1

- The R_HOLD field must also combine with the TA field to satisfy the Flash's nCE High to Output in HighImpedance time, tEHQZ:

R_HOLD + TA >= tEHQZ × fEM1CLK – 2

tEHQZ= td4+td6 = 0+60ns = 60ns

R_HOLD + TA >= tEHQZ × fEM1CLK – 2

R_HOLD + TA >= 60 ns × 200 MHz - 2

R_HOLD + TA >=  12-2 = 10-2

R_HOLD = 10-3 (lets take TA=3(margin)) =7 cycles

b) Write Timing calculations:

- W_STROBE >= tELEH × fEM1CLK – 1

tELEH = nCE Pulse Width Low

=   50ns

W_STROBE >= 70 ns × 200 MHz - 1

W_STROBE >= 13 (tw6 is considered according to the write timing which is almost equal to R_STROBE)

- The W_SETUP and W_HOLD fields should combine to satisfy the Flash's nCE Pulse Width High

constraint, tEHEL:

W_SETUP + W_HOLD > = tEHEL × fEM1CLK – 2

tEHEL = nCE Pulse Width Low = tw5 = 30ns

W_SETUP + W_HOLD > = 30 ns × 200 MHz - 2

W_SETUP + W_HOLD > = 4

- In addition, the entire Write access length must satisfy the Flash's minimum Write Cycle Time, tAVAV:

W_SETUP + W_STROBE + W_HOLD >= tAVAV × fEM1CLK – 3

tAVAV = td10+tw6+td11 = 10+50+10

W_SETUP + W_STROBE + W_HOLD >= 70 ns × 200 MHz - 3

W_SETUP + W_STROBE + W_HOLD >= 11

W_SETUP is almost equal to tD10 but the overall emif value is not greater than 11.