// The user must define CLA_C in the project linker settings if using the

// CLA C compiler

// Project Properties -> C2000 Linker -> Advanced Options -> Command File

// Preprocessing -> --define

#ifdef CLA_C

// Define a size for the CLA scratchpad area that will be used

// by the CLA compiler for local symbols and temps

// Also force references to the special symbols that mark the

// scratchpad are.

CLA_SCRATCHPAD_SIZE = 0x100;

--undef_sym=__cla_scratchpad_end

--undef_sym=__cla_scratchpad_start

#endif //CLA_C

MEMORY

{

PAGE 0 :

/* BEGIN is used for the "boot to SARAM" bootloader mode   */

BEGIN           	: origin = 0x000000, length = 0x000002

RAMM0           	: origin = 0x000122, length = 0x0002DE

RAMD0           	: origin = 0x00B000, length = 0x000800

RAMD1            : origin = 0x00B800, length = 0x000800

RAMLS0          	: origin = 0x008000, length = 0x000800

/* RAMLS4      	    : origin = 0x00A000, length = 0x000800

RAMLS5           : origin = 0x00A800, length = 0x000800*/

RAMLS4_LS5     	: origin = 0x00A000, length = 0x001000

RESET           	: origin = 0x3FFFC0, length = 0x000002

PAGE 1 :

BOOT_RSVD        : origin = 0x000002, length = 0x000120     /* Part of M0, BOOT rom will use this for stack */

RAMM1            : origin = 0x000400, length = 0x000400     /* on-chip RAM block M1 */

RAMLS1          	: origin = 0x008800, length = 0x000800

RAMLS2      		: origin = 0x009000, length = 0x000800

RAMLS3      		: origin = 0x009800, length = 0x000800

RAMGS0           : origin = 0x00C000, length = 0x001000

RAMGS1           : origin = 0x00D000, length = 0x001000

RAMGS2           : origin = 0x00E000, length = 0x001000

RAMGS3           : origin = 0x00F000, length = 0x001000

RAMGS4           : origin = 0x010000, length = 0x001000

RAMGS5           : origin = 0x011000, length = 0x001000

RAMGS6           : origin = 0x012000, length = 0x001000

RAMGS7           : origin = 0x013000, length = 0x001000

RAMGS8           : origin = 0x014000, length = 0x001000

RAMGS9           : origin = 0x015000, length = 0x001000

RAMGS10          : origin = 0x016000, length = 0x001000

RAMGS11          : origin = 0x017000, length = 0x001000

RAMGS12          : origin = 0x018000, length = 0x001000     /* Only Available on , F28377S, F28375S devices. Remove line on other devices. */

RAMGS13          : origin = 0x019000, length = 0x001000     /* Only Available on , F28377S, F28375S devices. Remove line on other devices. */

RAMGS14          : origin = 0x01A000, length = 0x001000     /* Only Available on , F28377S, F28375S devices. Remove line on other devices. */

RAMGS15          : origin = 0x01B000, length = 0x001000     /* Only Available on , F28377S, F28375S devices. Remove line on other devices. */

CANA_MSG_RAM     : origin = 0x049000, length = 0x000800

CANB_MSG_RAM     : origin = 0x04B000, length = 0x000800

CLA1_MSGRAMLOW   : origin = 0x001480, length = 0x000080

CLA1_MSGRAMHIGH  : origin = 0x001500, length = 0x000080

}

SECTIONS

{

codestart        : > BEGIN,     PAGE = 0

.text            : >> RAMD0|RAMD1|RAMLS0|RAMLS4_LS5,    PAGE = 0

.cinit           : > RAMM0,     PAGE = 0

.pinit           : > RAMM0,     PAGE = 0

.switch          : > RAMM0,     PAGE = 0

.reset           : > RESET,     PAGE = 0, TYPE = DSECT /* not used, */

.stack           : > RAMM1,     PAGE = 1

.ebss            : > RAMLS2,    PAGE = 1

.econst          : > RAMLS3,    PAGE = 1

.esysmem         : > RAMLS3,    PAGE = 1

Filter_RegsFile  : > RAMGS0,	   PAGE = 1

/* CLA specific sections */

Cla1Prog         : > RAMLS4_LS5, PAGE=0

//  CLADataLS0		: > RAMLS0, PAGE=1

CLADataLS1		: > RAMLS1, PAGE=1

Cla1ToCpuMsgRAM  : > CLA1_MSGRAMLOW,   PAGE = 1

CpuToCla1MsgRAM  : > CLA1_MSGRAMHIGH,  PAGE = 1

/* The following section definition are for SDFM examples */

Filter1_RegsFile : > RAMGS1,	PAGE = 1, fill=0x1111

Filter2_RegsFile : > RAMGS2,	PAGE = 1, fill=0x2222

Filter3_RegsFile : > RAMGS3,	PAGE = 1, fill=0x3333

Filter4_RegsFile : > RAMGS4,	PAGE = 1, fill=0x4444

#ifdef __TI_COMPILER_VERSION__

#if __TI_COMPILER_VERSION__ >= 15009000

.TI.ramfunc : {} > RAMM0,      PAGE = 0

#else

ramfuncs    : > RAMM0      PAGE = 0

#endif

#endif

#ifdef CLA_C

/* CLA C compiler sections */

//

// Must be allocated to memory the CLA has write access to

//

CLAscratch       :

{ *.obj(CLAscratch)

. += CLA_SCRATCHPAD_SIZE;

*.obj(CLAscratch_end) } >  RAMLS1,  PAGE = 1

.scratchpad      : > RAMLS1,       PAGE = 1

.bss_cla		    : > RAMLS1,       PAGE = 1

.const_cla	    : > RAMLS1,       PAGE = 1

#endif //CLA_C

}

/*

//===========================================================================

// End of file.

//===========================================================================

*/