Controller: TMS320F28388DZWT

SDRAM: IS42S16320F-7BL

Frequency: 100MHz

IC Corner: Typical

Duty Cycle:50%.

Simulation Observation:

Simulation Observation:

Ringing and non-monotonicity.

- Series Termination near Driver with 18 Ohm. Came up with resistor value using Sweep.

Waveform At Pin:

Simulation Observation:

Still Able to Observe Non- monotonicity at ViL threshold.

Waveform at Die:

Able to Observe non- monotonicity indeterminate region.

- Series Termination near Receiver with 48 Ohm. Came up with resistor value using Sweep.

At Pin:

Still Able to Observe non- monotonicity Above ViH.

At die: Signal Are Monotonous.