//#############################################################################

//

// FILE:   can_ex1_loopback.c

//

// TITLE:   CAN External Loopback Example

//

//! \addtogroup driver_example_list

//! <h1> CAN External Loopback </h1>

//!

//! This example shows the basic setup of CAN in order to transmit and receive

//! messages on the CAN bus.  The CAN peripheral is configured to transmit

//! messages with a specific CAN ID.  A message is then transmitted once per

//! second, using a simple delay loop for timing.  The message that is sent is

//! a 2 byte message that contains an incrementing pattern.

//!

//! This example sets up the CAN controller in External Loopback test mode.

//! Data transmitted is visible on the CANTXA pin and is received internally

//! back to the CAN Core.

//!

//! \b External \b Connections \n

//!  - None.

//!

//! \b Watch \b Variables \n

//!  - msgCount - A counter for the number of successful messages received

//!  - txMsgData - An array with the data being sent

//!  - rxMsgData - An array with the data that was received

//!

//

//#############################################################################

// $TI Release: F28004x Support Library v1.05.00.00 $

// $Release Date: Tue Jun 26 03:10:30 CDT 2018 $

// $Copyright:

// Copyright (C) 2018 Texas Instruments Incorporated - http://www.ti.com/

//

// Redistribution and use in source and binary forms, with or without

// modification, are permitted provided that the following conditions

// are met:

//

//   Redistributions of source code must retain the above copyright

//   notice, this list of conditions and the following disclaimer.

//

//   Redistributions in binary form must reproduce the above copyright

//   notice, this list of conditions and the following disclaimer in the

//   documentation and/or other materials provided with the

//   distribution.

//

//   Neither the name of Texas Instruments Incorporated nor the names of

//   its contributors may be used to endorse or promote products derived

//   from this software without specific prior written permission.

//

// THIS SOFTWARE IS PROVIDED BY THE COPYRIGHT HOLDERS AND CONTRIBUTORS

// "AS IS" AND ANY EXPRESS OR IMPLIED WARRANTIES, INCLUDING, BUT NOT

// LIMITED TO, THE IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR

// A PARTICULAR PURPOSE ARE DISCLAIMED. IN NO EVENT SHALL THE COPYRIGHT

// OWNER OR CONTRIBUTORS BE LIABLE FOR ANY DIRECT, INDIRECT, INCIDENTAL,

// SPECIAL, EXEMPLARY, OR CONSEQUENTIAL DAMAGES (INCLUDING, BUT NOT

// LIMITED TO, PROCUREMENT OF SUBSTITUTE GOODS OR SERVICES; LOSS OF USE,

// DATA, OR PROFITS; OR BUSINESS INTERRUPTION) HOWEVER CAUSED AND ON ANY

// THEORY OF LIABILITY, WHETHER IN CONTRACT, STRICT LIABILITY, OR TORT

// (INCLUDING NEGLIGENCE OR OTHERWISE) ARISING IN ANY WAY OUT OF THE USE

// OF THIS SOFTWARE, EVEN IF ADVISED OF THE POSSIBILITY OF SUCH DAMAGE.

// $

//#############################################################################

//

// Included Files

//

#include "driverlib.h"

#include "device.h"

//

// Defines

//

#define MSG_DATA_LENGTH    2

//

// Globals

//

volatile unsigned long msgCount = 0;

//

// Main

//

uint16_t txMsgData[2], rxMsgData[2];

void main(void)

{

//

// Initialize device clock and peripherals

//

Device_init();

//

// Initialize GPIO and configure GPIO pins for CANTX/CANRX

//

Device_initGPIO();

GPIO_setPinConfig(DEVICE_GPIO_CFG_CANRXA);

GPIO_setPinConfig(DEVICE_GPIO_CFG_CANTXA);

//

// Initialize the CAN controller

//

CAN_initModule(CANA_BASE);

//

// Set up the CAN bus bit rate to 500kHz

// Refer to the Driver Library User Guide for information on how to set

// tighter timing control. Additionally, consult the device data sheet

// for more information about the CAN module clocking.

//

CAN_setBitRate(CANA_BASE, DEVICE_SYSCLK_FREQ, 500000, 16);

//

// Initialize PIE and clear PIE registers. Disables CPU interrupts.

//

Interrupt_initModule();

//

// Initialize the PIE vector table with pointers to the shell Interrupt

// Service Routines (ISR).

//

Interrupt_initVectorTable();

//

// Enable Global Interrupt (INTM) and realtime interrupt (DBGM)

//

EINT;

ERTM;

//

// Enable CAN test mode with external loopback

//

CAN_enableTestMode(CANA_BASE, CAN_TEST_EXL);

//

// Initialize the transmit message object used for sending CAN messages.

// Message Object Parameters:

//      Message Object ID Number: 1

//      Message Identifier: 0x1234

//      Message Frame: Standard

//      Message Type: Transmit

//      Message ID Mask: 0x0

//      Message Object Flags: None

//      Message Data Length: 2 Bytes

//

CAN_setupMessageObject(CANA_BASE, 1, 0x1234, CAN_MSG_FRAME_STD,

CAN_MSG_OBJ_TYPE_TX, 0, CAN_MSG_OBJ_NO_FLAGS,

MSG_DATA_LENGTH);

//

// Initialize the receive message object used for receiving CAN messages.

// Message Object Parameters:

//      Message Object ID Number: 2

//      Message Identifier: 0x1234

//      Message Frame: Standard

//      Message Type: Receive

//      Message ID Mask: 0x0

//      Message Object Flags: None

//      Message Data Length: 2 Bytes

//

CAN_setupMessageObject(CANA_BASE, 2, 0x1234, CAN_MSG_FRAME_STD,

CAN_MSG_OBJ_TYPE_RX, 0, CAN_MSG_OBJ_NO_FLAGS,

MSG_DATA_LENGTH);

//

// Start CAN module operations

//

CAN_startModule(CANA_BASE);

//

// Setup send and receive buffers

//

txMsgData[0] = 0x01;

txMsgData[1] = 0x02;

*(uint16_t *)rxMsgData = 0;

//

// Loop Forever - Send and Receive data continuously

//

for(;;)

{

//

// Send CAN message data from message object 1

//

CAN_sendMessage(CANA_BASE, 1, MSG_DATA_LENGTH, txMsgData);

//

// Delay before receiving the data

//

DEVICE_DELAY_US(500000);

//

// Read CAN message object 2 and check for new data

//

if (CAN_readMessage(CANA_BASE, 2, rxMsgData))

{

//

// Check that received data matches sent data.

// Device will halt here during debug if data doesn't match.

//

if((txMsgData[0] != rxMsgData[0]) ||

(txMsgData[1] != rxMsgData[1]))

{

asm(" ESTOP0");

}

else

{

//

// Increment message received counter

//

msgCount++;

}

}

else

{

//

// Device will halt here during debug if no new data was received.

//

asm(" ESTOP0");

}

//

// Increment the value in the transmitted message data.

//

txMsgData[0] += 0x01;

txMsgData[1] += 0x01;

//

// Reset data if exceeds a byte

//

if(txMsgData[0] > 0xFF)

{

txMsgData[0] = 0;

}

if(txMsgData[1] > 0xFF)

{

txMsgData[1] = 0;

}

}

}

//

// End of File

//