//###########################################################################

//

// FILE:   epwm_up_aq_cpu01.c

//

// TITLE:  Action Qualifier Module - Using up count.

//

//! \addtogroup cpu01_example_list

//! <h1> EPWM Action Qualifier (epwm_up_aq)</h1>

//!

//! This example configures ePWM1, ePWM2, ePWM3 to produce an

//! waveform with independent modulation on EPWMxA and

//! EPWMxB.

//!

//! The compare values CMPA and CMPB are modified within the ePWM's ISR.

//!

//! The TB counter is in up count mode for this example.

//!

//! View the EPWM1A/B(PA0_GPIO0 & PA1_GPIO1), EPWM2A/B(PA2_GPIO2 & PA3_GPIO3)

//! and EPWM3A/B(PA4_GPIO4 & PA5_GPIO5) waveforms via an oscilloscope.

//!

//

//###########################################################################

// $TI Release: F2837xS Support Library v210 $

// $Release Date: Tue Nov  1 15:35:23 CDT 2016 $

// $Copyright: Copyright (C) 2014-2016 Texas Instruments Incorporated -

//             http://www.ti.com/ ALL RIGHTS RESERVED $

//###########################################################################

//

// Included Files

//

#include "F28x_Project.h"

#define EPWM2_TIMER_TBPRD  3999  // Period register

#define EPWM_CMP_UP           1

#define EPWM_CMP_DOWN         0

void InitEPwm2Example(void);

__interrupt void epwm2_isr(void);

void main(void)

{

InitSysCtrl();

CpuSysRegs.PCLKCR2.bit.EPWM2=1;

InitEPwm2Gpio();

DINT;

InitPieCtrl();

IER = 0x0000;

IFR = 0x0000;

InitPieVectTable();

EALLOW;

PieVectTable.EPWM2_INT = &epwm2_isr;

EDIS;

EALLOW;

CpuSysRegs.PCLKCR0.bit.TBCLKSYNC = 0;

EDIS;

InitEPwm2Example();

EALLOW;

CpuSysRegs.PCLKCR0.bit.TBCLKSYNC = 1;

EDIS;

IER |= M_INT3;

PieCtrlRegs.PIEIER3.bit.INTx1 = 1;

PieCtrlRegs.PIEIER3.bit.INTx2 = 1;

PieCtrlRegs.PIEIER3.bit.INTx3 = 1;

EINT;

ERTM;

for(;;)

{

asm ("  NOP");

}

}

__interrupt void epwm2_isr(void)

{

EPwm2Regs.ETCLR.bit.INT = 1;

PieCtrlRegs.PIEACK.all = PIEACK_GROUP3;

}

void InitEPwm2Example()

{

EPwm2Regs.TBCTL.bit.CTRMODE = TB_COUNT_UP; // Count up

EPwm2Regs.TBPRD = EPWM2_TIMER_TBPRD;       // Set timer period

EPwm2Regs.TBCTL.bit.PHSEN = TB_DISABLE;    // Disable phase loading

EPwm2Regs.TBPHS.bit.TBPHS = 0x0000;        // Phase is 0

EPwm2Regs.TBCTR = 0x0000;                  // Clear counter

EPwm2Regs.TBCTL.bit.HSPCLKDIV = TB_DIV1;   // Clock ratio to SYSCLKOUT

EPwm2Regs.TBCTL.bit.CLKDIV = 0x000;

EPwm2Regs.CMPCTL.bit.SHDWAMODE = CC_SHADOW;

EPwm2Regs.CMPCTL.bit.SHDWBMODE = CC_SHADOW;

EPwm2Regs.CMPCTL.bit.LOADAMODE = CC_CTR_ZERO;

EPwm2Regs.CMPCTL.bit.LOADBMODE = CC_CTR_ZERO;

EPwm2Regs.CMPA.bit.CMPA = 2000;//EPWM2_MIN_CMPA;      // Set compare A value

EPwm2Regs.AQCTLA.bit.PRD = AQ_SET;            // Clear PWM2A on Period

EPwm2Regs.AQCTLA.bit.CAU = AQ_CLEAR;              // Set PWM2A on event A,

// up count

//EPwm2Regs.AQCTLB.bit.PRD = AQ_CLEAR;            // Clear PWM2B on Period

//EPwm2Regs.AQCTLB.bit.CBU = AQ_SET;              // Set PWM2B on event B,

// up count

//

// Interrupt where we will change the Compare Values

//

EPwm2Regs.ETSEL.bit.INTSEL = ET_CTR_ZERO;       // Select INT on Zero event

EPwm2Regs.ETSEL.bit.INTEN = 1;                  // Enable INT

EPwm2Regs.ETPS.bit.INTPRD = ET_3RD;//ET_1ST;             // Generate INT on 3rd event

}