/*

* Inner_current_loop.c

*

*  Created on: 14-May-2022

*      Author: HE_MA

*/

//###########################################################################

//PEAK CURRENT MODE CONTROL

//###########################################################################

#include "F28x_Project.h"

/////////////////

// Defines

///////////////////////

//definitions for selecting DACH reference

#define REFERENCE_VDDA     0

#define REFERENCE_VDAC     1

//definitions for COMPH input selection

#define NEGIN_DAC          0

#define NEGIN_PIN          1

//definitions for CTRIPH/CTRIPOUTH output selection

#define CTRIP_ASYNCH       0

#define CTRIP_SYNCH        1

#define CTRIP_FILTER       2

#define CTRIP_LATCH        3

////////////////////

//definitions for selecting output pin

#define GPIO_CTRIP_PIN_NUM   10 //EPWM6A is mux'd with GPIO10

#define GPIO_CTRIP_PER_NUM   1 //EPWM6A is peripheral option 1 for GPIO10

/////////////////////////////////////////////////

////////////////////

//

// Function Prototypes

//

void InitCMPSS(void);

void InitEPWM(void);

////////////////////////////////////

// Main

#define EPWM6_TIMER_TBPRD  4999

#define EPWM6_TIMER_CMPA  1999

#define EPWM6_BLANK_WND_OFFSET  4949

#define EPWM6_BLANK_WND  100

#define CMPSS1_RAMPMAXREFS  40910

#define CMPSS1_RAMPDECVALS  9

//

////////////////////////////////////

void main(void)

{

//////////////////

InitSysCtrl();

///////////////////////

InitEPwm6Gpio();

///////////////////

DINT;

////////////////

InitPieCtrl();

///////////////////

IER = 0x0000;

IFR = 0x0000;

///////////////////

//////////////////////////////////

InitCMPSS();

/////////////////////////////////////

EALLOW;

CpuSysRegs.PCLKCR0.bit.TBCLKSYNC = 0

EDIS;

/////////////////////////////////

InitEPWM();

///////////////////////////////////

EALLOW;

CpuSysRegs.PCLKCR0.bit.TBCLKSYNC = 1

EDIS;

//////////////////////////////////////

/////////////////////////////////// Enable PWM

GPIO_SetupPinMux(GPIO_CTRIP_PIN_NUM, GPIO_MUX_CPU1, GPIO_CTRIP_PER_NUM);

///////////////////////////////////////////////////////////////

while(1)

{

}

////////////////

}

/////////////////////////////////

// InitCMPSS - Initialize CMPSS1 and configure settings

/////////////////////////////////

void InitCMPSS(void)

{

EALLOW;

//

//Enable CMPSS

//

Cmpss1Regs.COMPCTL.bit.COMPDACE = 1;

//

//NEG signal comes from DAC

//

Cmpss1Regs.COMPCTL.bit.COMPHSOURCE = NEGIN_DAC;

//

//Use VDDA as the reference for DAC

//

Cmpss1Regs.COMPDACCTL.bit.SELREF = REFERENCE_VDDA;

//

///////////////

//Select synchronous or asynchronous comparator triggering

Cmpss1Regs.COMPCTL.bit.ASYNCHEN = 1;

//Invert comparator output?

Cmpss1Regs.COMPCTL.bit.COMPHINV = 0;

////////////////

//Configure Digital Filter

//Maximum SAMPWIN value provides largest number of samples (5-bits

Cmpss1Regs.CTRIPHFILCTL.bit.SAMPWIN = 0x1F;/

//Maximum THRESH value requires static value for entire window }

//(5-bits THRESH should be GREATER than half of SAMPWIN

Cmpss1Regs.CTRIPHFILCTL.bit.THRESH = 0x1F;

//Maximum CLKPRESCALE value provides the most time between samples (12-bits

Cmpss1Regs.CTRIPHFILCLKCTL.bit.CLKPRESCALE = 0x3FF;///

//Reset filter logic & start filtering

Cmpss1Regs.CTRIPHFILCTL.bit.FILINIT = 1;

//Clear latch////

Cmpss1Regs.COMPSTSCLR.bit.HLATCHCLR = 1;//////

////////////////////////////////////////////////////////////////////////////

// Configure CTRIPOUT path

// Asynch output feeds CTRIPH

//

Cmpss1Regs.COMPCTL.bit.CTRIPHSEL = CTRIP_FILTER;/////////////////CTRIP_ASYNCH;

//

//sync clear enable

////High comparator latch EPWMSYNCPER clear. Enable EPWMSYNCPER reset of high comparator digital filter output latch COMPSTS[COMPHLATCH].

Cmpss1Regs.COMPSTSCLR.bit.HSYNCCLREN = 1;///////

////////////////////

///Configure DAC and slope compensation

// Emulation mode behavior, stop ramp immediately, after current ramp, or free-run?

Cmpss1Regs.COMPDACCTL.bit.FREESOFT = 2;//////////Ramp generator runs freely

//////////////

//DAC source select. Determines whether DACHVALA is updated from DACHVALS or from the ramp generator

//

Cmpss1Regs.COMPDACCTL.bit.DACSOURCE = 1;       // 0 DAC updated from DACHVALS, 1 DAC updated from the ramp generator

//

//Ramp generator source select. Determines which EPWMSYNCPER signal is used within the CMPSS module

//

Cmpss1Regs.COMPDACCTL.bit.RAMPSOURCE = 5;       // 5 EPWM6SYNCPER,

/////////////////

// Load ramp from shadow or immediate

Cmpss1Regs.COMPDACCTL.bit.RAMPLOADSEL = 1;/////////////RAMPSTS is loaded from RAMPMAXREFS

///////////////

EDIS;

/////////////////////////////////

/////////////CMPSS Ramp Decrement Value Shadow Register

Cmpss3Regs.RAMPDECVALS = CMPSS1_RAMPDECVALS;

//////////////////////////////////Ramp maximum reference shadow

Cmpss3Regs.RAMPMAXREFS = CMPSS1_RAMPMAXREFS;

}

//

// InitEPWM - Initialize EPWM1 module settings

//

void InitEPWM(void)

{

EALLOW;

//CpuSysRegs.PCLKCR0.bit.TBCLKSYNC = 0;

//Configure EPWM to run at SYSCLK

EPwm6Regs.TBCTL.bit.PRDLD = 0;//////////////

EPwm6Regs.TBCTL.bit.CLKDIV = 0;// Don't divide the PWM clock from the system clock

EPwm6Regs.TBCTL.bit.HSPCLKDIV = 0;

//Initialize dummy values for EPWM CTR/PRD

EPwm6Regs.TBCTR = 0x0000;

EPwm6Regs.TBPRD = EPWM6_TIMER_TBPRD;         // 40kHz PWM

/////////EPwm6Regs.CMPA.bit.CMPA = EPWM6_TIMER_CMPA

// TB_DISABLE:Phase loading disabled

EPwm6Regs.TBCTL.bit.PHSEN = 0;///////////

// Enable PWM

EPwm6Regs.TBCTL.bit.CTRMODE = TB_COUNT_UP;

//CpuSysRegs.PCLKCR0.bit.TBCLKSYNC = 1;

//////////////

// TB_SYNC_DISABLE:    Output sync signal disconnected

// (i.e. ePWMn+1 module will not be synced to the current, ePWMn, module)

EPwm6Regs.TBCTL.bit.SYNCOSEL = 3;//////////

EPwm6Regs.TBCTL2.bit.SYNCOSELX = 0;/////////////

//////////////

// Free run TB module when stopped in debut mode

EPwm6Regs.TBCTL.bit.FREE_SOFT = 0x3;////////////////////

// Load new compare value when counter == period

EPwm6Regs.CMPCTL.bit.LOADAMODE = 0; // load on CTR=PRD//////

// Use shadow register when updating compare register

EPwm6Regs.CMPCTL.bit.SHDWAMODE = 0;////////

/////////////////////////////////////

////////////// Set actions

EPwm6Regs.AQCTLA.bit.ZRO = 0x2;       // Set PWMxA on CTR=Zero;Set: force EPWMxA output high.

//////////////When TBCTR = CMPA on Up Count; Clear PWMxA on event A, up count; Clear: force EPWMxA output low

EPwm6Regs.AQTSRCSEL.bit.T2SEL = 0x1;  // configure DCA_EVT2 as trigger for T2 event;

EPwm6Regs.AQCTLA2.bit.T2U = 0x1;// Action when event occurs on T2 in UP-Count; Clear: force EPWMxA output low.

/////////////////////////////////////

EPwm6Regs.CMPA.all = 0;

EPwm6Regs.TBPHS.bit.TBPHS = 0;/////

// enable or disable loading of a PWM module's phase register into time-base coutner

EPwm6Regs.TBCTL.bit.PHSEN =  0;

// configure sync signal output select

EPwm6Regs.TBCTL.bit.SYNCOSEL = 1;/

EPwm6Regs.TZSEL.bit.DCAEVT2 = 1;// Enable DCAEVT2 as a CBC trip source for this ePWM module

// Configure PWM channel action when trip zone triggered;//When a trip event occurs the following action is taken on output EPWMxA.

//                                                                             2(TZ_FORCE_LO): Force EPWMxA to a low state)

//EPwm6Regs.TZCTL.bit.TZA = 2;///

///////////////////////////////////////

// Enable and configure dead-band module

EPwm6Regs.DBCTL.bit.OUT_MODE = 3;

// Enable and configure dead-band module

EPwm6Regs.DBCTL.bit.IN_MODE = 0;

// A and B channel polarity

EPwm6Regs.DBCTL.bit.POLSEL = 2;

// Configure falling- and rising-edge delay in TBCLK units

EPwm6Regs.DBFED.bit.DBFED = 0;

EPwm6Regs.DBRED.bit.DBRED = 0;

///////////////////////////////////////

// Select Digital Compare event to cause a trip zone event

// DCxEVT2/Event 2 signifies cycle-by-cycle event

EPwm6Regs.TZDCSEL.bit.DCAEVT2 = 2;//////////010: DCAH = high, DCAL = don't care

// Select Digital Compare high event from TRIP4IN

EPwm6Regs.DCTRIPSEL.bit.DCAHCOMPSEL = 3;

EPwm6Regs.DCACTL.bit.EVT2SRCSEL = 0; // 0: Source Is DCAEVT2 Signal; ///

EPwm6Regs.DCACTL.bit.EVT2FRCSYNCSEL = 1;//1: Source is passed through asynchronously//

////////////////////////////////

//////////(DCFCTL:Digital Compare Filter Control Register;SRCSEL:Filter Block Signal Source Select;

EPwm6Regs.DCFCTL.bit.SRCSEL = 1;            // 01: Source Is DCAEVT2 Signal

///(BLANKE:Blanking Window Enable/Disable;DC_BLANK_ENABLE:Blanking window is enabled)

EPwm6Regs.DCFCTL.bit.BLANKE = 1;            // 1: Blanking window is enabled

EPwm6Regs.DCFCTL.bit.BLANKINV = 0;          // 0: Blanking window not inverted;  1: Blanking window inverted

//////PULSESEL:Pulse Select For Blanking & Capture Alignment

EPwm6Regs.DCFCTL.bit.PULSESEL = 1;          // 01: Time-base counter equal to zero;

//////////////////////////////

EPwm6Regs.HRPCTL.bit.PWMSYNCSELX = 0;         // EPWMSYNCPER is defined by PWMSYNCSEL

EPwm6Regs.HRPCTL.bit.PWMSYNCSEL = 1;          // 1:CTR = zero;  0:CTR = PRD(PWMSYNC Source Select Bit: This bit selects the source for the

//Configure TRIP4 to be CTRIP3H

EPwmXbarRegs.TRIP4MUX0TO15CFG.bit.MUX4 = 0;

//Enable TRIP4 Mux for Output

EPwmXbarRegs.TRIP4MUXENABLE.bit.MUX4 = 1; ///EPWMSYNCPER signal that goes to the CMPSS )

////////

EDIS;

////////////

EPwm6Regs.DCFOFFSET = EPWM6_BLANK_WND_OFFSET;

EPwm6Regs.DCFWINDOW = EPWM6_BLANK_WND;

//////////////////

}

//